| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0056 |
| --- | --- |

| Title: | AHG13: Disparity vector constraint study on 4K stereo sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution studied coding efficiency impact of the disparity vector constraint on 4K stereoscopic test sequences as AHG13 activity. Test materials provided by National Institute of Information and Communications Technology (NICT) are encoded under the current MV-HEVC Stereo profile condition on HTM6.0. It is reported that the BD-rate difference due to the constraint is 0.0% on average of 8100 frames.

# Introduction

In the 3rd Geneva meeting, disparity vector constraint was introduced in MV-HEVC Stereo profile [2], [4]. The vertical component of disparity vector is constrained to be equal to or less than 56 in pixel for view-level parallel processing. It was questioned whether the fix constraint value is enough or not for high resolution input such as 4K or 8K, and further study was recommended as AHG13 activity.

In JCT3V-C0101 [1], 3D test materials were proposed for the international standardization activity by NICT and NTT. The materials include ultra-high resolution (4K) stereoscopic sequences. They have been distributed in Japan for academic purpose.

This contribution studies the coding efficiency impact of the disparity vector constraint on the NICT 4K stereo sequences.

# Test condition

- Sequence setting

The 4K stereo sequences provided by NICT contain lots of scenes, which are captured in various camera conditions. This contribution studied “okugai” (outdoor scene) and “okunai” (indoor scene) test sets. There are 15,935 and 19,449 frames for each view, respectively. It was divided into short scenes at the scene change position. The first 300 frames of the each scene were encoded. The exact start and end positions are shown in Table 1 and Table 2.

The original is provided in TIFF format (RGB444). Before encode, it was converted to YUV420 format. The spatial resolution is 3860x2160 pixel, and temporal resolution is 60 fps.

Table 1 Encoded position of “okugai” (outdoor scene) test set

| Cut No. | Start [frame #] | End [frame #] | Length [frame #] | Characteristics |
| --- | --- | --- | --- | --- |
| 1 | 730 | 1029 | 300 | Cycling Convergence change: large |
| 2 | 1376 | 1675 | 300 | Cycling Convergence change: small |
| 3 | 2097 | 2396 | 300 | Cycling Object: far to near |
| 4 | 3069 | 3368 | 300 | Cycling Object: far to middle |
| 5 | 4219 | 4618 | 300 | Park Convergence: far |
| 6 | 4850 | 5149 | 300 | Park Convergence: near |
| 7 | 5536 | 5835 | 300 | Park Convergence: middle |
| 8 | 6603 | 6902 | 300 | Fountain Camera angle: narrow Object: far |
| 9 | 7449 | 7748 | 300 | Fountain Camera angle: wide Object: near |
| 10 | 8161 | 8460 | 300 | Building Camera distance: narrow Object: far |
| 11 | 9007 | 9306 | 300 | Building Camera distance: wide Object: near |
| 12 | 9833 | 10132 | 300 | Building parallel view method |
| 13 | 10651 | 10950 | 300 | Building cross-eyed view method |

Table 2 Encoded position of “okunai” (indoor scene) test set

| Cut No. | Start [frame #] | End [frame #] | Length [frame #] | Characteristics |
| --- | --- | --- | --- | --- |
| 1 | 1180 | 1479 | 300 | Makeup Convergence change: large |
| 2 | 1841 | 2140 | 300 | Makeup Convergence change: small |
| 3 | 2702 | 3001 | 300 | Facewash Convergence point: far |
| 4 | 3185 | 3484 | 300 | Facewash Convergence point: near |
| 5 | 3734 | 4033 | 300 | Dressing Convergence point: far |
| 6 | 4401 | 4700 | 300 | Dressing Convergence point: near |
| 7 | 5376 | 5675 | 300 | Living room Convergence point: far |
| 8 | 6113 | 6412 | 300 | Living room Convergence point: near |
| 9 | 6808 | 7107 | 300 | Living room parallel view method |
| 10 | 7494 | 7793 | 300 | Living room cross-eyed view method |
| 11 | 10644 | 10943 | 300 | Dining room Lighting: well-lighted |
| 12 | 11109 | 11408 | 300 | Dining room Lighting: dark |
| 13 | 12613 | 12912 | 300 | Dressing Composition pattern1 |
| 14 | 13222 | 13521 | 300 | Dressing Composition pattern2 |

- Encoder setting

HTM6.0 is used. For MV-HEVC condition, the following macro setting is enabled.

#define QC_MVHEVC_B0046		1

For configuration file, “baseCfg_2view.cfg” is used. The following point is focused.

  - Anchor

DisparitySearchRangeRestriction  : 0 		# same as baseCfg_2view.cfg

- Tested

DisparitySearchRangeRestriction  : 1

VerticalDisparitySearchRange    : 56 		# same as baseCfg_2view.cfg

# Result

Test results are summarized in Table 3 and Table 4. The BD-rate differences for each test set are 0.0% and -0.1% on average about video PSNR / video bitrate. The detail is attached in the excel files.

Table 3 Results of “okugai” (outdoor scene) test set

| Cut No. | video 0 | video 1 | video PSNR / video bitrate |
| --- | --- | --- | --- |
| okugai_start730 | 0.0% | 0.0% | 0.0% |
| okugai_start1376 | 0.0% | -0.1% | 0.0% |
| okugai_start2097 | 0.0% | 0.1% | 0.0% |
| okugai_start3069 | 0.0% | 0.0% | 0.0% |
| okugai_start4219 | 0.0% | 0.0% | 0.0% |
| okugai_start4850 | 0.0% | -0.1% | 0.0% |
| okugai_start5536 | 0.0% | 0.0% | 0.0% |
| okugai_start6603 | 0.0% | 0.0% | 0.0% |
| okugai_start7449 | 0.0% | -0.1% | 0.0% |
| okugai_start8161 | 0.0% | -0.2% | 0.0% |
| okugai_start9007 | 0.0% | -0.2% | 0.0% |
| okugai_start9833 | 0.0% | -0.2% | 0.0% |
| okugai_start10651 | 0.0% | -0.2% | -0.1% |
| average | 0.0% | -0.1% | 0.0% |

Table 4 Results of “okunai” (indoor scene) test set

| Cut No. | video 0 | video 1 | video PSNR / video bitrate |
| --- | --- | --- | --- |
| okunai_start1180 | 0.0% | -0.2% | -0.1% |
| okunai_start1841 | 0.0% | 0.0% | 0.0% |
| okunai_start2702 | 0.0% | -0.4% | -0.2% |
| okunai_start3185 | 0.0% | 0.0% | 0.0% |
| okunai_start3734 | 0.0% | -0.1% | 0.0% |
| okunai_start4401 | 0.0% | -0.1% | 0.0% |
| okunai_start5376 | 0.0% | -0.2% | -0.1% |
| okunai_start6113 | 0.0% | -0.2% | -0.1% |
| okunai_start6808 | 0.0% | -0.1% | 0.0% |
| okunai_start7494 | 0.0% | 0.1% | 0.0% |
| okunai_start10644 | 0.0% | 0.0% | 0.0% |
| okunai_start11109 | 0.0% | -0.1% | 0.0% |
| okunai_start12613 | 0.0% | -0.1% | 0.0% |
| okunai_start13222 | 0.0% | -0.1% | -0.1% |
| average | 0.0% | -0.1% | -0.1% |

# Conclusion

This contribution studied the coding efficiency impact due to the DV constraint as 56 in pixel for the 4K stereo test sequences provided by NICT. The significant impact was not observed. It is concluded that the current constraint value is sufficient for the test sets though it is encouraged to continue the study.

# Acknowledgement

The authors appreciate NICT for providing the valuable 3D test materials.

# Reference

[1] JCT3V-C0101 “AHG11: 3D Test Materials from NICT-3D data set,” Geneva, Jan., 2012

[2] JCT3V-C0078 “AHG13: On disparity vector constraints,” Geneva, Jan., 2012

[3] JCT3V-C0083 “AHG13: Disparity vector restrictions,” Geneva, Jan., 2012

[4] JCT3V-C1004 “MV-HEVC Draft Text 3,” Geneva, Jan., 2012

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).