| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0057r1 |
| --- | --- |

| Title: | AHG7: On initialization process of reference picture lists for HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution is a copy of JCTVC-M0081 and a follow-up of JCT3V-C0079.

In the current MV-HEVC draft (JCT3V-C1004) and SHVC working draft (JCTVC-L1008), inter-layer reference picture (ILRP) is appended at the end of temporal reference picture list. It is noted that reference picture list modification (RPLM) syntax is necessary to put the inter-view reference forward.

This contribution proposes to modify the initialization process. ILRP is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating the temporal list. The motivation is to reduce the RPLM signalling for common coding structures.

Two options are studied in two contexts, scalable and multi-view extensions. Option1 modifies the initialization prosess in List0. Option2 does in both List0 and List1. Proposed options are tested on SHM1.0 (refidx approach) and HTM6.0.

Under SHVC test conditions, it is reported that the number of RPLM signalling is reduced from 14 to 7 in option1 and to 8 in option2, and that the BD-rate difference is 0.0% in all conditions for both two options. Under 3DV test conditions, the number of RPLM signalling is reduced from 8 to 5 in option1, but increased to 13 in option2 with the BD-rate difference -0.1% and 0.1%, respectively.

As conclusion, it is recommended to modify the process as option1. It is believed that the change is beneficial for the common reference picture structure (e.g. N15M3) in the extensions.

In the revision1 package, the filename of datasheet for HTM result is corrected. Thanks to the cross-checaker for pointing the bug. The content is not modified.

# Problem Statement

In the current MV-HEVC draft (JCT3V-C1004) and SHVC working draft (JCTVC-L1008), initialization process for reference picture lists is specified in G.8.3.4. An inter-view reference picture is always appended at the end of temporal reference picture list (RefPicListTempX).

Figure 1 visualizes four types of reference pictures to create RefPicListTempX in multi-view extensions context. White box with “C” represents a current picture. Green and red boxes stand for short-term reference pictures which are before and after the current picture in POC, respectively. Purples are long-term pictures, and blues are inter-view pictures. In the figure, the number in the boxes is indexed based on POC or layerId.

Figure 1: four types of reference picture to create RefPicListTemp0/1

An example of the current initialization process is reportedly shown in Figure 2.

Firstly, four lists, which are RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetLtCurr and RefPicSetIvCurr, are created from the reference pictures with used_by_curr_pic_flag. When used_by_curr_pic_sX_flag is equal to 0, the reference picture is not appended to the each list. In the example, two reference pictures with index 0 and 1 are appended to RefPicSetStCurrBefore.

Then, RefPicListTempX is created from the four lists.

The order to be appended for list0 is

RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetLtCurr and RefPicSetIvCurr.

The order to be appended for list1 is

RefPicSetStCurrAfter, RefPicSetStCurrBefore, RefPicSetLtCurr and RefPicSetIvCurr.

It is noted that RefPicSetIvCurr is appended at the end for both lists.

Finally, reference picture list (RefPicListX) is created from RefPicListTempX. When a reference picture list modification (RPLM) syntax is not present, the first (num_ref_idx_lx_active_minus1+1) pictures in RefPicListTempX is copied to RefPicListX. Otherwise, RefPicListX is created according to the RPLM syntax until the list is full.

Figure 2: an example of the current initialization process for reference picture lists

Here, following issues are raised regarding the current process.

- When num_ref_idx_lx_active_minus1 is small, inter-view reference picture might not be included in RefPicListX without sending RPLM syntax. In the example, the inter-view reference picture (blue box) is not included to the RefPicListX. It is noted that num_ref_idx_lx_active_minus1 is 3 or 4 in usual bitstream as CTC from decoder complexity point of view.

- Generally, it makes coding efficiency better to assign smaller reference index to more correlated reference picture. Since inter-view prediction gives significant coding gain, it is reasonable to append an inter-view picture forward in creating the default list.

- The position of RefPicSetIvCurr should be different in RefPicList0 and RefPicList1 by default. It increases the possibility that the two lists are unique and can contain all reference pictures. It is believed that this helps encoders to choice better prediction mode from the default list.

The same issues are pointed in SHVC refidx approach. Thus, this contribution studied an improvement in two contexts, scalable and multi-view extensions.

# Proposal

In proposal, an inter-layer reference picture is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating RefPicListTempX.

Two options are proposed. Option1 modifies List0 process only. It is noted that option1 is same as the proposal in JCT3V-C0079. Option2 modifies both List0 and List1 processes.

The proposed initialize process is as follows. See the draft text in Section 6.

## Option1 (same as JCT3V-C0079)

RefPicListTemp0 is created in the following order.

RefPicSetStCurrBefore, RefPicSetIvCurr, RefPicSetStCurrAfter and RefPicSetLtCurr.

Option1 doesn’t modify RefPicListTemp1 process.

## Option2

RefPicListTemp0 is created in the following order.

RefPicSetStCurrBefore, RefPicSetIvCurr, RefPicSetStCurrAfter and RefPicSetLtCurr.

RefPicListTemp1 is created in the following order.

RefPicSetStCurrAfter, RefPicSetIvCurr, RefPicSetStCurrBefore and RefPicSetLtCurr.

# Reference picture structure in SHVC and 3D test condition

Reference picture structure in SHVC and 3D CTCs are shown in from Table 1 to Table 6. The difference between anchor and proposal are written in red font.

The final reference picture structure is basically same between anchor and proposals. By the proposed initialization process, the number of RPLM signalling is changed.

It is pointed that the structure is different in POC4, 2, and 1 of List0 under SHVC RA test condition (See Table 1). The impact is reported in the next section.

Below is a legend for the value in RPLM column.

○: RPLM signaling is necessary

△: RPLM signaling is not necessary if UsedByCurr is used

-: RPLM signaling is not necessary

Table 1 the order of reference pictures in List0 under SHM RA test condition

|  |  | Current |  |  | Proposed (option1/2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L0Temp | RPLM | L0Final | L0Temp | RPLM | L0Final |
| 8 | 5 | -8 -10 -12 -16 IL | - | -8 -10 -12 -16 IL | -8 -10 -12 -16 IL | - | -8 -10 -12 -16 IL |
| 4 | 3 | -4 (-6) 4 IL | ○ | -4 (-6 or 4) IL | -4 (-6) IL 4 | - | -4 -6 IL or -4 IL 4 |
| 2 | 3 | -2 (-4) 2 6 IL | ○ | -2 (-4 or 2) IL | -2 (–4) IL 2 6 | - | -2 -4 IL or -2 IL 2 |
| 1 | 3 | -1 1 3 7 IL | ○ | -1 1 IL | -1 IL 1 3 7 | - | -1 IL 1 |
| 3 | 3 | -1 -3 1 5 IL | ○ | -1 -3 IL | -1 -3 IL 1 5 | - | -1 -3 IL |
| 6 | 3 | -2 -4 -6 2 IL | ○ | -2 -4 IL | -2 -4 -6 IL 2 | △ | -2 -4 IL |
| 5 | 3 | -1 -5 1 3 IL | ○ | -1 -5 IL | -1 -5 IL 1 3 | - | -1 -5 IL |
| 7 | 3 | -1 -3 -7 1 IL | ○ | -1 -3 IL | -1 -3 -7 IL 1 | △ | -1 -3 IL |

Table 2 the order of reference pictures in List1 under SHM RA test condition

|  |  | Current and Proposed (option1) |  |  | Proposed (option2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L1Temp | RPLM | L1Final | L1Temp | RPLM | L1Final |
| 8 | 5 | -8 -10 -12 -16 IL | - | -8 -10 -12 -16 IL | IL -8 -10 -12 -16 | ○ | -8 -10 -12 -16 IL |
| 4 | 3 | 4 -4 (-6) IL | ○ | 4 -4 IL | 4 IL -4 (-6) | ○ | 4 -4 IL |
| 2 | 3 | 2 6 -2 (-4) IL | ○ | 2 6 IL | 2 6 IL -2 (-4) | - | 2 6 IL |
| 1 | 3 | 1 3 7 -1 IL | ○ | 1 3 IL | 1 3 7 IL -1 | △ | 1 3 IL |
| 3 | 3 | 1 5 -1 -3 IL | ○ | 1 5 IL | 1 5 IL -1 -3 | - | 1 5 IL |
| 6 | 3 | 2 -2 -4 -6 IL | ○ | 2 -2 IL | 2 IL -2 -4 -6 | ○ | 2 -2 IL |
| 5 | 3 | 1 3 -1 -5 IL | ○ | 1 3 IL | 1 3 IL -1 -5 | - | 1 3 IL |
| 7 | 3 | 1 -1 -3 -7 IL | ○ | 1 -1 IL | 1 IL -1 -3 -7 | ○ | 1 -1 IL |

Table 3 the order of reference pictures in List0 under SHM LB and LP test condition

|  |  | Current |  |  | Proposed (option1/2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L0Temp | RPLM | L0Final | L0Temp | RPLM | L0Final |
| 1 | 5 | -1 -5 -9 -13 IL | - | -1 -5 -9 -13 IL | -1 -5 -9 -13 IL | - | -1 -5 -9 -13 IL |
| 2 | 5 | -1 -2 -6 -10 IL | - | -1 -2 -6 -10 IL | -1 -2 -6 -10 IL | - | -1 -2 -6 -10 IL |
| 3 | 5 | -1 -3 -7 -11 IL | - | -1 -3 -7 -11 IL | -1 -3 -7 -11 IL | - | -1 -3 -7 -11 IL |
| 4 | 5 | -1 -4 -8 -12 IL | - | -1 -4 -8 -12 IL | -1 -4 -8 -12 IL | - | -1 -4 -8 -12 IL |

Table 4 the order of reference pictures in List1 under SHM LB test condition

|  |  | Current and Proposed (option1) |  |  | Proposed (option2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L1Temp | RPLM | L1Final | L1Temp | RPLM | L1Final |
| 1 | 5 | -1 -5 -9 -13 IL | - | -1 -5 -9 -13 IL | IL -1 -5 -9 -13 | ○ | -1 -5 -9 -13 IL |
| 2 | 5 | -1 -2 -6 -10 IL | - | -1 -2 -6 -10 IL | IL -1 -2 -6 -10 | ○ | -1 -2 -6 -10 IL |
| 3 | 5 | -1 -3 -7 -11 IL | - | -1 -3 -7 -11 IL | IL -1 -3 -7 -11 | ○ | -1 -3 -7 -11 IL |
| 4 | 5 | -1 -4 -8 -12 IL | - | -1 -4 -8 -12 IL | IL -1 -4 -8 -12 | ○ | -1 -4 -8 -12 IL |

Table 5 the order of reference pictures in List0 under JCT-3V CTC (view1 and view2)

|  |  | Current |  |  | Proposed (option1/2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L0Temp | RPLM | L0Final | L0Temp | RPLM | L0Final |
| 8 | 4 | -8 -10 -12 -16 IL | ○ | -8 IL -10 -12 | -8 -10 -12 -16 IL | ○ | -8 IL -10 -12 |
| 4 | 3 | -4 (-6) 4 IL | ○ | -4 IL (-6 or 4) | -4 (-6) IL 4 | ○ | -4 IL (-6 or 4) |
| 2 | 3 | -2 (-4) 2 6 IL | ○ | -2 IL (-4 or 2) | -2 (–4) IL 2 6 | ○ | -2 IL (-4 or 2) |
| 1 | 3 | -1 1 3 7 IL | ○ | -1 IL 1 | -1 IL 1 3 7 | - | -1 IL 1 |
| 3 | 3 | -1 -3 1 5 IL | ○ | -1 -3 IL | -1 -3 IL 1 5 | - | -1 -3 IL |
| 6 | 3 | -2 -4 -6 2 IL | ○ | -2 -4 IL | -2 -4 -6 IL 2 | ○ | -2 -4 IL |
| 5 | 3 | -1 -5 1 3 IL | ○ | -1 -5 IL | -1 -5 IL 1 3 | - | -1 -5 IL |
| 7 | 3 | -1 -3 -7 1 IL | ○ | -1 -3 IL | -1 -3 -7 IL 1 | ○ | -1 -3 IL |

Table 6 the order of reference pictures in List1 under JCT-3V CTC (view1 and view2)

|  |  | Current and Proposed (option1) |  |  | Proposed (option2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L1Temp | RPLM | L1Final | L1Temp | RPLM | L1Final |
| 8 | 4 | -8 -10 -12 -16 IL | - | -8 -10 -12 -16 | IL -8 -10 -12 -16 | ○ | -8 -10 -12 -16 |
| 4 | 3 | 4 -4 (-6) IL | - | 4 -4 -6 or 4 -4 IL | 4 IL -4 (-6) | ○ | 4 -4 -6 or 4 -4 IL |
| 2 | 3 | 2 6 -2 (-4) IL | - | 2 6 -2 | 2 6 IL -2 (-4) | ○ | 2 6 -2 |
| 1 | 3 | 1 3 7 -1 IL | - | 1 3 7 | 1 3 7 IL -1 | ○ | 1 3 7 |
| 3 | 3 | 1 5 -1 -3 IL | - | 1 5 -1 | 1 5 IL -1 -3 | ○ | 1 5 -1 |
| 6 | 3 | 2 -2 -4 -6 IL | - | 2 -2 -4 | 2 IL -2 -4 -6 | ○ | 2 -2 -4 |
| 5 | 3 | 1 3 -1 -5 IL | - | 1 3 -1 | 1 3 IL -1 -5 | ○ | 1 3 -1 |
| 7 | 3 | 1 -1 -3 -7 IL | - | 1 -1 -3 | 1 IL -1 -3 -7 | ○ | 1 -1 -3 |

# Results

The test results on SHM1.0 and HTM6.0 are reported in this section.

## SHVC test condition

The result under SHVC test condition is shown in Table 7. It is reported that the result is 0.0% in all conditions. The reference picture structure is different for POC4, 2 and 1 in RA as shown inTable1 due to the reduction of RPLM, but the impact is negligible.

In SHVC test condition, the results of option1 and option2 are identical since the bitrate for RPLM signaling is not counted because of SHM1.0 implementation. That part is studied in 3D-HEVC condition, which is described in the next section.

Table 7 Option1 and Option2 result under SHVC test consitions

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 20.1% | 33.1% | 34.1% | 17.4% | 28.1% | 30.6% | 15.8% | 30.0% | 34.7% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 90.2% |  |  | 89.7% |  |  | 99.1% |  |  |
| Dec Time[%] | #VALUE! |  |  | 97.1% |  |  | #VALUE! |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 26.6% | 37.8% | 39.1% | 22.9% | 32.8% | 35.7% | 23.5% | 34.6% | 39.4% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98.9% |  |  | 99.0% |  |  | 99.0% |  |  |
| Dec Time[%] | 93.3% |  |  | 92.5% |  |  | 94.3% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 28.5% | 38.7% | 40.2% | 24.8% | 33.2% | 35.6% | 24.3% | 34.6% | 39.5% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 91.7% |  |  | 91.8% |  |  | 98.3% |  |  |
| Dec Time[%] | 90.8% |  |  | 86.9% |  |  | 87.0% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

## 3D-HEVC CTC

The test results for option1 and option2 under 3D-HEVC CTC are reportedly shown in Table 8 and Table 9, respectively. It is reported that the BD-difference from anchor is -0.1% for option1 and 0.1% for option2 in video and synth total bitrate.

In is noted that the reference structure is same between anchor and proposals as shown in Table 5 and Table 6. The difference comes from the bitrate for RPLM signaling.

Table 8 Option1 result under 3D-HEVC CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Kendo | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Newspaper_CC | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| GT_Fly | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Poznan_Street | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |

Table 9 Option2 result under 3D-HEVC CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Newspaper_CC | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| GT_Fly | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Poznan_Street | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| 1920x1088 | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| average | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |

## The number of RPLM signaling

The number of RPLM signaling is summarized as Table 10.

Under SHVC test conditions, the number of RPLM signalling is reduced from 14 to 7 in option1 and to 8 in option2.

Under 3DV test conditions, the number of RPLM signalling is reduced from 8 to 5 in option1, but increased to 13 in option2.

Table 10 the number of RPLM signaling

|  |  | Current | Proposed (option1) | Proposed (option2) |
| --- | --- | --- | --- | --- |
| SHM1.0(ridx) RA | List0 | 7/8 | 0/8† | 0/8† |
|  | List1 | 7/8 | 7/8 | 4/8 |
| SHM1.0(ridx) LB | List0 | 0/4 | 0/4 | 0/4 |
|  | List1 | 0/4 | 0/4 | 4/4 |
| SHM1.0(ridx) LP | List0 | 0/4 | 0/4 | 0/4 |
|  | List1 | - | - | - |
| Subtotal of SVC CTC |  | 14 | 7 | 8 |
| JCT-3V HTM CTC | List0 | 8/8‡ | 5/8‡ | 5/8‡ |
|  | List1 | 0/8‡ | 0/8‡ | 8/8‡ |
| Subtotal of 3DV CTC |  | 8 | 5 | 13 |
| Total |  | 22 | 12 | 21 |

†: The reference pictures order in the list is different from the anchor in some POC. But the test results show the impact is negligible.

‡: The number is for a dependent view. It is noted that HTM CTC uses two dependent views.

# Conclusion

As conclusion, the proponents recommended to modify the process as option1. It is believed that the change is beneficial to the common reference picture structure (e.g. N15M3).

# Draft text

The proposed text for option1 and option2 is attached. The change part is green highlighted on top of JCT3V-C1004_d3.

## Option1; the process in RefPicList0 is modified

- G.8.3.4	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]

}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-3)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i< NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

## Option2; the processes in both RefPicList0 and RefPicList1 are modified

- G.8.3.4	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-5)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]

}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-6)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i< NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-7)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i< NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-8)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# Reference

- JCT3V-C1004, “MV-HEVC Draft Text 3 (ISO/IEC 23008-2:201x/PDAM2)”

- JCTVC-L1008, “SHVC Draft Text 1”

- JCT3V-B0046, “AHG5: MV-HEVC software for HTM”

- JCTVC-L1003, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Last Call)”

- JCT3V-C0060, “AHG7: Reference picture list initialization for MV-HEVC”

- JCT3V-C0079, “AHG7: On initialization process for reference picture lists of MV-HEVC and 3DHTM”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).