| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0060 |
| --- | --- |

| Title: | 3D-CE5.h related: Removal of parsing dependency in illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In the current 3D-HEVC HTM, illumination compensation (IC) has a parsing problem that decoding an IC utilize flag, or ic_flag needs all motion parameters derivation in CU. This design could decrease CABAC throughput because that motion parameters derivation could stop CABAC stage which should ideally continue to operate independently. This contribution proposes a method to remove the parsing dependency by sending ic_flag independent of ref_idx when merge mode is utilized. The experimental results shows the proposed simplification shows 0.1 % loss in total video.

This combination result of this contribution (JCT3V-D0060) and a related contribution by the author (JCT3V-D0060) is reported in JCT3V-D0062.

# Introduction

In the current HTM, an illumination compensation utilize flag, or ic_flag is coded on CU basis on condition that the CU has one or more PU that uses inter-view prediction. This condition needs derivation of all ref_idx_lX in CU. Because ref_idx_lX is not explicitly sent in merge mode, relatively high complex derivation process of merge mode is inevitable in the design. In other words, all motion parameters derivation in CU is required before decoding ic_flag. It is considered that this parsing dependency could cause pauses in CABAC stage, which decreases its throughput. Therefore we think solving this dependency is necessary.

Similar parsing dependency related to residual prediction had been removed with the adoption of JCT3V-C0138 [1].

# Proposal

In AMVP mode, ic_flag is sent depends on ref_idx_lX (same as HTM5).

In Skip / Merge mode, ic_flag is conditionally sent depend on merge_idx and a new slice segment header flag slice_ic_skip_mergeidx0_flag. If ic_flag is not sent on merge mode, ic_flag is inferred to 0. Detailed condition is shown in Table 1.

Table 1 Condition of ic_flag sending

| Mode | Condition | Note |
| --- | --- | --- |
| Skip / Merge (merge_flag[ x0 ][ y0 ]) | !(merge_idx[ x0 ][ y0 ] == 0 && slice_ic_skip_mergeidx0_flag) | When inter-view candidate is used (merge_idx == 0) and encoder send a new flag slice_ic_skip_merge_flag, then ic_flag is not sent (inferred to 0) (independent of ref_idx_lX) |
| AMVP (!merge_flag[ x0 ][ y0 ]) | anyIvRefPicFlag | conditionally sent depending on ref_idx_lX (Same as HTM5) |

Basically when merge_idx is 0 (usually inter-view merge candidate is used), the usage of inter-view prediction is so small that illumination compensation is should be skipped to avoid ic_flag’s overhead.

In some pictures where motion vector candidate is mostly used, that skipping doesn’t work. So we only apply the merge_idx base skipping on condition that (POC % 8) is not 0. This POC based condition is not decided by decoder but by encoder, which send a new slice header flag slice_ic_skip_mergeidx0_flag. This allows encoders can control the condition depends on coding structure or sequences.

Furthermore, illumination compensation is only applied on 2Nx2N case.

# Proposed Text

Based on JCT3V-C1005_spec_d0.doc [2]

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| slice_ic_enable_flag | u(1) |
| slice_ic_skip_mergeidx0_flag | u(1) |
| … |  |
| } |  |

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| … | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| .. |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| … |  |
| } |  |
| } |  |

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

The variable icEnableFlag specifying whether ic_flag is present in the bitstream is derived as

  - icEnableFlag = slice_ic_enable_flag && PartMode == 2Nx2N && (merge_flag[ x0 ][ y0 ] && (!(merge_idx[ x0 ][ y0 ] == 0 && slice_ic_skip_mergeidx0_flag)) || (!merge_flag[ x0 ][ y0 ] && anyIvRefPicFlag)	(Error: Reference source not found-1)

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.3% | 0.10% | 0.08% | 0.02% | 99.3% | 99.8% | 98.9% |
| Kendo | 0.0% | 0.8% | 0.9% | 0.34% | 0.30% | 0.36% | 97.2% | 100.7% | 99.9% |
| Newspapercc | 0.0% | 0.1% | 0.3% | 0.10% | 0.10% | 0.03% | 98.0% | 100.5% | 100.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 98.9% | 98.1% | 97.4% |
| PoznanHall2 | 0.0% | -0.2% | 0.2% | 0.01% | -0.01% | 0.04% | 98.4% | 97.5% | 98.6% |
| PoznanStreet | 0.0% | 0.4% | 0.0% | 0.07% | 0.05% | 0.05% | 98.8% | 97.1% | 97.4% |
| UndoDancer | 0.0% | 0.1% | 0.1% | 0.02% | 0.02% | 0.03% | 98.0% | 96.6% | 97.5% |
| 1024x768 | 0.0% | 0.3% | 0.5% | 0.18% | 0.16% | 0.14% | 98.2% | 100.3% | 99.7% |
| 1920x1088 | 0.0% | 0.1% | 0.1% | 0.02% | 0.02% | 0.03% | 98.6% | 97.4% | 97.7% |
| average | 0.0% | 0.2% | 0.3% | 0.09% | 0.08% | 0.08% | 98.4% | 98.6% | 98.5% |

# Conclusion

This contribution proposes a method to remove the parsing dependency by sending ic_flag independent of  ref_idx when merge mode is utilized. The experimental results shows the proposed simplification shows 0.1 % loss in total video. It is recommended to adopt this method in the next HTM.

# References

[1] J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE4.h results on removal of parsing dependency for inter-view residual prediction”, JCT3V-C0138, JCT3V 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

[2] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3”, JCT3V-C1005, JCT3V 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

[3] T. Ikai, “3D-CE5.h related: Illumination compensation regression improvement and simplification,” JCT3V-D0061, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

[4] T. Ikai, “3D-CE5.h related: Combination of JCT3V-D0060 and JCT3V-D0061,” JCT3V-D0062, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).