| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0062 |
| --- | --- |

| Title: | 3D-CE5.h related: Combination of JCT3V-D0060 and JCT3V-D0061 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports combination of illumination contribution proposals, 3D-CE5.h related: Removal of parsing dependency for illumination compensation (JCT-D0060) and 3D-CE5.h related: Removal of parsing dependency for illumination compensation (JCT-D0061). The combination of JCT-D0060 and JCT-D0061 reportedly shows 0.1 % gain in total video while JCT-D0060 and JCT-D0061 show 0.1 % loss and 0.2 % gain in total video respectively. It is reported that the coding efficiency of two proposals is almost orthogonal. The author asserts this combination bring a number of simplification with small gain.

# Introduction

JCT-D0060 consists of

- Removal of parsing dependency

JCT-D0061 consists of

- Regression improvement

- Constant shift simplification

- IC parameters derivation simplification

- 2N x 2N simplification

- L0 Only simplification

# Experimental results

Table 1 shows the combination results and Table 2 shows the summary of related contributions.

Table 1 Result of combination of JCT3V-D0060 and JCT3V-D0061

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.9% | -1.0% | -0.4% | -0.3% | -0.3% | 100.0% | 98.5% | 98.8% |
| Kendo | 0.0% | -0.2% | -0.5% | -0.1% | -0.1% | 0.0% | 97.1% | 100.3% | 99.9% |
| Newspaper_CC | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | 98.7% | 100.4% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.9% | 99.9% |
| Poznan_Hall2 | 0.0% | -0.4% | -1.5% | -0.4% | -0.4% | -0.2% | 98.7% | 98.8% | 99.7% |
| Poznan_Street | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.0% | 100.5% | 98.4% | 100.2% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 100.2% | 100.4% |
| 1024x768 | 0.00% | -0.34% | -0.61% | -0.19% | -0.17% | -0.10% | 98.6% | 99.7% | 99.7% |
| 1920x1088 | 0.00% | -0.10% | -0.40% | -0.11% | -0.10% | -0.04% | 99.6% | 99.5% | 100.0% |
| average | 0.0% | -0.20% | -0.49% | -0.14% | -0.13% | -0.06% | 99.2% | 99.6% | 99.9% |

Table 2 Summary of related contributions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D0060 | 0.00% | 0.18% | 0.26% | 0.09% | 0.08% | 0.08% | 98.4% | 98.6% | 98.5% |
| D0061 | 0.0% | -0.51% | -0.86% | -0.28% | -0.24% | -0.18% | 99.0% | 99.8% | 100.1% |
| D0062 (D0060 + D0061) | 0.0% | -0.20% | -0.49% | -0.14% | -0.13% | -0.06% | 99.2% | 99.6% | 99.9% |

# Conclusion

The combination of JCT-D0060 and JCT-D0061 reportedly shows 0.1 % gain in total video, which means the coding efficiency of the two proposals is almost orthogonal. It is recommended to adopt this combination in the next HTM.

# References

[1] T. Ikai, “3D-CE5.h related: Removal of parsing dependency for illumination compensation,” JCT3V-D0060, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

[2] T. Ikai, “3D-CE5.h related: Illumination compensation regression improvement and simplification,” JCT3V-D0061, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).