| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0065r1 |
| --- | --- |

| Title: | On collocated picture and low-delay checking for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Xiaozhen Zheng, Xu Chen, Jianhua Zheng No. 156, Rd. Bei-qing Hai-dian district 100095 Beijing, China | Tel: Email: | +86 10 60612372 yblin@huawei.com zhengxiaozhen@huaei.com anderson.chen@huawei.com zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution proposes two parts for MV-HEVC:

1) add a constraint on collocated picture signalling for storage reduction prediction, such that the signalled collocated picture shall be an inter-view reference picture rather than a temporal reference picture for non-base view coding. A flag in SPS extension is used to indicate the bitstream constraint. As a result, decoder only needs to store motion information in the inter-view reference picture, not to store that of temporal reference pictures. Test results reportedly show the proposed method has negligible influence on BD-rate performance against MV-HEVC anchor under common test conditions

2) modification of low-delay checking process related to TMVP derivation. It is proposed that low-delay flag is set to true if collocated picture is an inter-view reference picture. The modification is a slice-level change to HTM reference software. Test results reportedly show the proposed modification achieves average BD-rate saving of -0.7% for 3-view coding and -1.8% for single view coding over MV-HEVC anchor under common test conditions.

In revision 1 of this contribution, the related WD text changes are updated.

# Introduction

In MV-HEVC, base-view reference picture is used as an additional reference picture for non-base view coding [1]. It means the base-view reference picture can be enabled as collocated picture in TMVP derivation for non-base view coding. Consequently motion information in the base-view reference picture needs to be stored for TMVP derivation, as well as motion information in temporal reference picture.

.This contribution firstly proposes to add constraint on collocated picture to reduce the storage of motion information in reference pictures. In addition, modification to low-delay checking process is also proposed for coding efficiency improvement.

# Proposed methods

- Add constraint on collocated picture for storage reduction

It is proposed to add a flag in SPS extension to indicate constraint on collocated picture, such that the collocated picture shall be an inter-view reference picture rather than temporal reference picture for non-base view coding.

As a result, only motion information in the inter-view reference picture needs to be stored whereas that in temporal reference pictures within current view are no longer stored. At decoder side, storage requirement for motion information in reference pictures can be reduced to 1/N (N denotes number of reference pictures).

- Set low-delay flag to true if collocated picture is inter-view reference picture for non-base view coding.

Low-delay checking is required for TMVP derivation in HEVC. Depending on the result of low-delay checking a collocated MV of collocated block is determined to derive target MV of current block [2]. In HM reference software, a low-delay flag is used to indicate whether all the reference pictures’ POCs are less than or equal to the current picture’s POC. The low-delay flag is determined at slice level and then used in TMVP derivation.

In MV-HEVC, when inter-view reference picture is used as collocated picture, the collocated MV in the same reference picture list as that of the target MV is desirable for TMVP prediction due to more correlation between the collocated MV and the target MV [3]. Base on the observation, it is proposed to set the low-delay flag to true if inter-view reference picture is used as collocated picture.

The modification is considered as slice-level change since the low-delay checking process is performed at slice-level in practical encoder/decoder implementation, e.g. HM implementation.

# Simulation results

HTM6.0 enabled with MV-HEVC configuration is used as anchor [4]. Simulations are carried out under common test conditions [5].

For the proposed method 1, encoder is forced to use inter-view reference picture as collocated picture by using the existing signalling method. The test results in Tab.1 show that the proposed method achieves average BD-rate saving of -0.1% for 3-view coding.

Table 1: performance of the proposed method 1 against MV-HEVC anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |  | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.9% | -1.1% | -0.9% | -0.9% |  | 102.5% | 98.9% |
| Kendo | 0.0% | -0.6% | 0.3% | -0.1% | -0.1% |  | 102.9% | 93.6% |
| Newspaper_CC | 0.0% | 0.7% | 2.1% | 0.7% | 0.7% |  | 103.4% | 99.4% |
| GT_Fly | 0.0% | 0.9% | 1.1% | -0.2% | -0.2% |  | 95.1% | 104.4% |
| Poznan_Hall2 | 0.0% | -2.0% | 0.6% | -0.1% | -0.1% |  | 102.0% | 105.9% |
| Poznan_Street | 0.0% | 0.7% | 1.1% | 0.3% | 0.3% |  | 102.2% | 102.3% |
| Undo_Dancer | 0.0% | -0.5% | 0.4% | 0.0% | 0.0% |  | 103.1% | 106.1% |
| 1024x768 | 0.0% | -0.6% | 0.4% | -0.1% | -0.1% |  | 102.9% | 97.3% |
| 1920x1088 | 0.0% | -0.3% | 0.8% | 0.0% | 0.0% |  | 100.6% | 104.7% |
| average | 0.0% | -0.4% | 0.6% | -0.1% | -0.1% |  | 101.6% | 101.5% |

For the proposed method 2, the low-delay checking process is modified, and encoder is forced to use inter-view reference picture as collocated picture by using the existing signalling method. The test results are shown in Tab.2. Average BD-rate saving of -0.7% can be observed for 3-view coding, and the average BD-rate saving for single view reaches up to -1.8%.

Table 2: Performance of the proposed method 2 against MV-HEVC anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |  | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -5.7% | -4.0% | -2.5% | -2.5% |  | 101.8% | 92.5% |
| Kendo | 0.0% | -3.2% | -2.0% | -1.4% | -1.4% |  | 103.0% | 98.4% |
| Newspaper_CC | 0.0% | -0.3% | 1.8% | 0.4% | 0.4% |  | 102.2% | 105.1% |
| GT_Fly | 0.0% | 0.1% | 0.5% | -0.5% | -0.5% |  | 94.9% | 105.3% |
| Poznan_Hall2 | 0.0% | -3.2% | 0.0% | -0.6% | -0.6% |  | 101.1% | 102.4% |
| Poznan_Street | 0.0% | 0.1% | 0.7% | 0.1% | 0.1% |  | 102.3% | 105.0% |
| Undo_Dancer | 0.0% | -0.6% | 0.4% | -0.1% | -0.1% |  | 103.3% | 105.1% |
| 1024x768 | 0.0% | -3.0% | -1.4% | -1.2% | -1.2% |  | 102.3% | 98.7% |
| 1920x1088 | 0.0% | -0.9% | 0.4% | -0.3% | -0.3% |  | 100.4% | 104.5% |
| average | 0.0% | -1.8% | -0.4% | -0.7% | -0.7% |  | 101.2% | 102.0% |

# Conclusion

Constraint on collocated picture is proposed for storage reduction, such that the collocated picture shall be an inter-view reference picture. In addition, low-delay checking is modified for performance improvement. The modification to the low-delay checking is carried out at slice level and considered as slice-level change. The proposed modification achieves overall BD-rate saving of -0.7% for 3-view coding under common test condition.

# References

[1] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 3”, JCT3V-C1004, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

[2] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” JCTVC-L1003, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[3] Y. Lin, X. Zheng, J. Zheng, “Hook on temporal motion vector prediction for MV-HEVC”, JCTVC-L0177, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

[4] 3D-HEVC HTM version 6.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0/

[5] D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

# Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix1: Text changes for 1st method

- G.7.3.2.2.1  Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| collocated_picture_constraint_flag | u(1) |
| } |  |

- G.7.4.3.2.1  Sequence parameter set extension semantics

inter_view_mv_vert_constraint_flag equal to 1 specifies that vertical component of motion vectors used for inter-layer prediction are constrained in the CVS. When inter_view_mv_vert_constraint_flag is equal to 1, the vertical component of the motion vectors used for inter-layer prediction shall be equal to or less than 56 in units of luma samples. When inter_view_mv_vert_constraint_flag is equal to 0, no constraint for of the vertical component of the motion vectors used for inter-layer prediction is signalled by this flag. When not present, the inter_view_mv_vert_constraint_flag is inferred to be equal to 0.

collocated_picture_constraint_flag equal to 1 specifies that collocated picture are constrained in the CVS. When collocated_picture_constraint_flag is equal to 1, the collocated picture shall be the refernce picture with nuh_layer_id not equal to that of current picture. When collocated_picture_constraint_flag is equal to 0, no constraint for the collocated picture is signalled by this flag. When not present, the collocated_picture_constraint_flag is inferred to be equal to 0.

- G.11.1.2  Stereo Main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- The bitstream shall contain one layer with nuh_layer_id equal to i for which ViewId[ i ] is greater than 0.

- When ViewId[ i ] is greater than 0, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS of any coded pictures with nuh_layer_id equal to i.

- When ViewId[ i ] is greater than 0, collocated_picture_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS of any coded pictures with nuh_layer_id equal to i.

Appendix1: Text changes for 2nd method

( Note: move low-delay checking process to slice-level, and use a flag as indicator. )

##### 8.3.4 Decoding process for reference picture lists construction

…

If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice, or DiffPicOrderCnt( colPic, currPic ) is equal to 0, a variable low_delay_falg is set to 1, otherwise the low_delay_falg is set to 0.

##### 8.5.3.2.8  Derivation process for collocated motion vectors

The variables mvLXCol and availableFlagLXCol are derived as follows:

…

  - Otherwise (predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 1), the following assignments are made:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,if low_delay_flag is equal to 1, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColPb ][ yColPb ], refIdxLXCol[ xColPb ][ yColPb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol, and listCol are set equal to mvLNCol[ xColPb ][ yColPb ], refIdxLNCol[ xColPb ][ yColPb ], and LN, respectively, with N being the value of collocated_from_l0_flag.

and mvLXCol and availableFlagLXCol are derived as follows:

…