| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0070r1 M28873r1 |
| --- | --- |

| Title: | AHG8 Revision on Depth View Information SEI message in 3D-AVC DT5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Yotsuya-dori 1-13, Chikusa-ku, Nagoya 464-0819, Japan | Tel: Email: | +81-42-327-7262 {senoh, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-781-1883 tanimoto@nagoya-u.ac.jp, kazuyoshi-suzuki@cb3.so-net.ne.jp |
| Source: | National Institute of Information and Communications Technology(NICT), Nagoya Industrial Science Research Institute(NISRI), |  |  |

_____________________________

# Abstract

Since the current description on depth view information SEI message in 3D-AVC DT5 is editorially insufficient, this contribution proposes a revised description.

# Proposed modification

J.13.1.2 Depth view information SEI message syntax should be re-written as follows.

      - Depth view information SEI message syntax

| depth_view_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| } |  |  |
| } |  |  |

J.13.2.2 Depth view information SEI message semantics should be re-written as follows.

      - Depth view information SEI message semantics

The depth view information SEI message specifies different types of 3D format. The depth view information SEI message when present shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence. The depth view information SEI message shall only be used when VSP is disabled.

depth_type specifies the type of the depth view information. When depth_type is equal to 0, the SEI message signals Global View and Depth (GVD) format. When depth_type is equal to 0, inter-view prediction shall be disabled. The values of depth_type larger than 0 are reserved.

GVD is an Alternative 3D Format that removes inter-view redundancy by global view and depth. GVD makes encoding and decoding simple and fast by executing view synthesis prediction in frame layer. Decision whether some area is redundant or not is made by depth processing.

In GVD format (depth_type=0), input N texture views and associated N depth maps (N=2, 3, 4 and up to 5) are converted into Global texture View (one base texture view and one residual texture view) and Global depth map (one base depth map or one base depth map plus one residual depth map). They are encoded/ decoded and used for synthesis of target multi-views as illustrated in Figure 1.

Figure 1 – Global view and depth (GVD) format for depth_tytpe=0

Base texture view (B) is the input central texture view as shown in Figure 2. In 2-view case, view ID=2 and 3 are used. In 3-view case, view ID=2, 3 and 4 are used. In 4-view case, view ID=1, 2, 3 and 4 are used. In 5-view case, view ID=1, 2, 3, 4 and 5 are used. If the number of input texture views is even, base texture view is the first right view from the center. The other N-1 input texture views are converted to N-1 half-size sub-residual texture views (R-x) and packed in the residual texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. The residual texture view represents occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views. The number of sub-residual texture views is signalled by num_residual_texture_views_minus1.

num_residual_texture_views_minus1 plus1 specifies the number of sub-residual texture views packed in the residual texture view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive.

Figure 2 – Frame packing structure of global texture view

Base depth view (B) is an average of all input depth views as shown in Figure3. Residual depth view may exist or may not exist. The existence of residual depth view is signalled by residual_depth_flag. When residual_deptrh_flag = 1, residual depth view exists and the base depth view is associated with the base texture view. The other N-1 input depth views are converted to N-1 quarter-size sub-residual depth views (R-x) and packed in the residual depth view in the same positions as the sub-residual texture views are. Each sub-residual depth view associates with its sub-residual texture view. When residual_deptrh_flag = 0 and the number of input views is even (2, 4), base depth view is the center of input views (V2 and V3).

Since the input depth views are originally sub-sample to the half-size of the input texture views, the sub-residual depth views are sub-sampled to the quarter-size of input texture views. The residual depth view represents occluded area or out-of-frame area of the base depth view when it is projected to the N-1 input depth views.

residual_depth_flag specifies the existence (=1) or absence (=0) of residual depth view.

Figure 3 – Frame packing structure of global depth view

- View synthesis for Global View and Depth format

- (1) When residual_depth_flag = 0, the target view is synthesized as follows.

- Decoded base depth view is up-sampled to the size of the base texture view. Then, the up-sampled depth map is projected to the target viewpoint according to equation (1). Where f is camera focal length. B is baseline length between the base depth view and the target synthesis view. Znear and Zfar are the nearest object distance and the farthest object distance. d(x, y) is the base depth value. D(x, y) is the depth value of the target view. Equation (1) is for the case where target view resides on the right had side of the base depth view. If the target view resides on the other side, D(x+Δ, y)=d(x,y) is used.

- Δx=fB255(1Znear−1Zfar)d(x,y)+fBZfarD(x−Δx,y)=d(x,y)	(1)

- If residual_depth_flag = 0 and num_residual_texture_views_minus1 = 0 or 2, the base depth view is in the center of all texture views. Otherwise, the base depth view is associated with the base texture view. After the forward projection, holes are inpainted with the smaller sample at the left or right edge of the hole.

- Then, the base texture view is inversely projected to the target view. Before the base texture view projection, occlusion holes are detected by checking the target depth values D(x, y) using the following equation, where B is the baseline length between the target view and the base texture view and th is 5 (view synthesizer dependent).

- for(0≤δ≤fBZnear)if Δ=(D(x+δ,y)−D(x,y))≥th, (x,y) might be in an occlusion hole.	(2)

- Then, the disparity differenceεis calculated from the depth difference Δ as follows, where k = 0.6 (view synthesizer dependent) is a margin for the mismatch between the depth map and texture.

- ε=fB255(1Znear−1Zfar)Δ+fB255∗1Zfarif[Δ≥δ, or D(x+ε,y)−D(x,y)≥k∗Δ], (x,y) is in an occlusion hole.	(3)

- If (x, y) is in an occlusion hole, the pixel (x, y) is not projected but is registered to an occlusion hole mask. Equations (2) and (3) are for the case where the target view resides on the left hand side of the base view. They include some tolerance for depth error. If the target view resides on the right hand side of the base view, x-δ instead of x+δ is used in Equations (2) and (3). The target texture view is synthesized in sub-pixel accuracy by averaging two contiguous pixels in the base texture view, weighted by the mantissa of the disparity value calculated from the target depth value. The area that is not covered by the base texture view is also registered to the occlusion hole mask. Then, the occlusion hole mask is dilated by two pixels, allowing the mismatch between the target depth map and target texture. Then, the decoded sub-residual texture view, which is closest to the target view but located on the opposite side of the target view relative to the base view, is up-sampled to the same size as the base texture view. Then, it is projected to the occlusion hole in the target texture view. During the projection, the occlusion hole in the sub-residual texture view is detected and registered to a common hole mask in the same way as for the base texture view projection. Then, the common hole area in the target view is inpainted by the surrounding pixels.

- (2) When residual_depth_flag = 1, the target view is synthesized as follows.

First, the base depth view is up-sampled to the same size as the base texture view. The base depth view is associated with the base texture view. Then, the up-sampled base depth is projected to the target viewpoint in the same way as case (1). In addition, the up-sampled base depth is projected to the nearest sub-residual texture viewpoint, which resides on the opposite side of the target view relative to the base view, for generating residual hole masks. Next, the decoded sub-residual depth view, which is closest to the target view but residing in the opposite side of the target view comparing to the base view, is up-sampled repeatedly to the same size as the base texture view in the same way as for the base depth. Then, the residual area of the up-sampled residual depth is projected to the target viewpoint by using the generated residual hole masks. After that, remaining holes in the target depth are inpainted with the smaller sample at the left or right edge of the hole. Then, the base texture view is inversely projected to the target view. Before base texture view projection, occlusion holes are detected in the same way as for the case (1). Then, the occlusion hole mask is dilated by two pixels allowing for a mismatch between the depth map and texture. Then, the decoded sub-residual texture view, which resides closest to the target view but on the opposite side of the target view relative to the base texture view, is up-sampled to the same size as the base texture view. Then, it is backward projected to the occlusion hole in the target view. During the projection, the occlusion hole in the projected residual view is detected and registered to a common hole mask in the same way as for the base view projection. Then, the common hole is inpainted by the surrounding pixels.

Further details are described in the following references.

JCT3V-D0071| ISO/IEC JTC1/SC29/WG11 M28874, AHG08 Description of Global View and Depth Format for Depth View SEI Message (April, 2013)

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) and NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).