| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0070 M28873 |
| --- | --- |

| Title: | AHG8 Revision on Depth View Information SEI message in 3D-AVC DT5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Zehn 101, Kawana-cho 2-64, Showa-ku, Nagoya, 466-0856, Japan | Tel: Email: | +81-42-327-7262 {senoh, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-223-6639 tanimoto@nagoya-u.ac.jp, kazuyoshi-suzuki@cb3.so-net.ne.jp |
| Source: | National Institute of Information and Communications Technology(NICT), Nagoya Industrial Science Research Institute(NISRI), |  |  |

_____________________________

# Abstract

Since the current description on depth view information SEI message in 3D-AVC DT5 is editorially insufficient, this contribution proposes a revised description.

# Proposed modification

J.13.2.2 Depth view information SEI message semantics should be re-written as follows.

      - Depth view information SEI message syntax

| depth_view_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| num_residual_depth_views | 5 | ue(v) |
| } |  |  |
| } |  |  |

      - Depth view information SEI message semantics

The depth view information SEI message specifies different types of 3D format. The depth view information SEI message when present shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence.

When depth_type is equal to 0, this SEI message signals Global View and Depth (GVD) type of 3D format. In this case, the 3D format data contains two texture views and one or two depth views. The coded video sequence contains information derived from up to five original texture views and five original depth views which are coded into two texture views and up to two depth views.

When this SEI message is present, inter-view prediction for either texture or depth views is disabled.

depth_type specifies the type of the depth view information. When depth_type is equal to 0 (GVD), the different 3D format contains coded data corresponding to texture information from up to five views and depth information from up to five depth views. The non-base texture view composites texture information from up to four views, each of which has a quarter resolution of a texture view component of base view. The non-base depth view composites depth information from up tp four views, each of which has a quarter resolution of a depth view component of base view. The values of depth_type larger than 0 are reserved.

num_residual_texture_views_minus1 plus1 specifies the number of texture views packed into a texture view component of the non-base view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive. Each view that is packed into a texture view component of the non-base view has a quarter resolution of that of the textre view component of the base view. The available quarter-resolution views are present in a texture view component of the non-base view in the vertical raster scan order. [Ed.(CY): specifying the dummy region is unnecessary for the decoder.]

num_residual_depth_views specifies the number of depth views packed into a depth view component of the non-base view. The value of num_residual_depth_views is in the range of 0 to 4, inclusive. When num_residual_depth_views is equal to 0, there is only one depth view present in the coded video sequence. Each view that is packed into a depth view component of the non-base view has a quarter resolution of that of the depth view component of the base view. The available quarter-resolution views are present in a depth view component of a non-base view in the vertical raster scan order. [Ed.(CY): specifying the dummy region is unnecessary for the decoder.]

[Ed. (CY): more informative text that may be present e.g., in a note needs to be provided later.]

NOTE: each quarter-resolution texture or depth view that is packed into a texture or depth view component of the non-base view may be the residual signal between the current view and the base view and has been further down-sampled.

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) and NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).