| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0071r1 M28874r1 |
| --- | --- |

| Title: | AHG8 Description of Global View and Depth Format for Depth View SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, and Kenji Yamamoto, Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Yotsuya-dori 1-13, Chikusa-ku, Nagoya 464-0819, Japan | Tel: Email: | +81-42-327-7262 {senoh, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp, +81-52-781-1883 tanimototentative3@yahoo.co.jp kazuyoshi-suzuki@cb3.so-net.ne.jp |
| Source: | National Institute of Information and Communications Technology(NICT), Nagoya Industry Science Research Institute(NISRI) |  |  |

_____________________________

# Abstract

According to the following mandate of AHG8, this contribution describes Global View and Depth format for Depth View SEI message.

•	Produce an informative description to help thorough understanding of the depth view information SEI message.

# Introduction

Global View and Depth format is an Alternative 3D Video format for efficient 3D video coding. The features of Global View and Depth format are,

- Simple and fast since inter-view prediction is done by depth map in frame-base. No block-base inter-view prediction is required.

- Compact and light since the residual views for textures or depth maps are packed in a residual texture stream or a residual depth map stream. The resultant texture or depth map streams are compatible with the normal 3DV texture stream or depth stream such as 3D-AVC or 3D-HEVC.

Signaling of Global View and Depth format for view synthesis process is done by Depth View SEI message in 3D-AVC specifications or the same SEI message in 3D-HEVC specifications.

Global View and Depth format has two types, namely Type-1 and Type-2. Type-1 merges the input depth maps into a global depth map. Type-2 generates residual depth map besides the global depth map.

The following sections describe the encoding, decoding and view synthesis process for Type-1 and Type-2 of Global View and Depth format.

# Type-1 Global View and Depth format

## Data Format and System Description for type-1

Input Data are multi-view textures and the associated depth maps as illustrated in Figure 1. The global view and depth codec consists of global depth generator, residual view generator, core codecs, and view synthesizer. Global view is a logical set of the base view, which is one of the input view, and the residual view, which is generated by the residual view generator. The core codec is 3D-AVC or 3D-HEVC. Currently supported number of views are 2 and 3.

Global depth generator generates, encodes and decodes a global depth map from the input depth maps. Since the residual view generator has no interaction mechanism with the core codec, global depth map is encoded and decoded once in the global depth generator. Then, residual view generator generates a residual view from the input multi-views based on the decoded global depth map given by the global depth generator. After that, the core codec encodes and decodes the base view, residual view and global depth map together. View synthesizer synthesizes multiple views from the decoded base view, global depth map and residual view. Base view is forward-compatible with the conventional 2D codec such as MPEG-4 AVC or HEVC.

Figure 1: Overview of system structure and data format for Type-1 Global View and Depth format

## Coding Algorithm

### Generation of Global Depth

For 2-view case, global depth map is an average of input depth maps after projecting them to the center of the two input views as shown in Figure 2(a). The depth value d1(x1, y) in the left depth map and d2(x2, y) in the right depth map are projected to the global depth values dgi(xgi, y) in the left global depth map and the right global depth map, respectively according to the following equations. The holes in each projected depth map are inpainted with the smaller depth value in left or right of the hole. Then two projected global depth maps are averaged.

Δxi=fB255(1Znear−1Zfar)di(xi,y)+fBZfar (i=1,2)dg1(x1−Δx12,y)=d1(x1,y), dg2(x2+Δx22,y)=d2(x2,y)dg(x,y)=dg1(x,y)+dg2(x,y)2	(1)

For3-view case, the global depth map is a median of input depth maps after projecting them to the center view by the following equations. Since the input depth maps may include some depth errors, generated global depth maps increase the reliability.

Δxi=fB255(1Znear−1Zfar)di(xi,y)+fBZfar (i=1,3)dg1(x1−Δx1,y)=d1(x1,y), dg3(x3+Δx3,y)=d3(x3,y)dg(x,y)=mediandg1(x,y), d2(x,y), dg3(x3,y)	(2)

(a) for 2view case                            (b) for 3view case

Figure 2 Global depth map generation from the input depth maps

Generated global depth map is sub-sampled to half horizontally and vertically and then it is encoded and decoded by a codec in the global depth generator, which function ois same as the core codec. The decoded depth map is used by the residual view generator.

Figure 3: Global depth coding

### Generation of Residual view

Since the base view covers most of the area of the other views when it is projected to the other views based on the depth map, Residual view keeps only the uncovered area. This uncovered area (residual view) is derived as illustrated in Figure 4 and 5. In order to avoid mismatch between the encoder and decoder, locally decoded global depth map is used to detect the residual area. First, decoded global depth map is up-sampled to the original size, and then projected to the other view (Left view or Right view). During the up-sampling, simple 2-tap filter generates in-between pixels if contiguous two pixel difference is smaller than a threshold (=5, encoder dependent). If not, larger pixel is copied (depth dilation). After that, the depth map is smoothed by 5x5 median filter. Median filter smoothens slant depth edges. Dis-occluded area and uncovered area in the projected depth map is inpainted with the smaller depth value on the left or right of the hole.

Figure 4: Residual view for 2-view case

Figure 5: Residual view for 3-view case

Then, occlusion holes are detected by checking the depth map as follows. If the base view is on the right hand side of the currently checking depth map as shown in Figure 6, an occlusion hole exists where a foreground object (large depth value) exists on the right of the currently checked pixel. The pixel is in the occlusion hole or not is determined by the following steps.

- Start from the maximum disparity distance δ= (fB/(1/Znear-1/Zfar), compare two depth values of dx (x, y) and dx+δ(x+δ, y). If (dx+δ- dx) >= th, pixel (x, y) may be in an occlusion hole. th is given in the following equation. Where f=focal length and B= baseline length.

if (dx+δ−dx)≥th, x is in occlusion hole.th=255fB(1Znear−1Zfar)δ, (0≤δ≤fBZnear−fBZfar)	(3)

- If not, reduceδby 1/8 and repeat step (1). This repetition detects skinny foreground objects like poles.

- When the pixel is possibly in a occlusion hole, further check the depth value of d x+ε(x+ε, y). εis a disparity value of (dx+δ- dx) and given in the following equation.

ε=fB255(1Znear−1Zfar)(dx+δ−dx)	(4)

- If (dx+ε- dx) >= kth, the pixel (x, y) is registered to the hole mask. Where k is 0.5 and saves the decay of depth edge.

- Uncovered area on the left edge in the depth map is also registered to the hole mask. This area is detected by checking the address of (x-Δ)≥0 in equation (1).

- If the base view is in the left side of the currently checked depth map, the above steps are repeated by changing the +δto –δ, +εto –ε, and –Δ to +Δ.

Figure 6: Occlusion hole detection

This occlusion hole detection is repeated at four view points (D1.0, D1.25, D1.5, D1.75) for left residual view and at another four view points (D2.25, 2.5, 2.75, 3.0) for right residual view. These repetition ensures that the residual view covers all holes including pseudo common holes caused by the wrong depth values. Generated hole masks are projected to the nearest left view (V1) or the right view (v3) and merged there. After that, they are dilated by 8 to increase the margin of residual views. With the merged hole masks (M1 and M3), left view and right view are cut out to the left residual (R1) and the right residual (R3). These residuals are down-sampled to half vertically and horizontally after filtered with a (1/4, 1/2, 1/4)-tap low pass filter and then stuffed into a frame in a vertical scan order, from top-left, bottom-left, top-right, and bottom-right. The no-pixel area is paved with the average color value of the input views. This paving eases the artifacts of no-pixel area in the target view at the decoder. Three pixels at the edge of no-pixel area is linearly smoothed to increase the coding efficiency. Then the residual view is encoded and decoded by the core codec independently.

Figure 7: Residual view coding

### Coding of Base View, Global depth and Residual view

Base view is the right view texture for 2-view case and the center view texture for 3-view case. The base view, residual view and global depth map are encoded and decoded by the core codec. The base view is independently encoded and keeps forward compatibility with the conventional 2D codecs such as MPEG-4/AVC(ITU-T H.264) or HEVC.

## View synthesis

After decoding bitstreams, View synthesizer synthesizes target views including left view and right view from decoded base view, global depth map and residual view. Figure 8 illustrates a case where target view is on the left of the base view. First, decoded global depth map is up-sampled (D2’) and projected to the target view (Dt) as the same manner as in the residual coding process. Next, decoded base view (V2’) is projected to the target view (Vt) by the target depth map (Dt). During the projection, occlusion holes are detected and registered to the base hole mask (Mb) as same manner as in the residual coding process. Then, decoded residual view is separated to left residual view and right residual view. They are up-sampled to the original size (R1’, R3’) with a (1/2, 1/2)-tap filter by smoothing the no-pixel edge. The up-sampled residual (R1’) is projected to the target view (Rt). The projection of base view and residual view is done by up-sampling Chroma components to the original Luma component size for a simplicity. View projection with sub-pixel accuracy is realized by the interpolation from two contiguous pixels according to the mantissa of disparity value calculated from the depth value.

Then, the base hole mask is dilated by two pixels to cover the mismatch between the depth map and texture. Dilated base hole mask limits the residual projection only inside of the base hole mask. This limitation increases the view synthesis speed. During the residual projection, occlusion holes are detected and registered to the residual hole mask (Mr) as well. Projected residual (Rt) is over-written on the hole area in the target view (Vt) according to the base hole mask (Mb). After that, common hole mask is created from the base hole mask and the residual hole mask. Common holes are inpainted according to the common hole mask. Then, each pixel on the both sides of the base hole mask edge are filtered by (1/4, 1/2, 1/4)-tap filter to reduce the hole edge contour if their difference is smaller than 20. If the difference is equal or larger than 20, they are left as the object edge. Finally the target view is out-put as a synthesized view (St). The right target view is synthesized in the same manner.

Figure 8: View synthesis

# Type-2 of Global View and Depth format

## Data Format and System Description for Type-2

Input data format is same as those of 3D video coding. Figures 9 shows Type-2 Global View and Depth data converted from input MVD data for two or three viewpoints, where the parameters W and H show the width of input full-size view and the height of input full-size view, respectively. Type-2 GVD data also consists of global view and global depth. Furthermore, the Type-2 global view consists of a base view and residual views as same as that for Type-1 global view. On the other hand, Type-1 global depth consists of only base depth, while the Type-2 global depth consists of base depth and residual depth.

Figure 9: Type-2 GVD data format

Figure 10 shows an overview of system structure for Global View and Depth data format. The system consists of Codec, Base view generator, Base depth generator, Residual view generator, Residual depth generator and View synthesizer. The codec is 3DV-AVC or 3D-HEVC. Currently supported number of views are 2 and 3.

The function of base depth generator is completely same as that of the global depth generator for the Type-1 GVD data format described in Section 2.1. Residual depth generator generates, encodes and decodes a residual depth map from the input depth maps V1 and V3 and the decoded base depth that was output from the base depth generator. The residual depth map is encoded and decoded once in the residual depth generator. Then, residual view generator generates a residual view from the input multi-views based on the decoded base depth map and residual depth map given by the base depth generator and residual depth generator. View synthesizer synthesizes multiple views from the decoded base view, base depth map, residual view and residual depth map. Base view is forward-compatible with the conventional 2D codec such MPEG-4 AVC or HEVC.

Figure 10: Overview of system structure for Type-2 Global View and Depth format

## Coding Algorithm

### Generation of Base Depth

The method for generating base depth of the Type-2 global depth is same as that for Type-1 global depth. The base depth map is an average of all input depth maps by projecting them to the base depth point, or may be the original depth map at the base viewpoint. The obtained base depth is sub-sampled to quarter size.

### Generation of Residual Depth

Since base view can cover most of the area of the other views when it is projected to the other views based on the base depth map, residual view and residual depth map keep only the uncovered area. Depth information, which is used for filling this uncovered area, is derived as illustrated in Figures 11 and 12. The depth information is residual depth map. It should be noted that in order to avoid mismatch between hole mask for residual depth generation at the encoder side and that at the decoder side, locally decoded base depth map is used to detect the residual area in non-base depth maps. The detail of residual depth generation process is as follows.

- Decoded base depth is up-sampled to that of the original size. During the up-sample, simple 2-tap filter generates in-between pixels if contiguous two pixel difference is smaller than a threshold (=5). If not, larger pixel is copied (depth dilation).

- Shift the up-sampled base depth to Left or Right viewpoint.

- The shifted depth map is smoothed by 3x3 median filter. Median filter smoothens slant depth edges.

- Dis-occluded areas or uncovered areas are detected from the smoothed depth map and are registered to a hole mask for left or right residual depth, which are dilated by 4 pixels to add a margin to the residual depth.

- By using the left or right hole mask that was obtained in (4), left depth or right depth is cut out to the left or right residual depth, respectively. The no-pixel area is paved with the average depth value of base depth.

- The left or right residual depth that was obtained in (5) is down sampled to 1/4 vertically and 1/4 horizontally after filtered with a three-tap low pass filter and stacked to a frame. Finally, the residual depth are packed into a frame of quarter of the full frame size in the vertical raster scan order.

Here, it should be noted that hole masks for residual depth are not transmitted to a receiver side but are reconstructed at the receiver side.

Figure 11: Residual depth generation for 2 view case

Figure 12: Residual depth generation for 3 view case

### Generation of Residual View

The method for generating Type-2 residual views is similar to that for Type-1 residual views. The area, which is uncovered by the base view, is derived as illustrated in Figures 13 and 14. In order to avoid mismatch between hole mask for residual view generation at the encoder side and that at the decoder side, decoded base depth and decoded residual depth are used to detect uncovered area. The detail of residual view generation process is as follows.

- Decoded base depth and decoded residual depth are up-sampled to the original size as same as the residual depth generation process.

- The up-sampled base depth is shifted to Left or Right view and hole masks for Left or Right residual depth are generated as same as the residual depth generation process.

- The up-sampled base depth is also shifted to intermediate target viewpoints. Hole pixels in each shifted depth map are detected and are registered to a temporal hole mask, which is smoothed by 3x3 median filter and is dilated by 2 pixels. It should be noted that each depth map in intermediate viewpoint is not smoothed in this step yet.

- Only the residual pixels of Residual Left or Right depth is shifted to each target viewpoint by using their residual hole masks. Only the pixels of depth map in intermediate viewpoint, which are considered hole pixels in the smoothed hole mask in (3), are filled with the shifted residual pixels.

- The depth map that was obtained in (4) are smoothed by 3x3 median filter. Remaining hole pixels of the depth map are filled by inpainting with the smaller depth value between existing left and right pixels around the hole.

- For each intermediate viewpoint, the pixels that were not filled with base depth but filled with residual depth or inpainting are backwardly warped to the left or right view. For each intermediate viewpoint, corresponding pixels in the left or right view are detected and are registered to a temporal hole mask for residual view. The temporal hole mask is smoothed by 3x3 median filter.

- The hole masks that were obtained in (2) and (6) are merged and are dilated by 4 pixels to add a margin to the residual depth.

- By using the hole mask that was obtained in (7), left or right view is cut out to left or right residual view, respectively. The no-pixel area is paved with the average value of base view.

- The left or right residual view that was obtained in (8) is down sampled to half vertically and half horizontally after filtered with a three-tap low pass filter and stacked to a frame. Finally, the residual view are packed into a frame of the original frame size in the vertical raster scan order as shown in Fig. 16.

Here, it should be noted that hole masks for residual views are not transmitted to a receiver side but are reconstructed at the receiver side.

Figure 13: Residual view generation for 2 view case

Figure 14: Residual view generation for 3 view case

## View Synthesis

At the decoder side, view synthesizer synthesizes target views including left and right views from decoded base view, base depth, residual view and residual depth. Figure 15 illustrates a case where target view is on the left of base view. The detail of view synthesis is described in below.

- Decoded base depth is up-sampled and shifted to the target view as same manner as in the residual depth and residual view generating process. Then, hole masks for target views are generated

- Decoded residual depth is separated to those for non-base viewpoints. The residual depth in each non-base viewpoint is up-sampled and shifted to target views by using hole masks generated in (1) as same manner as in the residual view generating process.

- Remaining hole pixels of target views are filled by inpainting.

- Decoded base view is backwardly projected to target views by using the target depth and the hole mask that were generated in (1).

- Decoded residual view is separated to those for non-base viewpoints, which are up-sampled to the original size with simple two-tap filter. The up-sampled residual is backwardly projected to the target view by the target depth map generated in (2) and the hole mask generated in (1).

- Common hole mask is created from the base hole mask and the residual hole mask. Common holes are inpainted according to the common hole mask. Then, two pixels on the both sides of base hole mask edge are filtered by three-tap filter to reduce the hole edge contour. Finally the target view is out-put as a synthesized view. The right target view is synthesized in the same manner.

Figure 15. View Synthesis

# Software

## Software

The source code for the software will be available in the MPEG SVN repository. A preliminary version of the software is available upon request by e-mail.

tanimototentative3@yahoo.co.jp for Type-2 global view and depth format

senoh@nict.go.jp for Type-1 global view and depth map format.

## Build system

The software can be built under Windows using Microsoft Visual Studio.Net. The software is using Open CV2.2. For UNIX and Linux, makefiles are provided.

# Reference

[1] 	ISO/IEC JTC1/SC29/WG11/M26014[JCT2-A0072]: " Proposal on Simple Test Model for 3DV Coding based on CE7," (July 2012)

[2] 	ISO/IEC JTC1/SC29/WG11/M27793[JCT2-C0058]: "AHG08: Technical Description of residual data generation and target view synthesis in GVD (Global View and Depth) 3D Format," (Jan 2013)

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).