| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, Switzerland, 17–23 Jan. 2013 | Document: JCT3V-D0073 |
| --- | --- |

| Title: | AHG4: Interlace coding on ATM 7.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ching-Chieh Lin, Fang-Chu Chen, Chao-Hsiung Hung | Tel: Email: | +886-3-5914782 jacklin@itri.org.tw fcchen@itri.org.tw chhung@itri.org.tw |
| Source: | Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This document describes the modifications that have been added to ATM 7.1, which only supports frame coding, to enable field coding for the current released ATM 7.2.

# Introduction

The lack of interlace support for ATM as MVC+D reference software has been discussed ever since the the 2nd JCT-3V meeting. At the third JCT-3V meeting in Geneva, ITRI demonstrated that it was possible to modify ATM for field coding with modified HP configurations [1]. It was then agreed to choose ATM over JMVC as a candidate for MVC+D reference software, pending the completion of the integration work. At this time of writing, just before the fourth JCT-3V meeting, interlace coding has been successfully integrated into ATM and been adopted and released as ATM 7.2. This document provides the details of modifications that have been added to ATM 7.1, as well as the operation of ATM 7.2 to enable interlace coding. Please note that some of the descriptions refer to the previous contribution JCT3V-C0069 [1] where the work on interlace integration to ATM was still in progress.

The current version of ATM 7.2 supports the following cases for both encode and decode:

- Case 1: coding progressive video with progressive depth map for MVC+D

- Case 2: coding stereoscopic interlace video with interlace depth map for MVC+D

- Case 3: coding stereoscopic interlace video with progressive depth map for MVC+D

- Case 4: coding progressive video with progressive depth map for 3D-AVC

# Issues of field coding with ATM V7.1 and its solutions

The issues with interlace support of ATM V7.1 are as follows:

- Field coding is specifically not allowed

refer to JCT3V-C0069 [1]

- Field based structure or variables are marked off

refer to JCT3V-C0069 [1]

- Some field coding source code is eliminated

refer to JCT3V-C0069 [1]

- Original field coding source code needs modifications for depth

refer to JCT3V-C0069 [1]

- Reference list and reordering issues

refer to JCT3V-C0069 [1]

- GOP issues

refer to JCT3V-C0069 [1]

- No support for different setting of texture and depth

In the original ATM, some of the setting for depth is directly copied from that of texture, such as: level idc (LevelIDC) and number of reference frames (num_ref_frames). This introduces problems in the case of interlaced texture and progressive depth. To be more specific, for interlace coding the maximum LevelIDC value is limited to 41. However, for progressive it can be up to 51, the value used by CTC. A similar limitation can be seen on number of reference frames where interlace coding usually uses less number than progressive coding. Since it is important to ensure the best possible coding quality for depth, so different setting should be allowed in the case of interlace texture plus progressive depth coding. Therefore we modified the software so that for MVC+D the setting for the depth is directly read from the depth configuration file instead of copy from texture, to allow for different setting for texture and for depth.

- Write out order issues

According to MVC+D specification[2], there is a specified access unit format for each combination of interlace and progressive texture and depth. For example, for interlaced texture and interlaced depth, a depth field view component, with a particular view_id and POC, shall be present in the same access unit as the coded texture field view component with the same view_id and POC. In the original ATM, the write out of a picture follows directly after the completion of the coding of this picture. For the case of interlace coding, the two fields of the same texture picture are coded together before the depth picture can be coded. However, the coded fields of the texture cannot be written out until the corresponding depth fields are coded. Therefore the coding flow has to be modified so that the coded texture fields are put into buffer to wait for the completion of depth fields coding so that they can be packed together according to the specification. The flow is, again, different and needs further modifiation for the case of interlaced texture and progressive depth.

- Decoder

The following issues with the encoder also exist for the decoder in the original ATM:

  - Some field coding source code is eliminated

  - Original field coding source code needs modifications for depth

  - No support for different setting of texture and depth

The measures to resolve the issues in the decoder are very similar to those used in encoder, so will not be elaborated here in this document.

# The interlace coding configuration for ATM 7.1

The main changes of the configurations for interlace coding can be summarized in the flowing tables:

For interlaced texture:

Note that, for class A, since the memory requirement is big, so the software can only work with GOP = 4 and NumberBFrame = 3. However for class C, GOP can be set to 8 and NumberBFrames set to 7, as with the following setting :

  - NumberBFrames          = 7

  - ExplicitHierarchyFormat = "B3r4t1B1r5t2B0e6t3B2e6t3B5r5t2B4e6t3B6e6t3"

For interlaced depth

To accommodate these modifications in HP configurations, four configuration files have been created to go with the software. The usage guide is as follows:

The four configuration files designated for interlace coding mode are as follows:

-configs\3dv_enc_25Hz_depth_interlace_HP.cfg

-configs\3dv_enc_25Hz_texture_interlace_HP.cfg

-configs\3dv_enc_30Hz_depth_interlace_HP.cfg

-configs\3dv_enc_30Hz_texture_interlace_HP.cfg

Commands for execution of encoding:

- Example of command line for encoding progressive texture with progressive depth map:

lencod-R.exe -d "..\3dv_enc_30Hz_texture_HP.cfg" -depd "..\3dv_enc_30Hz_depth_HP.cfg" -p OutputFile="test_texture.264" -p 3DVConfigFile="..\3dv_texture_kendo.cfg" -depp 3DVConfigFile="..\3dv_depth_kendo.cfg" -p FramesToBeEncoded="300" -p QPISlice="41" -p QPPSlice="41" -p QPBSlice="41" -depp QPISlice="41" -depp QPPSlice="41" -depp QPBSlice="41" > HPC3S05R1.log

- Example of command line for encoding stereoscopic interlace texture with interlace depth map:

lencod-R.exe -d "..\3dv_enc_30Hz_texture_interlace_HP.cfg" -depd "..\3dv_enc_30Hz_depth_interlace_HP.cfg" -p OutputFile="test_texture.264" -p 3DVConfigFile="..\3dv_texture_kendo.cfg" -depp 3DVConfigFile="..\3dv_depth_kendo.cfg" -p NumberOfViews=2 -p 3DVCodingOrder="T0D0D1T1" -depp NumberOfViews=2 -p FramesToBeEncoded="300" -p QPISlice="41" -p QPPSlice="41" -p QPBSlice="41" -depp QPISlice="41" -depp QPPSlice="41" -depp QPBSlice="41" > HPC2S05R1.log

- Example of command line for encoding stereoscopic interlace texture with progressive depth map:

lencod-R.exe -d "..\3dv_enc_30Hz_texture_interlace_HP.cfg" -depd "..\3dv_enc_30Hz_depth_HP.cfg" -p OutputFile="test_texture.264" -p 3DVConfigFile="..\3dv_texture_kendo.cfg" -depp 3DVConfigFile="..\3dv_depth_kendo.cfg" -p NumberOfViews=2 –p 3DVCodingOrder="T0D0D1T1" -depp NumberOfViews=2 -p FramesToBeEncoded="300" -p QPISlice="41" -p QPPSlice="41" -p QPBSlice="41" -depp QPISlice="41" -depp QPPSlice="41" -depp QPBSlice="41" > HPC2S05R1.log

The bit-rate and PSNR values in the coding reporting excel files are extracted from the encoder console which is directed to a log file, such as “HPC2S05R1.log,” in the above command line.

# Conclusion and Future Work

The interlace coding has been integrated into ATM as ATM 7.2. It can now be used as the software to generate conformance bitstreams for MVC+D.

# Reference

- Ching-Chieh Lin, Fang-Chu Chen “AHG4: MVC+D syntax mismatch with ATM 6.0 (frame coding)”, 3rd JCT-3V meeting, JCT3V-C0071, Geneva. Switzerland, Jan. 2013

- Miska M. Hannuksela, Ying Chen, Teruhiko Suzuki, Shinobu Hattori, Gary J. Sullivan ”Text of ISO/IEC 14496-10:2012/FDAM2 MVC extension for inclusion of depth maps“, ISO/IEC JTC1/SC29/WG11/N13327, Geneva, Switzerland, January 2013.

# Patent rights declaration(s)

Industrial Technology Research Institute may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).