| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0074 |
| --- | --- |

| Title: | MV-HEVC: On Marking for sub-layer non-reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang Nanjing R&D Center, 101 Software Avenue, Yuhuatai District, Nanjing, P.R.China 200012 | Tel: Email: | +86 25 56620730 yx.yangxiaofeng@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

The conditions that marking process for sub-layer non-reference pictures applied only for pictures with nuh_layer_id greater than 0 restricts the effect of DPB space saving, this contribution proposes also to apply the marking process for base-layer pictures.

# Introduction

Following texts from the current MV-HEVC specification defines the marking process for sub-layer non-reference pictures not needed for inter-layer prediction.

- F.8.1.2	Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

……

–	When TemporalId is equal to HighestTid, the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause Error: Reference source not found is invoked with latestDecLayerId equal to nuh_layer_id as input.

- F.8.1.2.1 Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

……

Section F.8.1.2.1 can be summarized as marking any previously decoded pictures of the current AU ‘unused for reference’ if all of the following conditions are true:

- The picture is a sub-layer non-reference picture with TemporalId is equal to HighestTid

- The picture is no referenced by any not-yet decoded pictures of the same AU

In essence, the current specification removes previously decoded pictures of the current AU that are not used for either inter prediction of inter-layer prediction as early as possible.

# Proposal

This contribution proposes to apply this process to BL pictures too instead only to EL pictures. So that the BL pictures that are not used for either inter prediction or interlayer prediction could be removed from DPB earlier.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).