| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0076 |
| --- | --- |

| Title: | CE6.H related: Results on Fixes for Region Boundary Chain Coding in 3D-HEVC draft specification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 15968882677 yiqiu8932@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This proposal fixes the misalignment between 3D-HTM software and 3D-HEVC draft specification for region boundary chain coding. To align 3D-HTM software with 3D-HEVC draft specification, this proposal added a condition before parsing the syntax element edge_intra_flag in General Coding unit syntax. Coding performance remains the same as the anchor since the software is not modified.

# Problem Statement

Boundary chain coding is a coding method for the depth-map, which can describe even a curved depth pattern that has weak or little inter-component correlation between texture and depth [1].

In 3D-HTM software, this coding tool is only applied for depth component. However, according to 3D-HEVC draft specification [2], the decoder will not check if the current coding unit belongs to depth component before parsing the syntax element edge_intra_flag, which results in the mismatch between software and specification.

# Proposed specification modification

Changes of the specification by this proposal are indicated with yellow highlighting base on [2].

In H.7.3.9.1	General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| ...... |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( DepthFlag && rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| …… |  |
| } |  |
| } |  |
| …… |  |
| } |  |

Coding performance remains the same as the anchor since the software is not modified.

# Patent rights declaration(s)

Zhejiang University does not have any current or pending patent rights relating to the technology described in this contribution.

[1] “CE6.h Region Boundary Chain Coding for Depth-map,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A0070, July 2012.

[2] “3D-HEVC Test Model Description, draft specification,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1005, Jan 2013.