| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0077 |
| --- | --- |

| Title: | CE6.H related: Results on Modified Binarization for Region Boundary Chain Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 15968882677 yiqiu8932@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This proposal modifies the binarization and CABAC context of the syntax element edge_code in region boundary chain coding. This proposal achieves 0.1% and 0.005% BD-rate gains compared to 3D-HTM6.0 under the common test condition (CTC) and all intra test condition with the same encoding/decoding time.

# Problem Statement

Boundary chain coding is a coding method for the depth-map, which can describe even a curved depth pattern that has weak or little inter-component correlation between texture and depth [1].

In boundary chain coding, the syntax element edge_code is used to derive the edge direction as Table 1-1 shows. Boundary chain coding supports 7 relative edge directions: 0, 45, -45, 90, -90, 135, -135 degree.

Table 1-1. The relationship between edge_code and edge direction

| edge_code | edge direction |
| --- | --- |
| 0 | 0° |
| 1 | 45° |
| 2 | −45° |
| 3 | 90° |
| 4 | −90° |
| 5 | 135° |
| 6 | −135° |

Currently, truncated unary binarization is applied for the syntax element edge_code, as shown in Table 1-2. The same CABAC context is used for every bin of the bin string.

Table 1-2. Truncated unary binarization of edge_code

| edge_code | edge direction | bin string |
| --- | --- | --- |
| 0 | 0° | 0 |
| 1 | 45° | 10 |
| 2 | −45° | 110 |
| 3 | 90° | 1110 |
| 4 | −90° | 11110 |
| 5 | 135° | 111110 |
| 6 | −135° | 111111 |

However, statistical analysis shows that the occurring probabilities of the edge direction differ importantly. Larger relative edge directions have smaller occurring probabilities and the edge directions with the same absolute value but different sign have similar occurring probabilities. The occurring probabilities of each edge direction are shown in Table1-3.

Table 1-3. Occurring probabilities of each edge direction

| edge direction | occurring probability |
| --- | --- |
| 0° | ≈69% |
| 45° | ≈15% |
| −45° | ≈15% |
| 90° | ≈0.43% |
| −90° | ≈0.43% |
| 135° | ≈0.02% |
| −135° | ≈0.02% |

The statistical analysis implies that this characteristic can be utilized to achieve better coding performance.

# Proposed Method

To utilize the probability distribution in last section, a new method of binarization is proposed, as Table 2-1 shows.

Table 2-1. Proposed binarization of edge_code

| edge_code | edge direction | bin string |
| --- | --- | --- |
| 0 | 0° | 1 |
| 1 | 45° | 011 |
| 2 | −45° | 010 |
| 3 | 90° | 0011 |
| 4 | −90° | 0001 |
| 5 | 135° | 0010 |
| 6 | −135° | 0000 |

Assuming the probability of each direction is Pn (n=0,1,2,3,4,5,6). The probability that each bin is 1 can be calculated as:

The probability that the 1st bin is 1: P0 ≈ 69%

The probability that the 2nd bin is 1: (P1 + P2) / (P1 + P2 + P3 + P4 + P5 + P6) ≈ 97%

The probability that the 3rd bin is 1: (P1 + P3 + P5) / (P1 + P2 + P3 + P4 + P5 + P6) ≈ 50%

The probability that the 4th bin is 1: (P3 + P4) / (P3 + P4 + P5 + P6) ≈ 96%

According to the above calculation, bypass mode is used for the 3rd bin and 3 CABAC contexts are used for the 1st bin, the 2nd bin and the 4th bin, respectively. The initial value of the CABAC contexts can also be calculated using the probabilities above. More detail can be found in section 4 of this proposal.

# Experimental Results

Proposed method is implemented into HTM 6.0 software, and proposed method is compared with HTM 6.0. The test condition is CTC and All Intra Configuration.

Experimental Results show that the proposed method achieves 0.08% and 0.005% BD-rate gains compared to 3D-HTM6.0 under the common test condition (CTC) and all intra test condition with the same encoding/decoding time.

## Results of CTC

Table 3-1: BD rate results for proposed method (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.03% | -0.04% | -0.02% | -0.01% | -0.06% |
| Kendo | 0.00% | 0.04% | 0.03% | 0.01% | -0.03% | -0.04% |
| Newspaper_CC | 0.00% | -0.29% | 0.08% | -0.03% | 0.00% | -0.09% |
| GT_Fly | 0.00% | -0.03% | 0.08% | 0.01% | 0.03% | 0.06% |
| Poznan_Hall2 | 0.00% | 0.12% | 0.02% | 0.02% | -0.01% | -0.03% |
| Poznan_Street | 0.00% | 0.09% | 0.02% | 0.02% | 0.01% | 0.02% |
| Undo_Dancer | 0.00% | -0.02% | -0.10% | -0.01% | -0.01% | -0.38% |
| 1024x768 | 0.00% | -0.09% | 0.02% | -0.01% | -0.01% | -0.07% |
| 1920x1088 | 0.00% | 0.04% | 0.01% | 0.01% | 0.01% | -0.08% |
| average | 0.00% | -0.02% | 0.01% | 0.00% | 0.00% | -0.08% |

Table 3-2: Complexity results for proposed method (CTC)

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 87.72% | 100.08% |
| Kendo | 88.46% | 99.46% |
| Newspaper_CC | 81.71% | 103.79% |
| GT_Fly | 97.90% | 99.99% |
| Poznan_Hall2 | 90.76% | 103.12% |
| Poznan_Street | 92.24% | 96.15% |
| Undo_Dancer | 88.28% | 100.85% |
| 1024x768 | 85.96% | 101.11% |
| 1920x1088 | 92.29% | 100.03% |
| average | 89.58% | 100.49% |

## Results of All Intra Configuration

Table 3-3: BD rate results for proposed method (All Intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | 0.000% | 0.000% | 0.000% | -0.001% | 0.001% |
| Kendo | 0.000% | 0.000% | 0.000% | 0.000% | -0.001% | 0.005% |
| Newspaper_CC | 0.000% | 0.000% | 0.000% | 0.000% | -0.001% | -0.005% |
| GT_Fly | 0.000% | 0.000% | 0.000% | 0.000% | -0.001% | -0.004% |
| Poznan_Hall2 | 0.000% | 0.000% | 0.000% | 0.000% | -0.004% | -0.009% |
| Poznan_Street | 0.000% | 0.000% | 0.000% | 0.000% | -0.003% | -0.005% |
| Undo_Dancer | 0.000% | 0.000% | 0.000% | 0.000% | -0.007% | -0.015% |
| 1024x768 | 0.000% | 0.000% | 0.000% | 0.000% | -0.001% | 0.000% |
| 1920x1088 | 0.000% | 0.000% | 0.000% | 0.000% | -0.004% | -0.008% |
| average | 0.000% | 0.000% | 0.000% | 0.000% | -0.003% | -0.005% |

Table 3-4: Complexity results for proposed method (All Intra)

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 96.303% | 100.422% |
| Kendo | 98.695% | 100.388% |
| Newspaper_CC | 98.098% | 97.212% |
| GT_Fly | 100.429% | 95.233% |
| Poznan_Hall2 | 101.249% | 101.059% |
| Poznan_Street | 99.794% | 99.061% |
| Undo_Dancer | 96.597% | 99.624% |
| 1024x768 | 97.699% | 99.341% |
| 1920x1088 | 99.518% | 98.744% |
| average | 98.738% | 99.000% |

# Proposed specification modification

Changes of the specification by this proposal are indicated with yellow highlighting base on [2].

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | edge_code | Table Error: Reference source not found-2 | 0..2 | 3..5 | 6..8 |

Table Error: Reference source not found-2 – Values of variable initValue for edge_code ctxIdx

| Initialization variable | edge_code |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 155 | 191 | 111 | 155 | 191 | 111 | 155 | 191 | 111 |

Table Error: Reference source not found-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| edge_code | 0 | H.9.3.2.10 | 0 | Table Error: Reference source not found-2 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-2 | 3 |
|  | 2 |  | 0 | Table Error: Reference source not found-2 | 6 |

H.9.3.2.10 Binarization process for inter_pred_idc

Inputs to this process are a request for a binarization for the syntax element edge_code.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element edge_code is specified in Table H-XX.

Table H-xx – Binarization for edge_code

| Value of edge_code | Bin string |
| --- | --- |
| 0 | 1 |
| 1 | 011 |
| 2 | 010 |
| 3 | 0011 |
| 4 | 0001 |
| 5 | 0010 |
| 6 | 0000 |

Table Error: Reference source not found-4 – Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| edge_code | 0 | 1 | bypass | 2 | na | na |

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[1] “CE6.h Region Boundary Chain Coding for Depth-map,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A0070, July 2012.

[2] “3D-HEVC Test Model Description, draft specification,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1005, Jan 2013.