| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0085 (v.2) |
| --- | --- |

| Title: | Remarks on Multi-resolution Frame-Compatible (MFC) extension of AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Comment and Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This contribution remarks on the state of the work on the Multi-resolution Frame-Compatible (MFC) extension of AVC. It asserts that up to this point, there has been rather limited participation in this work, and suggests that care should be taken to ensure that this work is studied carefully and to make sure that the technical design is fully appropriate and the editorial specification is complete, correct, and clear. Some specific aspects are discussed, most of which are editorial. Two technical issues are identified as questionable: 1) a duplicate representation of the low-frequency image signal, and 2) a constraint on the left and top cropping parameters.

# Introduction / Problem Statement

Work is now under way to standardize a Multi-resolution Frame-Compatible (MFC) extension of AVC. The current text (JCT3V-C1007 / WG 11 N 13331) has been issued for a relatively-brief (two month) ballot (closing date 2013-04-14).

Up to this point, there has been rather limited participation in this work. We hope that this amendment will receive a more appropriate level of attention as the work moves forward to completion.

# Technical issues

After cursory reading of the text, we note two technical issues that seem questionable:

- The low-frequency content of the pictures seems to be represented twice, with the base-layer representation being subtracted off and discarded from the final representation. This seems rather questionable for two reasons: a) images tend to be low-pass and thus a large percentage of the bit rate seems to be wasted by doing this, and b) if the enhancement signal did not carry the duplicate low-frequency content, the complexity could be reduced by basically eliminating the "reference processing unit" (RPU) stage and also eliminating the subtraction of the duplicate information from the reconstruction formula. (For example, the discarding of the duplicate signal can be observed in the step expressed as "e. resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )" in the specification.) If there is no need for inter-view referencing between the base and enhancement layer view components, the scheme could be simplified and possibly become somewhat similar to that found in "MVC + Depth". We suggest that disabling the deblocking filter for the enhancement layer decoding process may be beneficial when the low-frequency signal is not encoded in a duplicate fashion.

- The left and top frame cropping parameters are required to be equal to 0 (in H.8.4.1). We suggest that this loss of flexibility seems ill-advised, as we have never imposed similar constraints in other profiles and the cropping seems basically unrelated to the added functionality.

# Editorial issues

We also note the following significant editorial issues. Some other editorial remarks have been communicated informally.

- Some of the changes conflict with changes made for at least one prior approved amendment (MVC + Depth). This is probably unavoidable at this stage, since much of this was drafted prior to completion of other amendments, but we need to remember not to let this happen in the final version.

- We suggest that the frame/field handling aspects be especially studied to ensure clarity and completeness, as the handling of these seems quite different than in previous work in AVC.

- The rpu_type syntax element has no actual meaning and could simply be removed or identified as a reserved syntax element with no usage implied by its naming.

- In several places, "bitstream" should probably be "coded video sequence".

- It seems unclear whether an FPA SEI message is required to be in the bitstream or not. If not, then it may be desirable to add a NOTE to say this.

- The concepts of a "left view" and "right view" and "view boundary" are used in H.8.4.1 without providing any definition or explanation to show what these mean.

- Since the concepts of views and view components in this text are somewhat different that prior usage of these terms, adding more explanation may be desirable.

- The concept of an "MFC bitstream" is used in H.8.4.1 and the concept of an "MFC sytem" is used in H.8.6 without any definition of what these are (see also the comment about "bitstream" above).

- In some places, the text says how to change equations and processes found nearby rather than simply providing the modified definition, which would seem preferable in style.

- We suggest adding a NOTE in H.10.1.3 to help the reader understand why the constraints of the profile have no apparent differences relative to the Stereo High profile (i.e. because the syntax is modified when the profile_idc value indicates the use of the MFC High profile).

- The indirect use of the Stereo High profile to specify the level constraints of the MFC High profile may be confusing. We suggest to instead add specific mentions of the MFC High profile in the section that establishes the level constraints.

- The profile or its profile_idc value should be accounted for in 7.4.2.1.1 in a similar manner as for the Stereo High profile.

- It is editorially undesirable for the name of a variable or syntax element to be a sub-string of the name of a different variable or syntax element. (Ideally, variable names and syntax element names should be uniquely searchable – otherwise it can be difficult to find the particular place in the text that contains something, as this may require searching through many irrelevant occurrences of the same string, and this convention may also generally help avoid confusion.) This problem is evident in a few of the variable names used in the MFC draft text.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).