| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0086 |
| --- | --- |

| Title: | 3D-CE5.h related: Crosscheck of removal of parsing dependency for illumination compensation (JCT3V-D0152) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu (hongbin.liu@lge.com) |  |  |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of JCT3V-D0152 which proposes two methods to remove the parsing dependency (there is parsing dependency in merge mode) in illumination compensation (IC). Method 1 simply encodes IC flag for all inter-mode CUs in non-base view. Method 2 proposes an advanced method to handle two cases: a) only inter-view reference picture is in the reference list. In this case, there is no parsing dependency problem, IC is not changed. b) Temporal reference picture is also in the reference list. In this case, for merge mode, IC flag is also merged, therefore, no IC flag is required for merge mode; for non-merge inter-mode, IC is only allowed for Size_2Nx2N case. Experiment results perfectly match with that provided in JCT3V-D0152. The software matches the document description.

# Introduction

Two simulation sets are tested.

- Method 1: encode IC flag for all inter-mode CUs in non-base view.

- Method 2: a) only inter-view reference picture is in the reference list. In this case, there is no parsing dependency problem, IC is not changed. b) Temporal reference picture is also in the reference list. In this case, for merge mode, IC flag is also merged, therefore, no IC flag is required for merge mode; for non-merge inter-mode, IC is only allowed for Size_2Nx2N case.

# Results

Table 1: performance of method 1 (compared with HTM-6.0, CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.6% | 0.6% | 0.3% | 0.2% | 0.2% |
| Kendo | 0.0% | 1.6% | 2.1% | 0.8% | 0.6% | 0.6% |
| Newspaper_CC | 0.0% | 0.2% | 0.5% | 0.1% | 0.1% | 0.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.5% | 0.0% | 0.1% | 0.1% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Undo_Dancer | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | 0.8% | 1.0% | 0.4% | 0.3% | 0.3% |
| 1920x1088 | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | 0.3% | 0.5% | 0.2% | 0.1% | 0.1% |

Table 2: Performance of method 2 (compared with HTM-6.0, CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Kendo | 0.0% | 0.7% | 0.6% | 0.3% | 0.2% | 0.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Poznan_Hall2 | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | 0.0% |
| Poznan_Street | 0.0% | 0.4% | -0.3% | 0.0% | 0.0% | 0.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | 0.3% | 0.3% | 0.1% | 0.1% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |

# Conclusion

The results perfectly match with JCT3V-D0152 and the software matches with the document description.