| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0088 |
| --- | --- |

| Title: | Crosscheck of on collocated picture and low-delay checking for MV-HEVC (JCT3V-D0065) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu (hongbin.liu@lge.com) Jiwook.Jung (jiwook.jung@lge.com) Shiqi Wang (sqwang@jdl.ac.cn) |  |  |
| Source: | LG Electronics, Peking University |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of JCT3V-D0065 which proposes to always set the signaled collocated picture as inter-view reference picture for non-base view and set the low delay flag to true when collocated picture is inter-view reference picture. Experiment results perfectly match with that provided in JCT3V-D0065. The software matches the document description.

# Introduction

Two simulation sets are tested.

- Always set the signaled collocated picture as inter-view reference picture for non-base view.

- Always set the signaled collocated picture as inter-view reference picture for non-base view, and set the low delay flag to true when collocated picture is inter-view reference picture.

# Results

Table 1: performance of method 1 (compared with MV-HEVC anchor)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |  |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.9% | -1.1% | -0.9% | -0.9% |  |
| Kendo | 0.0% | -0.6% | 0.3% | -0.1% | -0.1% |  |
| Newspaper_CC | 0.0% | 0.7% | 2.1% | 0.7% | 0.7% |  |
| GT_Fly | 0.0% | 0.9% | 1.1% | -0.2% | -0.2% |  |
| Poznan_Hall2 | 0.0% | -2.0% | 0.6% | -0.1% | -0.1% |  |
| Poznan_Street | 0.0% | 0.7% | 1.1% | 0.3% | 0.3% |  |
| Undo_Dancer | 0.0% | -0.5% | 0.4% | 0.0% | 0.0% |  |
| 1024x768 | 0.0% | -0.6% | 0.4% | -0.1% | -0.1% |  |
| 1920x1088 | 0.0% | -0.3% | 0.8% | 0.0% | 0.0% |  |
| average | 0.0% | -0.4% | 0.6% | -0.1% | -0.1% |  |

Table 2: Performance of method 2 (compared with MV-HEVC anchor)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |  |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -5.7% | -4.0% | -2.5% | -2.5% |  |
| Kendo | 0.0% | -3.2% | -2.0% | -1.4% | -1.4% |  |
| Newspaper_CC | 0.0% | -0.3% | 1.8% | 0.4% | 0.4% |  |
| GT_Fly | 0.0% | 0.1% | 0.5% | -0.5% | -0.5% |  |
| Poznan_Hall2 | 0.0% | -3.2% | 0.0% | -0.6% | -0.6% |  |
| Poznan_Street | 0.0% | 0.1% | 0.7% | 0.1% | 0.1% |  |
| Undo_Dancer | 0.0% | -0.6% | 0.4% | -0.1% | -0.1% |  |
| 1024x768 | 0.0% | -3.0% | -1.4% | -1.2% | -1.2% |  |
| 1920x1088 | 0.0% | -0.9% | 0.4% | -0.3% | -0.3% |  |
| average | 0.0% | -1.8% | -0.4% | -0.7% | -0.7% |  |

# Conclusion

The results perfectly match with JCT3V-D0065 and the software matches with the document description. Method 2 improves the coding performance significantly.