| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0089 |
| --- | --- |

| Title: | 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Corresponding Complexity Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Fu Chen, Gwo Giun Lee, Bo-Syun Li, Chunhui Cui, Yan Huo No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) and Hong Kong Applied Science and Technology Research Institute (ASTRI) |  |  |

_____________________________

# Abstract

Due to the fact that the backward view synthesis prediction (B-VSP) [1] has been adopted in JCT3V 3rd meeting, and the depth-oriented neighboring block disparity vector (DoNBDV) [2] was also adopted to improve the accuracy of the disparity vector which is used to estimate the reference blocks in reference texture view. Accordingly, this proposal would like to propose the techniques which would adaptively partition the depth block to derive the disparity vectors. This adaptively partition scheme is based on the information of depth block, the supported block sizes include 4×8, 8×4, and 8×8 block sizes. This proposal provide four experiments on difference depth analysis methods. The results shows that method 1 reports no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average , with 1.3 % increase in decoding time; method 2 reports no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average , with 0.2 % increase in decoding time; method 3 reports no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average , with 0.4 % increase in decoding time; method 4 reports an average bitrate increasing of 0.1% for video PSNR vs. video bitrate but no bitrate increasing for video PSNR vs. total bitrate and for synth PSNR vs. total bitrate in average, with 0.2% increase in decoding time. However, with the help of supported blocks, the B-VSP modules can be aligned with motion compensation module. On the other hand, this proposal also reports the complexity analysis in terms of number of operations, data storage requirement, and data transfer rate as compared to the B-VSP in 3D-HEVC. The data transfer rate could be reduced about 10% ~ 20% as compared to current B-VSP in 3D-HEVC due to the fact that the fetched reference texture data would be reused more efficiently.

# Introduction and Problem Statement

According to the CE1 description [3], the B-VSP would be tested at 8×8 or larger block size or adaptive block sizes and the block size would not be smaller than 4×8 or 8×4 block size in order to align with motion compensation. However, when the block size becomes larger, the depth distribution of this block was varied. That is, the disparity vector which is used to perform backward warping may be inaccurate due to the fact that this disparity vector is derived based on the maximum depth value of virtual depth block. Thus, this proposal would like to resolve the issue by content adaptively partitioning the depth block.

# Proposed Algorithm

## Algorithm description

This proposal presents an algorithm to content adaptive partition the virtual depth block to derive one or two disparity vectors to accmomondate the content information of the virtual depth block. This proposal work at the 8×8 block basis and extracts the gradient information of 8×8 block to determine that an 8×8 block would be further partitioned to two 4×8 blocks or 8×4 blocks. To achieve an efficiency analysis method on the depth block, four methods of depth content analysis are evaluated. Since there are less texture characteristic in the depth data, full-pixels gradient computation may not be necessary. Sub-sampled pixels gradient computation is also capable of partitioning the virtual depth block in a well manner. Method 1 is the full-pixels gradient computation as shwon in Fig. 1 (a); this method computated the gradient between two consecutive pixels in horizontal and vertical directions repectively. On the other hand, method 2 utilizes the outmost pixels which located at the block boundary to compute the gradient information of this block since the variation of one block would be simple. The visualization of methof 2 is shown in Fig. 1 (b). The remaining two methods are the downsampled version of method 1 and method 2 repectively due to the fact that the depth transition should be observed for the downsampled block. Method 3 and method 4 are illustrated in Fig. 1 (c) and Fig. 1 (d). In Fig. 1, the gradient operations would be performed on the pixels with dark blue color.

Fig. 1 Methods of gradient computation for depth content analysis

Upon having the gradient information, GradH and GradV, of the virtual depth block, the number of line changes on horizontal and vertical directions, NoLCH and NoLCV, are counted according to the number of lines whose gradients has at least one gradient value larger than the threshold. The thresholds for horizontal and vertical directions,THH and THV, are determined by the half of the maximum gradient respectively. Finally, this proposal utilized the NoLCH and NoLCV to pratition the virtual depth block in accordance with the flowchart illustrated in Fig. 2, where the TH is predefined parameters.

Fig. 2 Partition type decision flow

## Complexity assessment

Since this proposal also would like to achieve the complexity reduction on B-VSP, the following sections provide the complexity assessment in terms of number of operations, data storage requirement, and data transfer rate to justify method 4 of the proposed B-VSP is less complexity than the current B-VSP and the coding gain can be retained, too.

This paragraph illustrates the analysis work on the proposed and current B-VSP and follows the analysis method in [5]. The data granularity is one prediction unit (PU), and the PU size can be 64×64, 64×32, etc. That is, the number of operations is accumulated within one PU process. The data storage requirement for the input and output data is calculated according to one PU; however, the data storage requirement for the intermediate data is counted for one sub-block process since this buffer can be reused for the next subblock process. The data transfer rate is estimated the amount of input data and output data would be transferred between internal or external storage during one PU process, respectively.

Due to the fact that the proposed algorithm splits the PU into several 8×8 depth block and each sub-block process is independent, and thus the analyzed results of one PU would be multiples of the analyzed results of 8×8 blocks. On the other hand, the current B-VSP is 4×4 block-based process, and hence the analyzed results of one PU would also be multiples of the analyzed results of 4×4 block.

The flowchart of the proposed algorithm are illustrated in Fig. 3, and the difference between the proposed and current B-VSP is the blocks surrounding by a blue box. That is, there is no depth block analysis of current B-VSP. According to the flowchart, the complexity of the proposed and current B-VSP are assessed and the measured metrics include number of operations, data storage requirement, and data transfer rate. The document shows overall summary results only; for the detail information, please refer to the spreadsheet.

### Number of operations

With the flowchart in Fig. 3, the modules of the proposed and current B-VSP are identified and listed in the following entries. Moreover, the mathematical expression would be used to provide high accurate number of operations calculation. The analyzed number of operations are listed in Table 1 and the results show that there is 5.37% computation complexity increasing as compared to current B-VSP. (M and N is the block width and height in horizontal and vertical direction respectively; that is, number of 8×8 blocks in one PU is (M/8)×(N/8))

- Vertical direction gradient calculation: 4 subtractions, 4 absolutes and 7 comparison for one 8×8 block

- Horizontal direction gradient calculation: 4 subtractions, 4 absolutes and 7 comparison for one 8×8 block

- Partition type determination: 2 comparisons for one 8×8 block

- Representative virtual depth generation: (width of subblock partition)×(height of subblock partition)-1 comparisons for one subblock parition

- Reference texture block position derivation: 1 addition of one subblock partition

- Interpolation: (Worst case ) 7 additions/subtractions, 6 constant multiplications are needed for one sub-pixel of luminance compoenet and 3 additions/subtractions and 4 constant multiplications are needed for one sub-pixel of chromiance compoenet. (M×N times operations for Luma. And 2×(M/2)×(N/2) times operations for Chroma. are needed.)

Table 1 Number of operations of proposed (method 4) and current B-VSP.

|  | Number of operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposal (Average) |  | Anchor |  | Ratio (to anchor) |  |
| PU_size | Add/Sub/Abs/Comp | Cons_Mul | Add/Sub/Abs/Comp | Cons_Mul | Add/Sub/Abs/Comp | Cons_Mul |
| 64×64 | 34880 | 24576 | 32768 | 24576 | 106.45% | 100.00% |
| 64×32 | 17440 | 12288 | 16384 | 12288 | 106.45% | 100.00% |
| 32×64 | 17440 | 12288 | 16384 | 12288 | 106.45% | 100.00% |
| 32×32 | 8720 | 6144 | 8192 | 6144 | 106.45% | 100.00% |
| 32×16 | 4360 | 3072 | 4096 | 3072 | 106.45% | 100.00% |
| 16×32 | 4360 | 3072 | 4096 | 3072 | 106.45% | 100.00% |
| 16×16 | 2180 | 1536 | 2048 | 1536 | 106.45% | 100.00% |
| 16×8 | 1090 | 768 | 1024 | 768 | 106.45% | 100.00% |
| 8×16 | 1090 | 768 | 1024 | 768 | 106.45% | 100.00% |
| 8×8 | 545 | 384 | 512 | 384 | 106.45% | 100.00% |
| 8×4 | 256 | 192 | 256 | 192 | 100.00% | 100.00% |
| 4×8 | 256 | 192 | 256 | 192 | 100.00% | 100.00% |
| Average |  |  |  |  | 105.37% | 100.00% |

### Data storage requirment

The data storage requirement are listed in Table 3; for the current B-VSP, the major data should be stored are (I) reference virtual depth block, (II) reference texture block with extended taps, and (III) compensated block. On the other hand, the additional data storage requirement for analyzing virtual depth block of the proposed B-VSP are (IV) analyzed gradient map and buffer of depth maximums in one 4×4 block. Other miscellaneous data are recorded in the spreadsheet. When a PU size is M×N, M is the block width and N is the block height in horizontal and vertical direction respectively; the bits of one pixel of texture or depth is 8 bits. The data storage requirement is illustrated in Table 2.

Since the block size of the proposed is higher than the current B-VSP, the buffer size for storing the reference texture data is increased. Hence required data storage size of the proposed method are higher than the current B-VSP. Table 3 shows that the data storage size is increased up to 50% when PU size is 4×8; however, the exactly increased bits are only 448 bits (56 bytes). This number can be viewed as a neglected storage size increasing.

Table 2 Data storage requirement for one PU (bits)

|  | Proposed B-VSP | Anchor |
| --- | --- | --- |
| Reference virtual depth block | M×N×8 | M×N×8 |
| Reference texture block with extended taps | (3+8+4)×8×8 | (3+4+4)×4×8 |
| Compensated block | M×N×8 | M×N×8 |
| Analyzed gradient map and buffer of depth maximums in one 4x4 block | ((4+4)+4)×8 | N/A |

Table 3 Data storage requirement and data transfer rate of proposed (method 4) and current B-VSP

|  | Data storage requirement (Bits) |  |  |  |
| --- | --- | --- | --- | --- |
| PU_size | Proposal (Average) | Anchor | Ratio (to anchor) | Difference bits (to anchor) |
| 64×64 | 66604 | 65900 | 101.07% | 704 |
| 64×32 | 33836 | 33132 | 102.12% | 704 |
| 32×64 | 33836 | 33132 | 102.12% | 704 |
| 32×32 | 17452 | 16748 | 104.20% | 704 |
| 32×16 | 9260 | 8556 | 108.23% | 704 |
| 16×32 | 9260 | 8556 | 108.23% | 704 |
| 16×16 | 5164 | 4460 | 115.78% | 704 |
| 16×8 | 3116 | 2412 | 129.19% | 704 |
| 8×16 | 3116 | 2412 | 129.19% | 704 |
| 8×8 | 2092 | 1388 | 150.72% | 704 |
| 8×4 | 1100 | 876 | 125.57% | 224 |
| 4×8 | 1324 | 876 | 151.14% | 448 |
| Average |  |  | 118.96% | 643 |

### Data transfer rate

The data transfer rate are listed in Table 5 and the input and output data transfer rate would be discussed, respectively. For the proposed and current B-VSP, the major data should be transferred are identical, including, (I) reference virtual depth block, (II) reference texture block with extended taps, and (III) compensated block. The input data rate includes (I) and (II) and the output data rate is (III). 	When a PU size is M×N, M is the block width and N is the block height in horizontal and vertical direction respectively; the bits of one pixel of texture or depth is 8 bits. The data transfer rate is illustrated in Table 4.

The data transfer rate of the proposed B-VSP is less than the current B-VSP. The From Table 5, the proposed B-VSP can reduced 18% data transfer rate in average and it is significant data rate reduction. On the other hand, the number of data fetching from decoded picture buffer of the proposed B-VSP is fewer than the current B-VSP, too. The reduction ratio of number of data fetching is about 50% - 75%. (50% for 4×8 block and 8×4 block; 75% for 8×8 block)

Table 4 Data transfer rate for one PU (bits/PU)

|  | Proposed B-VSP | Anchor |
| --- | --- | --- |
| Reference virtual depth block | M×N×8 | M×N×8 |
| Reference texture block with extended taps | (3+8+4)×8×(M/8)×(N/8)×8 + 2×(1+4+2)×4×(M/4)×(N/4)×8 | (3+4+4)×4×(M/4)×(N/4)×8 + 2×(1+2+2)×4×(M/2)×(N/2)×8 |
| Compensated block | M×N×8 + 2×(M/2)×(N/2)×8 | M×N×8 + 2×(M/2)×(N/2)×8 |

Table 5 Data transfer rate of proposed (method 4) and current B-VSP

|  | Data transfer rate (Bits/PU) |  |  |
| --- | --- | --- | --- |
| PU_size | Proposal (Average) | Anchor | Ratio (to anchor) |
| 64×64 | 185698 | 213004 | 87.18% |
| 64×32 | 92855 | 114700 | 80.95% |
| 32×64 | 92855 | 114700 | 80.95% |
| 32×32 | 46434 | 57356 | 80.96% |
| 32×16 | 23223 | 28684 | 80.96% |
| 16×32 | 23223 | 28684 | 80.96% |
| 16×16 | 11618 | 14348 | 80.97% |
| 16×8 | 5815 | 7180 | 80.99% |
| 8×16 | 5815 | 7180 | 80.99% |
| 8×8 | 2914 | 3596 | 81.03% |
| 8×4 | 1356 | 1804 | 75.17% |
| 4×8 | 1676 | 1804 | 92.90% |
| Average |  |  | 82.00% |

Fig. 3 Flowchart of the proposed adaptive block parathion B-VSP

# Experimental Results

The anchor software is HTM-6.0 as demanded in CTC [4]. The tests in this proposal composing lots of different parameters combination as listed in Table 6.

Table 6 Simulation configurations of four methods

|  | Gradient Computation Method | TH for partition descition |
| --- | --- | --- |
| Test 01 | Full pixels | 4 |
| Test 02 | Outmost pixels | 4 |
| Test 03 | Full pixels (down-sample by 2) | 2 |
| Test 04 | Outmost pixels (down-sample by 2) | 2 |

Table 7 Experimental results of test 01 (Full pixels gradient computation)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.7% | 100.1% | 100.0% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.6% | 100.3% | 99.9% |
| Newspaper_CC | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 99.8% | 100.0% |
| GT_Fly | 0.0% | 0.5% | 0.1% | 0.1% | 0.1% | 0.1% | 100.6% | 105.3% | 99.9% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.3% | -0.2% | -0.2% | -0.1% | 100.6% | 100.8% | 99.8% |
| Poznan_Street | 0.0% | 0.3% | 0.2% | 0.1% | 0.1% | 0.1% | 100.2% | 100.3% | 101.7% |
| Undo_Dancer | 0.0% | 0.6% | 0.7% | 0.2% | 0.1% | -0.2% | 100.5% | 102.4% | 100.2% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.6% | 100.1% | 100.0% |
| 1920x1088 | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | -0.1% | 100.5% | 102.2% | 100.4% |
| average | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.5% | 101.3% | 100.2% |

Table 8 Experimental results of test 02 (Outmost pixels gradient computation)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 100.2% | 100.2% | 101.1% |
| Kendo | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 100.5% | 99.8% | 99.8% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 99.2% | 100.3% |
| GT_Fly | 0.0% | 0.4% | 0.2% | 0.1% | 0.1% | 0.0% | 100.3% | 102.0% | 100.6% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.1% | -0.1% | -0.1% | -0.2% | 100.1% | 99.9% | 99.8% |
| Poznan_Street | 0.0% | 0.4% | 0.3% | 0.1% | 0.1% | 0.1% | 100.3% | 99.6% | 100.7% |
| Undo_Dancer | 0.0% | 0.7% | 0.8% | 0.2% | 0.2% | -0.1% | 100.1% | 100.8% | 100.3% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 99.8% | 100.4% |
| 1920x1088 | 0.0% | 0.3% | 0.3% | 0.1% | 0.0% | 0.0% | 100.2% | 100.6% | 100.3% |
| average | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 100.3% | 100.2% | 100.4% |

Table 9 Experimental results of test 03 (Full pixels gradient computation downsampled by 2)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.2% | 99.9% | 99.9% |
| Kendo | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 100.1% | 99.9% |
| Newspaper_CC | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 100.3% | 99.6% | 100.1% |
| GT_Fly | 0.0% | 0.4% | 0.5% | 0.1% | 0.1% | 0.1% | 100.2% | 102.4% | 99.9% |
| Poznan_Hall2 | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.1% | 100.0% | 100.1% | 99.6% |
| Poznan_Street | 0.0% | 0.3% | 0.4% | 0.1% | 0.1% | 0.1% | 99.2% | 99.8% | 100.5% |
| Undo_Dancer | 0.0% | 0.9% | 0.9% | 0.2% | 0.2% | 0.0% | 100.1% | 101.0% | 100.4% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.3% | 99.9% | 99.9% |
| 1920x1088 | 0.0% | 0.3% | 0.5% | 0.1% | 0.1% | 0.0% | 99.9% | 100.9% | 100.1% |
| average | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 100.1% | 100.4% | 100.0% |

Table 10 Experimental results of test 04 (Outmost pixels gradient computation downsampled by 2)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 100.1% | 99.9% | 100.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 100.1% | 99.5% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.1% | 99.7% | 99.7% |
| GT_Fly | 0.0% | 0.7% | 0.3% | 0.1% | 0.1% | 0.1% | 100.2% | 101.6% | 100.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 100.1% | 100.2% | 100.1% |
| Poznan_Street | 0.0% | 0.5% | 0.2% | 0.1% | 0.1% | 0.1% | 100.2% | 99.4% | 100.6% |
| Undo_Dancer | 0.0% | 0.7% | 1.0% | 0.2% | 0.2% | 0.0% | 99.9% | 100.5% | 99.7% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 99.9% | 99.8% |
| 1920x1088 | 0.0% | 0.5% | 0.4% | 0.1% | 0.1% | 0.1% | 100.1% | 100.4% | 100.1% |
| average | 0.0% | 0.3% | 0.2% | 0.1% | 0.0% | 0.0% | 100.1% | 100.2% | 100.0% |

# Conclusion

This proposal proposed an adaptive virtual depth block partition for B-VSP to derive the appropriate disparities for each sub-block and the supported block size is 4×8, 8×4, and 8×8 block. That is, the proposed B-VSP can be aligned to MC to share the identical module via time-multiplexing method. According to the coding efficiency, method 4 reports an average bitrate increasing of 0.1% for video PSNR vs. video bitrate but no bitrate increasing for video PSNR vs. total bitrate and synth PSNR vs. total bitrate in average, with 0.2% increase in decoding time; however, the assessed complexity reveals that the number of operations and data storage size are increased slightly, with 4.5% and 19% increasing in average respectively. However, the increased data storage is not significant due to the fact that the exactly increased size is about 224 ~ 704 bits. On the other hand, the data transfer rate per PU is significantly reduced about 18% and the reference texture data access becomes more regular than the current B-VSP. Hence, this proposal recommends that method 4 of this proposal could be adopted and further studied on larger sub-block.

# Acknowledge

Thanks for the cross-check by and ITRI on test 02 and test 04 (JCT3V-D0292) and the help of high performance computing cluster system from NCKU computer and network center.

# Reference

- JCT3V-C0152, “CE1.h: Backward View Synthesis Prediction using Neighbouring Blocks, “Geneva, CH, 17-23 January 2013.

- JCT3V-C0131, “3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval, “Geneva, CH, 17-23 January 2013.

- JCT3V-C1101, “Description of Core Experiment 1 (CE1) on View Synthesis Prediction, “Geneva, CH, 17-23 January 2013.

- JCT3V-C1100, “Common Test Conditions of 3DV Core Experiments, “Geneva, CH, 17-23 January 2013.

- JCT3V-D0090, “AhG 10: Complexity Assessment on Adaptive Luminance Compensation and Plane Segmentation based Intra Prediction in 3D-AVC, “Incheon, KR, 20-26 April 2013.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-3V and contributions to the JCT-3V are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-3V proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. Two examples of such an informal IPR notification statement for a contribution are provided below.)

NCKU and ASTRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).