| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0089 |
| --- | --- |

| Title: | 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Fu Chen Gwo Giun Lee Bo-Syun Li No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) |  |  |

_____________________________

# Abstract

Due to the fact that the backward view synthesis prediction (B-VSP) [1] has been adopted in JCT3V 3rd meeting, and the depth-oriented neighboring block disparity vector (DoNBDV) [2] was also adopted to improve the accuracy of the disparity vector which is used to estimate the reference blocks in reference texture view. Accordingly, this proposal would like to propose the techniques which would adaptively partition the depth block to derive the disparity vectors. This adaptively partition scheme is based on the information of depth block, the supported block sizes include 4x8, 8x4, and 8x8 block sizes. This proposal provide four experiments on difference depth analysis methods. The results shows that method 1 reports no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average , with 1.3 % increase in decoding time; method 2 reports no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average , with 0.2 % increase in decoding time; method 3 reports no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average , with 0.4 % increase in decoding time; method 4 reports an average bitrate increasing of 0.1% for video PSNR vs. video bitrate but no bitrate increasing for video PSNR vs. total bitrate and for synth PSNR vs. total bitrate in average, with 0.2% increase in decoding time. However, with the help of supported blocks, the B-VSP MC modules can be aligned with MC module. On the other hand, this proposal also reports the complexity analysis in terms of number of operations, data storage requirement, and data transfer rate as compared to the B-VSP in 3D-HEVC. The data transfer rate could be reduced about 10% ~ 20% as compared to current B-VSP in 3D-HEVC due to the fact that the fetched reference texture would be reused more efficiently.

# Introduction and Problem Statement

According to the CE1 description [3], the B-VSP would be tested at 8x8 block size or adaptive block sizes and the block size would not be smaller than 4x8 or 8x4 block size. Since the block size becomes larger and the DoNBDV is derived based on the maximum depth value within virtual depth block to represent the depth of entire block, the DoNBDV may be inaccurate when the depth distribution of this block is varied. Thus, this proposal would like to resolve the issue by content adaptively partitioning the depth block.

# Proposed Algorithm

## Algorithm description

This proposal present an algorithm to content adaptive partition the virtual depth block to derive multiple disparity vectors to accmomondate the content information of the virtual depth block. This proposal extract the gradient information to determine that an 8x8 block would be further partitioned to two 4x8 blocks or 8x4 blocks or no partition. This proposal would like to achieve an efficiency analysis method on the depth block, and thus, four methods of depth content analysis are evaluated. Since there are less texture characteristic in the depth data, full-pixels gradient computation may not be necessary. Sub-sampled pixels gradient computation is also capable of partitioning the virtual depth block in a well manner. Method 1 is the full-pixels gradient computation as shwon in Fig. 1 (a); this method denotes that the gradient between two pixels are computated in horizontal and vertical directions repectively. On the other hand, the another method utilizes only the pixel difference at block boundary to represent the gradient information of the block since the variation of one block would be simple. The visualation of methof 2 is shown in Fig. 1 (b). The remaining two methods are the downsampled version of method 1 and method 2 repectively due to the fact that the depth transition can be observed for the downsampled block. Method 3 and method 4 are illustrated in Fig. 1 (c) and Fig. 1 (d). In Fig. 1, the gradient computation would be performed on the pixels with dark blue color.

Fig. 1 Methods of gradient computation for depth content analysis

Upon haveing the gradient information, GradH and GradV, of the virtual depth block, the number of line changes on horizontal and vertical directions, NoLCH and NoLCV, are counted according to the number of lines whose gradients has at least one gradient value larger the threshold. The thresholds for horizontal and vertical directions,THH and THV, are determined by the half of the maximum gradient respectively. Finally, this proposal utilized the NoLCH and NoLCV to pratition the virtual depth block in accordance with the flowchart illustrated in Fig. 2, where the TH is predefined parameters.

Fig. 2 Partition type decision flow

## Complexity assessment

Since this proposal also would like to achieve the complexity reduction on B-VSP, the following sections provide the complexity assessment in terms of number of operations, data storage requirement, and data transfer rate. According to these metrcis, the complexity of 4x4 block-based B-VSP and the proposal are compared. Note that, only method 4 of the proposal are analyzed.

This paragraph illustrates the analysis work on B-VSP and the proposal and the data granularity is one prediction unit (PU). That is, the number of operations is accumulated within one PU process. The data storage requirement for the input and immediate buffer is calculated according to one PU since some buffer can be reused for the next sub-block process; conversely, the data storage requirement for the output buffer is one compensated PU. The data transfer rate is estimated the amount of input data and output data would be transferred within one PU process, respectively. Please also note that this assessment avoided to discuss the data reuse issue to provide a fair assessment.

Due to the fact that the proposed algorithm splits the PU into several 8x8 depth block and each block process is independent and thus, the analyzed results of one PU would be multiples of the analyzed results of 8x8 block. On the other hand, the current B-VSP is 4x4 block-based process, and hence the analyzed results of one PU would be multiples of the analyzed results of 4x4 block.

The flowchart of the proposed algorithm are illustrated in Fig. 3, and the difference between the prosed and current B-VSP is the blue block. There is no depth block analysis during B-VSP. According to the flowchart, the complexity of the proposed and current B-VSP are assessed and the measured metrics include number of operations, data storage requirement, and data transfer rate. The document shows overall summary results only; for the detail information, please refer to the spreadsheet.

### Number of operations

With the flowchart in Fig. 3, the modules of the proposed and current B-VSP are identified and listed in the following entries. Moreover, the mathematical expression would be used to provide high accurate number of operations calculation. The analyzed number of operations are listed in Table 1 and the results show that there is 6.58% computation complexity increasing only.

- Vertical direction gradient calculation: 4 subtractions, 4 absolutes and 7 comparison for one subblock

- Horizontal direction gradient calculation: 4 subtractions, 4 absolutes and 7 comparison for one subblock

- Partition type determination: 2 comparisons for one subblock

- Representative virtual depth generation: (subblock_width)*(subblock_height)-1 comparisons for one subblock

- Reference texture block position derivation: 1 addition of one sub-block

- Interpolation: 7 additions/subtractions, 6 constant multiplications are needed for one sub-pixel (2/4 pixel position)

Table 1 Number of operations of proposal (method 4) and anchor

|  | Number of operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposal (Average) |  | Anchor |  | Ratio |  |
| PU_size | Add/Sub/Abs/Comp | Cons_Mul | Add/Sub/Abs/Comp | Cons_Mul | Add/Sub/Abs/Comp | Cons_Mul |
| 64x64 | 34922.67 | 24576 | 32768 | 24576 | 106.58% | 100.00% |
| 64x32 | 17461.33 | 12288 | 16384 | 12288 | 106.58% | 100.00% |
| 32x64 | 17461.33 | 12288 | 16384 | 12288 | 106.58% | 100.00% |
| 32x32 | 8730.667 | 6144 | 8192 | 6144 | 106.58% | 100.00% |
| 32x16 | 4365.333 | 3072 | 4096 | 3072 | 106.58% | 100.00% |
| 16x32 | 4365.333 | 3072 | 4096 | 3072 | 106.58% | 100.00% |
| 16x16 | 2182.667 | 1536 | 2048 | 1536 | 106.58% | 100.00% |
| 16x8 | 1091.333 | 768 | 1024 | 768 | 106.58% | 100.00% |
| 8x16 | 1091.333 | 768 | 1024 | 768 | 106.58% | 100.00% |
| 8x8 | 545.6667 | 384 | 512 | 384 | 106.58% | 100.00% |
| 8x4 | 256 | 192 | 256 | 192 | 100.00% | 100.00% |
| 4x8 | 256 | 192 | 256 | 192 | 100.00% | 100.00% |

### Data storage requirment

The data storage requirement are listed in Table 2; for the current B-VSP, the major data should be stored are (I) disparity vector of each PU, (II) reference virtual depth block, (III) reference texture block with extend taps, and (IV)compensated block. On the other hand, there are some additional data storage requirement for analyzing virtual depth block of the proposed B-VSP are (V) Analyzed gradient map and buffer of depth maximums in one 4x4 block. Since the block size of the proposed is higher than the current B-VSP, the buffer size for storing the reference texture data is increased. Hence required data storage size of the proposed method are higher than the current B-VSP. Table 2 shows that the data storage size is increased up to 50% when PU size is 4x8; however, the exactly additional bits are only 480 bits. This number can be viewed as a neglected storage size increasing.

Table 2 Data storage requirement and data transfer rate of proposal (method 4) and anchor

|  | Data storage (Bits) |  |  |  |
| --- | --- | --- | --- | --- |
| PU_size | Proposal (Average) | Anchor | Ratio | Difference bits |
| 64x64 | 66668 | 65932 | 101.12% | 736 |
| 64x32 | 33900 | 33164 | 102.22% | 736 |
| 32x64 | 33900 | 33164 | 102.22% | 736 |
| 32x32 | 17516 | 16780 | 104.39% | 736 |
| 32x16 | 9324 | 8588 | 108.57% | 736 |
| 16x32 | 9324 | 8588 | 108.57% | 736 |
| 16x16 | 5228 | 4492 | 116.38% | 736 |
| 16x8 | 3180 | 2444 | 130.11% | 736 |
| 8x16 | 3180 | 2444 | 130.11% | 736 |
| 8x8 | 2156 | 1420 | 151.83% | 736 |
| 8x4 | 1132 | 908 | 124.67% | 224 |
| 4x8 | 1388 | 908 | 152.86% | 480 |
| Average |  |  | 119.42% |  |

### Data transfer rate

The data transfer rate are listed in Table 3 and the input and output data transfer rate would be discussed, respectively. For the proposed and current B-VSP, the major data should be transferred are identical, including (I) disparity vector of PU, (II) reference virtual depth block, (III) reference texture block with extend taps, and (IV)compensated block. The input data rate includes (I), (II), and (III) and the output data rate is (IV). The difference between the proposed and current B-VSP is that the amount of reference texture data of the proposed B-VSP is smaller and the number of times to fetching data in decoded picture buffer of the proposed B-VSP is less. From Table 3, the proposed B-VSP can reduced about 20% data transfer rate and it is significant data rate reduction.

Table 3 Data transfer rate of proposal (method 4) and anchor

|  | Data transfer rate (Bits/PU) |  |  |
| --- | --- | --- | --- |
| PU_size | Proposal (Average) | Anchor | Ratio |
| 64x64 | 142006.7 | 163852 | 86.67% |
| 64x32 | 71009.33 | 90124 | 78.79% |
| 32x64 | 71009.33 | 90124 | 78.79% |
| 32x32 | 35510.67 | 45068 | 78.79% |
| 32x16 | 17761.33 | 22540 | 78.80% |
| 16x32 | 17761.33 | 22540 | 78.80% |
| 16x16 | 8886.667 | 11276 | 78.81% |
| 16x8 | 4449.333 | 5644 | 78.83% |
| 8x16 | 4449.333 | 5644 | 78.83% |
| 8x8 | 2230.667 | 2828 | 78.88% |
| 8x4 | 1036 | 1420 | 72.96% |
| 4x8 | 1292 | 1420 | 90.99% |
| Average |  |  | 79.99% |

Fig. 3 Flowchart of the proposed adaptive block parathion B-VSP

# Experimental Results

The anchor software is HTM-6.0 as demanded in CTC [4]. The tests in this proposal composing lots of different parameters combination as listed in Table 4.

Table 4 Simulation configurations of four methods

|  | Gradient Computation Methd | TH |
| --- | --- | --- |
| Test 01 | Full pixels | 4 |
| Test 02 | Outmost pixels | 4 |
| Test 03 | Full pixels (down-sample by 2) | 2 |
| Test 04 | Outmost pixels (down-sample by 2) | 2 |

Table 5 Experimental results of test 01 (Full pixels gradient computation)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.7% | 100.1% | 100.0% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.6% | 100.3% | 99.9% |
| Newspaper_CC | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 99.8% | 100.0% |
| GT_Fly | 0.0% | 0.5% | 0.1% | 0.1% | 0.1% | 0.1% | 100.6% | 105.3% | 99.9% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.3% | -0.2% | -0.2% | -0.1% | 100.6% | 100.8% | 99.8% |
| Poznan_Street | 0.0% | 0.3% | 0.2% | 0.1% | 0.1% | 0.1% | 100.2% | 100.3% | 101.7% |
| Undo_Dancer | 0.0% | 0.6% | 0.7% | 0.2% | 0.1% | -0.2% | 100.5% | 102.4% | 100.2% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.6% | 100.1% | 100.0% |
| 1920x1088 | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | -0.1% | 100.5% | 102.2% | 100.4% |
| average | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.5% | 101.3% | 100.2% |

Table 6 Experimental results of test 02 (Outmost pixels gradient computation)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 100.2% | 100.2% | 101.1% |
| Kendo | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 100.5% | 99.8% | 99.8% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 99.2% | 100.3% |
| GT_Fly | 0.0% | 0.4% | 0.2% | 0.1% | 0.1% | 0.0% | 100.3% | 102.0% | 100.6% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.1% | -0.1% | -0.1% | -0.2% | 100.1% | 99.9% | 99.8% |
| Poznan_Street | 0.0% | 0.4% | 0.3% | 0.1% | 0.1% | 0.1% | 100.3% | 99.6% | 100.7% |
| Undo_Dancer | 0.0% | 0.7% | 0.8% | 0.2% | 0.2% | -0.1% | 100.1% | 100.8% | 100.3% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 99.8% | 100.4% |
| 1920x1088 | 0.0% | 0.3% | 0.3% | 0.1% | 0.0% | 0.0% | 100.2% | 100.6% | 100.3% |
| average | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 100.3% | 100.2% | 100.4% |

Table 7 Experimental results of test 03 (Full pixels gradient computation downsampled by 2)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.2% | 99.9% | 99.9% |
| Kendo | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 100.1% | 99.9% |
| Newspaper_CC | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 100.3% | 99.6% | 100.1% |
| GT_Fly | 0.0% | 0.4% | 0.5% | 0.1% | 0.1% | 0.1% | 100.2% | 102.4% | 99.9% |
| Poznan_Hall2 | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.1% | 100.0% | 100.1% | 99.6% |
| Poznan_Street | 0.0% | 0.3% | 0.4% | 0.1% | 0.1% | 0.1% | 99.2% | 99.8% | 100.5% |
| Undo_Dancer | 0.0% | 0.9% | 0.9% | 0.2% | 0.2% | 0.0% | 100.1% | 101.0% | 100.4% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.3% | 99.9% | 99.9% |
| 1920x1088 | 0.0% | 0.3% | 0.5% | 0.1% | 0.1% | 0.0% | 99.9% | 100.9% | 100.1% |
| average | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 100.1% | 100.4% | 100.0% |

Table 8 Experimental results of test 04 (Outmost pixels gradient computation downsampled by 2)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 100.1% | 99.9% | 100.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 100.1% | 99.5% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.1% | 99.7% | 99.7% |
| GT_Fly | 0.0% | 0.7% | 0.3% | 0.1% | 0.1% | 0.1% | 100.2% | 101.6% | 100.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 100.1% | 100.2% | 100.1% |
| Poznan_Street | 0.0% | 0.5% | 0.2% | 0.1% | 0.1% | 0.1% | 100.2% | 99.4% | 100.6% |
| Undo_Dancer | 0.0% | 0.7% | 1.0% | 0.2% | 0.2% | 0.0% | 99.9% | 100.5% | 99.7% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 99.9% | 99.8% |
| 1920x1088 | 0.0% | 0.5% | 0.4% | 0.1% | 0.1% | 0.1% | 100.1% | 100.4% | 100.1% |
| average | 0.0% | 0.3% | 0.2% | 0.1% | 0.0% | 0.0% | 100.1% | 100.2% | 100.0% |

# Conclusion

This proposal proposed an adaptive virtual depth block partition for B-VSP to derive the appropriate disparities for each block partition and the supported block size is 4x8, 8x4, and 8x8 block. That is, the B-VSP module can be aligned to MC module to share the identical module via time-multiplexing method. According to the coding efficiency, method 4 reports an average bitrate increasing of 0.1% for video PSNR vs. video bitrate but no bitrate increasing for video PSNR vs. total bitrate and synth PSNR vs. total bitrate in average, with 0.2% increase in decoding time when anchor software is HTM-6.0; however, the assessed complexity reveals that the number of operations and data storage size are increased slightly, with 6% and 20% increasing respectively. However, the increased data storage is not significant due to the fact that the exactly size is about 480 ~ 736 bits only. On the other hand, the data transfer rate per PU is significantly reduced about 20% since the reference texture data access is more regular. Hence, this proposal hopes that method 4 of this proposal could be adopted or further studied on larger block partition method.

# Reference

- JCT3V-C0152, “CE1.h: Backward View Synthesis Prediction using Neighbouring Blocks, “Geneva, CH, 17-23 January 2013.

- JCT3V-C0131, “3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval, “Geneva, CH, 17-23 January 2013.

- JCT3V-C1101, “Description of Core Experiment 1 (CE1) on View Synthesis Prediction, “Geneva, CH, 17-23 January 2013.

- JCT3V-C1100, “Common Test Conditions of 3DV Core Experiments, “Geneva, CH, 17-23 January 2013.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-3V and contributions to the JCT-3V are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-3V proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. Two examples of such an informal IPR notification statement for a contribution are provided below.)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).