| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0090 |
| --- | --- |

| Title: | AhG 10: Complexity Assessment on Adaptive Luminance Compensation and Plane Segmentation based Intra Prediction in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chun-Fu Chen Gwo Giun Lee No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University |  |  |

_____________________________

# Abstract

This proposal provide two case studies on the complexity assessment, including the adaptive luminance compensation (ALC) of texture coding tool and plane segmentation based intra prediction (PSIP) of depth coding tool in 3D-AVC. The measured metrics contains number of operations, data storage requirement, data transfer rate, coding performance, and encoding/decoding time. Moreover, these metrics would be independent from the platform or specific architecture. With the help of the assessed complexity, they can be the indices to make the trade-off between the algorithmic performance and complexity and then the evaluation of coding tool could be more objective. Hence, this proposal recommends that this analysis method would be taken into consideration for the further coding tool development.

# Introduction

The complexity issue has been studied for a long period; however, these studies may analyze the complexity based on the specific platforms. Thus, the assessed complexity would be the platform-dependent and it would be varied on different platform since the analyzed results cannot be transparent to another platform. Thus, this proposal provide an analysis manner to extract the intrinsic algorithmic complexity and the analyzed complexity would be platform-independent and transparent to each platform directly. Two case studies are shown in this proposal, including the adaptive luminance compensation (ALC) of texture coding tool and plane segmentation based intra prediction (PSIP) of depth coding tool in 3D-AVC.

# Method of Complexity Assessment

To assess the complexity, the flowchart for one iteration should be constructed first to determine data causality and one important issue should be concerned during the flowchart construction, the major processing or key modules should be identified in this stage in order to achieve the further analysis work. However, the major processing identification would be affected by the analyzer; thus, this proposal also provide a general concept to construct the flowchart. Subsequently, the complexity metrics, such as data storage requirement, and data transfer rate, can be assessed in accordance with the constructed flowchart and the identified major processes or key modules.

In addition, the complexity should be assessed at the level of one data granularity and the data granularity means that amount of data should be performed or produced within one iteration. Then, by analyzing at different data granularities, the complexity of the algorithm can be studied in a well manner.

- Number of operations

To estimate the number of operations of an algorithm, the starting point would be the mathematical expression due to the fact that the mathematical expression is more intrinsic to the algorithm and the mathematical expression would not be affected by platform. Some basic operation types are considered only, such as addition, multiplication, and division, due to the fact that some equivalent operations would be treated as identical complexity. E.g. the subtraction can be easily implementation by the addition with 2’s complement representation plus a NOT logic and an incermentor and the shift operations can be implemented with the wire reconnection.

- Data storage requirement

The data storage requirement was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and generated output data should be stored to complete one data granularity and some intermediate data would be stored according to the algorithm. However, the data cannot be consecutive fetched data would not be stored entirely. These data would be stored for one subblock process. E.g. the reference texture data of ALC, the position of reference texture was calculated by current position and the disparity vector; thus, the position of the data is unknown before processing the current MB. Since the assessed results would like to be transparent to different platforms, the data storage configuration shall not be considered at the initial stage.

- Data storage requirement

The data transfer rate was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and produced output data should be transferred within the process of one data granularity. The intermediate data would not be took into consideration. In addition, the input and output data transfer rate shall be discussed individually since the characteristic of sending and fetching are different.

In this proposal, the flowchart is constructed according to the specification and the test model [1][2] and the major process is defined according to that is there any data to be processed or not. On the other hand, the data granularity of ALC and PSIP in this proposal is one marcoblock (MB) and all supported mb_types are analyzed. While the ALC is OFF, the disparity compensation prediction (DCP) or motion compensation prediction (MCP) would be performed and hence, the complexity of these two tools would be assessed and compared. On the other hand, the DC intra-prediction is used when PSIP is OFF.

## ALC vs. DCP/MCP

This paragraph illustrates the analysis work on ALC and DCP/MCP and the data granularity is one MB. According to the flowchart in Fig. 1 constructed by specification and test model [1][2], the major processes of ALC are (I) interpolation, (II) luminance-compensated weight generation, and (III) weight motion compensation; the major data storage of ALC are (I) reference template and block, (II) decoded template, and (III) compensated block and some of these data shall be fetched from decoded picture buffer or on-chip line buffers. Accordingly, this proposal assesses the complexity of these factors which can represent the ALC. In addition, the flowchart of DCP/MCP in Fig. 2 shows that all processes in DCP/MCP are the subset of ALC. That is, the complexity of DCP/MCP can be extracted from the analysis results of ALC.

Fig. 1 Flowchart of ALC

Fig. 2 Flowchart of DCP

### Number of operations

According to specification and test model [1][2], the processes of ALC are illustrated as Eq.  to Eq. . Equation  is utilized for the interpolation process, including 6-tap and 2-tap FIR filters. To generate the luminance-compensated weight, the left decoded (LD), upper decoded (UD), left reference (LR), upper reference (UR) template are needed and the formulas of weight are listed in Eq. , Eq.  and Eq. . Once having the weight, the weighted motion compensation is performed as illustrated in Eq.  to obtain the compensated block. In addition, the processes of DCP/MCP are the subset of ALC; thus, the number of operations of DCP/MCP is counted according to interpolation processing and hence it can be extracted from the ALC directly. Table 1 shows a summary of number of operations of ALC and DCP/MCP. The spreadsheet recorded the analyzed results in detail.

- Interpolation: the worst case, two filter0 and one filter1 are needed.

- Luminance-compensated weight generation:

- Weight motion compensation

Table 1 Number of operations of ALC and DCP/MCP

|  |  | Number of operations |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ALC |  |  | DCP/MCP |  |
| mb_type |  | Mul. | Constant Mul | Add./ Sub./ Abs./ Comp. | Constant Mul. | Add./ Sub./ Abs./ Comp. |
| P_L0_16x16 |  | 257 | 2048 | 5895 | 2048 | 3585 |
| P_L0_L0_16x8 |  | 258 | 2048 | 6926 | 2048 | 3586 |
| P_L0_L0_8x16 |  | 258 | 2048 | 6926 | 2048 | 3586 |
| P_Skip |  | 257 | 2048 | 5895 | 2048 | 3585 |
| P_8x8, P_8x8ref0 | P_L0_8x8 | 260 | 2048 | 7964 | 2048 | 3588 |
|  | P_L0_8x4 | 264 | 2048 | 7992 | 2048 | 3592 |
|  | P_L0_4x8 | 264 | 2048 | 7992 | 2048 | 3592 |
|  | P_L0_4x4 | 272 | 2048 | 12144 | 2048 | 3600 |

### Data storage requirement

The data storage requirement are listed in Table 3; for ALC, the major data should be stored are (I) displacement vectors, (II) reference block with extend taps, (III) reference and decoded template, and (IV) compensated block. On the other hand, the major data should be stored of DCP/MCP are (I) disparity vector of each block, (II) reference block with extend taps, and (III) compensated block. Let M and N stand for the width and height of the mb_type; the bits of one pixel is considered as 8 bits; one component of disparity vector is 16 bits. The needed storage size can be calculated as illustrated in Table 2. The difference of data should be stored is that the reference and decoded template are required to estimated weight coefficient and this table also shows that the ratio of data storage requirement.

Table 2 Data storage requirement for one MB (bits)

|  | ALC | DCP/MCP |
| --- | --- | --- |
| Displacement vector | (16/M)x(16/N)x32 | (16/M)x(16/N)x32 |
| Reference block with extend taps | (M+5)x(N+5)x8 | (M+5)x(N+5)x8 |
| Reference and decoded template | ((4x16)+(16x4))x2x8 | N/A |
| Compensated block | 16x16x8 | 16x16x8 |

- Note:
Reference and decoded template: 
For P_L0_16x16, P_L0_L0_16x8, P_L0_L0_8x16, P_Skip, the size of reference and decoded template is ((4x16)+(16x4))x2x8 bits. 
When mb_type is P_L0_8x8 or P_L0_8x8ref0, there are several combination of the subblocks and hence the worst case are considered. The size of reference and decoded template of the worst case, the P_L0_8x8 or P_L0_4x4, is ((4x16)+(16x4))x2x8 bits.
Note that: only the data outside the current MB would be calculated and the worst case of 8x8 block type is considered. Fig. 3 shows the required template of different mb_types, the data should be stored is the block colored in green since the data colored in orange can be obtained from the compensated block.

Fig. 3 The data storage requirement for the reference/decoded template.

Table 3 Data storage requirement of ALC and DCP/MCP

|  |  | Data storage requirement (bits) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ALC |  |  |  |  | DCP/MCP |  |  |  | Ratio (Total) |
| mb_type |  | (I) | (II) | (III) | (IV) | Total | (I) | (II) | (III) | Total |  |
| P_L0_16x16 |  | 32 | 3528 | 2048 | 2048 | 7656 | 32 | 3528 | 2048 | 5608 | 136.52% |
| P_L0_L0_16x8 |  | 64 | 2184 | 2048 | 2048 | 6344 | 32 | 2184 | 2048 | 4296 | 147.67% |
| P_L0_L0_8x16 |  | 64 | 2184 | 2048 | 2048 | 6344 | 32 | 2184 | 2048 | 4296 | 147.67% |
| P_Skip |  | 32 | 3528 | 2048 | 2048 | 7656 | 32 | 3528 | 2048 | 5608 | 136.52% |
| P_8x8, P_8x8ref0 | P_L0_8x8 | 128 | 1352 | 2048 | 2048 | 5576 | 32 | 1352 | 2048 | 3528 | 158.05% |
|  | P_L0_8x4 | 256 | 936 | 2048 | 2048 | 5288 | 32 | 936 | 2048 | 3240 | 163.21% |
|  | P_L0_4x8 | 256 | 936 | 2048 | 2048 | 5288 | 32 | 936 | 2048 | 3240 | 163.21% |
|  | P_L0_4x4 | 512 | 648 | 2048 | 2048 | 5256 | 32 | 648 | 2048 | 3208 | 163.84% |

### Data transfer rate

The data transfer rate are listed in Table 5 and the input and output data transfer rate would be discussed, respectively. For ALC, the major data should be transferred are (I) disparity vector of each block, (II) reference block with extend taps, (III) reference and decoded template, and (IV) compensated block. On the other hand, the major data should be transferred of DCP/MCP are (I) disparity vector of each block, (II) reference block with extend taps, and (III) compensated block.

For ALC, the input data transfer rate includes (I) disparity vector of each block, (II) reference block with extend taps, and (III) reference and decoded template and the output data transfer rate contained (IV) compensated block; on the other hand, for DCP/MCP, the input data transfer rate includes (I) disparity vector of each block and (II) reference block and the output data transfer rate is (III) compensated block. Let M and N stand for the width and height of the mb_type; the bits of one pixel is considered as 8 bits; one component of disparity vector is 16 bits. The data transfer rate can be calculated as illustrated in Table 4. The difference of data should be transferred is that the reference and decoded template are required to estimate weight coefficient and this table also shows that the ratio of data transfer rate.

Table 4 Data transfer rate for one MB (bits/MB)

|  | ALC | DCP/MCP |
| --- | --- | --- |
| Displacement vector | (16/M)x(16/N)x32 | (16/M)x(16/N)x32 |
| Reference block with extend taps | (M+5)x(N+5) x(16/M)x(16/N)x8 | (M+5)x(N+5) x(16/M)x(16/N)x8 |
| Reference and decoded template | ((4x16)+(16x4))x2x8 | N/A |
| Compensated block | 16x16x8 | 16x16x8 |

- Note: 
Reference and decoded template: 
For P_L0_16x16, P_L0_L0_16x8, P_L0_L0_8x16, P_Skip, the size of reference and decoded template is ((4x16)+(16x4))x2x8 bits/MB
When mb_type is P_L0_8x8 or P_L0_8x8ref0, there are several combination of the subblocks and hence the worst case are considered. The data transfer rate of the worst case, the P_L0_8x8 or P_L0_4x4, is ((4x16)+(16x4))x2x8 bits/MB.
Note that: only the data outside the current MB would be calculated and the worst case of 8x8 block type is considered. Fig. 3 shows the required template of different mb_types, the data should be stored is the block colored in green since the data colored in orange can be obtained from the compensated block.

Table 5 Data transfer rate of ALC and DCP/MCP

|  |  | Data transfer rate (bits/MB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ALC |  |  |  |  | DCP/MCP |  |  |  | Ratio (Total) |
| mb_type |  | (I) | (II) | (III) | (IV) | Total | (I) | (II) | (III) | Total |  |
| P_L0_16x16 |  | 32 | 3528 | 2048 | 2048 | 7624 | 32 | 3528 | 2048 | 5608 | 135.95% |
| P_L0_L0_16x8 |  | 64 | 4368 | 2048 | 2048 | 8464 | 64 | 4368 | 2048 | 6480 | 130.62% |
| P_L0_L0_8x16 |  | 64 | 4368 | 2048 | 2048 | 8464 | 64 | 4368 | 2048 | 6480 | 130.62% |
| P_Skip |  | 32 | 3528 | 2048 | 2048 | 7624 | 32 | 3528 | 2048 | 5608 | 135.95% |
| P_8x8, P_8x8ref0 | P_L0_8x8 | 128 | 5408 | 2048 | 2048 | 9504 | 128 | 5408 | 2048 | 7584 | 125.32% |
|  | P_L0_8x4 | 256 | 7488 | 2048 | 2048 | 11584 | 256 | 7488 | 2048 | 9792 | 118.30% |
|  | P_L0_4x8 | 256 | 7488 | 2048 | 2048 | 11584 | 256 | 7488 | 2048 | 9792 | 118.30% |
|  | P_L0_4x4 | 512 | 10368 | 2048 | 2048 | 14464 | 512 | 10368 | 2048 | 12928 | 111.88% |

### Coding Performance, Encoding/Decoding Time

The anchor software is 3DV-ATMv7.0 and the tested is 3DV-ATMv7.0 with ALC OFF. From Table 6, the coding gain loss a lot without ALC but encoding/decoding time are reduced about 1.86% and 0.68% respectively.

Table 6 Coding performance, encoding/decoding time comparison

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 2.05 | -0.07 | 0.00 | 0.00 | 1.94 | -0.06 | 1.10 | -0.04 | 98.11 | 99.20 | 99.60 |
| S02 | 0.36 | -0.01 | 0.00 | 0.00 | 0.40 | -0.01 | -0.30 | 0.01 | 98.47 | 98.14 | 97.91 |
| S03 | 0.33 | -0.01 | 0.00 | 0.00 | 0.32 | -0.01 | 0.10 | 0.00 | 98.19 | 99.43 | 100.54 |
| S04 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | -0.02 | 0.00 | 98.11 | 99.63 | 100.24 |
| S05 | 3.32 | -0.16 | 0.00 | 0.00 | 2.97 | -0.13 | 1.97 | -0.08 | 97.99 | 99.91 | 101.84 |
| S06 | 3.54 | -0.17 | 0.00 | 0.00 | 3.36 | -0.16 | 2.04 | -0.09 | 98.20 | 100.07 | 102.53 |
| S08 | 2.80 | -0.12 | 0.00 | 0.00 | 2.72 | -0.11 | 1.40 | -0.05 | 97.93 | 98.85 | 100.13 |
| Average | 1.78 | -0.08 | 0.00 | 0.00 | 1.68 | -0.07 | 0.90 | -0.04 | 98.14 | 99.32 | 100.40 |

## PSIP vs. DC intra-prediction

This paragraph illustrates the analysis work on PSIP and DC intra-prediction and the data granularity is one MB. According to the flowchart in Fig. 4 constructed by specification and test model [1][2], the major processes of PSIP are (I) two representation depth value generation, (II) prediction code of neighboring line, and (III) prediction code generation of current block and prediction map to depth conversion; the major data storage of PSIP are (I) upper and left lines, (II) prediction code difference, and (III) compensated block and some of these data shall be fetched from decoded picture buffer or on-chip line buffers. Accordingly, this proposal assesses the complexity of these factors which can represent the PSIP. Conversely, the flowchart of DC intra-prediction in Fig. 5 shows that the major processing is an average filter only.

Fig. 4 Flowchart of PSIP

Fig. 5 Flowchart of DC intra-predictions

### Number of operations

To segment the depth block, some decision thresholds should be estimated by the upper and left lines according to Eq.  to Eq. . (Note: ref[i], 0<i<2N, is composed of the upper and left lines and N is the size of subblock.) Upon having the decision thresholds (Th2), the prediction code of neighboring line (horizontal or vertical) can be retrieved by the lookup table illustrated in the specification [1]. Subsequently, the prediction map of current block can be calculated by Eq.  and followed by a lookup table. Finally, the compensated depth block is obtained according to the calculated predicted map and Eq. . For DC intra-prediction, there is just an average filter which illustrated in Eq. . Note that the log2(2N) in Eq.  would always be integer due to the fact that N is the number of power of 2.

The overall summary of number of operations of PSIP and DC intra-prediction is listed in Table 7. More operations are needed for PSIP than DC intra-prediction can be clearly observed.

- Two Representative Depth Value Generation:

- Prediction code of neighbor line: retrieval one data from lookup table.

- Prediction map generation of current block

Retrieval Prediction Map from lookup table.

- Prediction Map to Depth Value Conversion

- Average filter

Table 7 Number of operations of PSIP and DC intra-prediction

|  | Number of operations |  |  |
| --- | --- | --- | --- |
|  | PSIP |  | DC intra-prediction |
| mb_type | Div | Add./ Comp. | Add. |
| Intra_16x16 | 2 | 413 | 32 |
| Intra_8x8 | 8 | 564 | 64 |
| Intra_4x4 | 32 | 848 | 96 |

### Data storage requirement

The data storage requirement are listed in Table 9; for PSIP, the major data should be stored are (I) upper and left lines, (II) prediction code difference, (III) look up table stored the prediction map to prediction code bi-conversion, and (IV) compensated block. On the other hand, the major data should be stored of DC intra-prediction are (I) upper and left lines, and (II) compensated block. Let N stand for both width and height of the mb_type; the bits of one pixel is considered as 8 bits. The needed storage size can be calculated as illustrated in Table 8. The difference of data should be stored is that the prediction code difference and look up table are required to constructed the original prediction block and this table also shows that the ratio of data storage requirement.

Table 8 Data storage requirement for one MB (bits)

|  | PSIP | DC intra-prediction |
| --- | --- | --- |
| Upper and left lines | (16+16)x8 | (16+16)x8 |
| Prediction code difference | Nx((16/N)x(16/N))xlog2(2N) | N/A |
| Table for prediction map | 2NxM | N/A |
| Compensated block | 16x16x8 | 16x16x8 |

Table 9 Data storage requirement of PSIP and DC intra-prediction

|  | Data storage requirement (bits) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSIP |  |  |  |  | DC intra-prediction |  |  | Ratio (Total) |
| mb_type | (I) | (II) | (III) | (IV) | Total | (I) | (II) | Total |  |
| Intra_16x16 | 256 | 80 | 512 | 2048 | 2896 | 256 | 2048 | 2304 | 125.69% |
| Intra_8x8 | 256 | 128 | 128 | 2048 | 2560 | 256 | 2048 | 2304 | 111.11% |
| Intra_4x4 | 256 | 192 | 32 | 2048 | 2528 | 256 | 2048 | 2304 | 109.72% |

### Data transfer rate

The data transfer rate are listed in Table 11 and the input and output data transfer rate would be discussed, respectively. For PSIP, the major data should be transferred are (I) upper and left lines, (II) prediction code difference, and (III) compensated block. The input data rate includes (I), (II), and (III) and the output data rate is (IV). On the other hand, the major data should be transferred of DC intra-prediction are (I) upper and left lines and (II) compensated block.

For PSIP, the input data transfer rate is accumulated by (I) upper and left lines and (II) prediction code difference and the output data transfer rate is counted (III) compensated block; on the other hand, for DC intra-prediction, the input data transfer rate is accumulated by (I) upper and left lines and the output transfer data rate is counted (III) compensated block. Let N stand for both width and height of the mb_type; the bits of one pixel is considered as 8 bits. The data transfer rate can be calculated as illustrated in Table 10. The difference of data should be transferred is that the prediction code difference are required to reconstruct the prediction code for each line and this table also shows that the ratio of data transfer rate.

Table 10 Data transfer rate for one MB (bits/MB)

|  | PSIP | DC intra-prediction |
| --- | --- | --- |
| Upper and left lines | (16+16)x8 | (16+16)x8 |
| Prediction code difference | Nx((16/N)x(16/N))xlog2(2N) | N/A |
| Compensated block | 16x16x8 | 16x16x8 |

Table 11 Data transfer rate of PSIP and DC intra-prediction

|  | Data transfer rate (bits/MB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSIP |  |  |  | DC intra-prediction |  |  | Ratio (Total) |
| mb_type | (I) | (II) | (III) | Total | (I) | (II) | Total |  |
| Intra_16x16 | 256 | 80 | 2048 | 2384 | 256 | 2048 | 2304 | 103.47% |
| Intra_8x8 | 256 | 128 | 2048 | 2432 | 256 | 2048 | 2304 | 105.56% |
| Intra_4x4 | 256 | 192 | 2048 | 2496 | 256 | 2048 | 2304 | 108.33% |

### Coding Performance, Encoding/Decoding Time

The anchor software is 3DV-ATMv7.0 and the tested is 3DV-ATMv7.0 with PSIP OFF. From Table 12, there is only slight coding gain loss without PSIP but decoding time are reduced about 7.24%. The decoding time reduction comes from the CABAC process of PSIP difference code.

Table 12 Coding performance, encoding/decoding time comparison

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR,dB | dBR, % | dPSNR, dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.13 | -0.01 | -0.17 | 0.00 | 0.09 | 0.00 | 0.85 | -0.03 | 99.49 | 91.92 | 96.60 |
| S02 | 0.08 | 0.00 | -0.26 | 0.02 | 0.01 | 0.00 | 0.09 | 0.00 | 100.28 | 90.68 | 97.06 |
| S03 | 0.08 | 0.00 | 0.06 | -0.01 | -0.02 | 0.00 | 0.70 | -0.02 | 99.93 | 93.54 | 101.69 |
| S04 | 0.04 | 0.00 | -1.09 | 0.07 | -0.06 | 0.00 | 0.08 | 0.00 | 99.87 | 94.39 | 100.31 |
| S05 | 0.08 | 0.00 | -0.52 | 0.02 | 0.01 | 0.00 | 0.17 | -0.01 | 99.69 | 93.17 | 98.93 |
| S06 | 0.02 | 0.00 | -0.80 | 0.04 | -0.07 | 0.00 | 0.02 | 0.00 | 100.00 | 93.50 | 101.62 |
| S08 | 0.02 | 0.00 | 0.44 | -0.02 | -0.07 | 0.00 | 0.17 | 0.00 | 99.71 | 92.14 | 99.25 |
| Average | 0.06 | 0.00 | -0.34 | 0.02 | -0.02 | 0.00 | 0.30 | -0.01 | 99.85 | 92.76 | 99.35 |

# Conclusion

From Table 1 to Table 12, ALC needs more number of operations, larger data storage, and higher data transfer rate than DCP/MCP; however the coding gain provided by ALC is significant. Trade-off between performance and complexity is usually a critical issue during the algorithm development. On the other hand, the PSIP increase slightly operations but the coding gain is slightly loss. (0.3 % in synthesized PSNR); however, it could provide the better visual quality on depth data.

With the help of the intrinsic complexity analysis, this proposal would assist the designer to evaluate the developed coding tools. This proposal provides not only an evaluated complexity of the coding tools but also an instruction on that how to assess the intrinsic algorithmic complexity. Due to the fact that the assessed complexity is independent from any platform or architecture, these numbers provide a fair complexity comparison. Hence, this proposal recommends that this analysis method would be took into consideration for the further coding tool development.

# Reference

- JCT3V-C1002, “3D-AVC Draft Text 5, “Geneva, CH, 17-23 January 2013.

- JCT3V-C1003, “3D-AVC Test Model 5, “Geneva, CH, 17-23 January 2013.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-3V and contributions to the JCT-3V are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-3V proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. Two examples of such an informal IPR notification statement for a contribution are provided below.)

NCKU Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).