| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0091 |
| --- | --- |

| Title: | CE5.h related : Inter-view SAO process in 3DV coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taesup, Kim Jin, Heo Sehoon, Yea #221, Yangjae-dong, Seocho-gu, Seoul, 137-130, Korea | Tel: Email: | +82-2-6912-6477 taesup.kim@lge.com jin78.heo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, SAO process in the 3D HEVC (HTM 6.0) [1] is migrated to that in the current HEVC (HM 9.1). The main modification is that SAO parameters are derived from LCU-based optimization (interleaving mode) rather than picture-based optimization (APS mode). Inter-view SAO process is proposed and modeled on the top of it to confirm the benefit of sharing base view’s SAO parameters on the dependent views through RD-optimization, and reportedly shows 0.3% and 0.3% gains on view 1 and 2, respectively.

# Introduction

SAO process consists of a number of parameters that each LCU has merging left or up flag and edge type index with corresponding offset values. These parameters are derived by RD-optimization. During RD optimization, neighbored LCUs could be merged or clustered into a single partition sharing a SAO edge type and offset values. Similarly, neighbored views could also have somewhat chances to reduce redundancies on SAO parameters.

# Proposed method

Inter-view SAO process [1] was proposed during the 3rd JCT-3V meeting with an assumption that SAO parameters could be shared between neighbored views without any additional process. It only derived and transmitted SAO parameters for the base view (view 0), and all dependent views (view 1, 2) share the base view’s derived SAO parameters. On dependent views, each LCU reuses SAO parameters of the collocated LCU on the base view. However, in this contribution, dependent views reuse base view’s SAO parameters with RD-optimization. Only edge offset values are reused, and SAO edge type is appropriately derived and transmitted from dependent views with reusing base view’s edge offset values. Moreover, merging process is also considered for dependent views during RD-optimization. Consequently, dependent views’ LCUs transmit newly derived merge flags and SAO edge type index without edge offset values.

# Experimental results

## Migration of SAO process to the current HEVC (HM 9.1)

Table 1 provides results on migration of SAO process. The current HTM 6.0 SAO process is replaced by the current HM 9.2 SAO process. Due to the main change on the partitioning process for SAO process, all views result changed.

Table 1. Rate-distortion and running time ratio results on 
‘Migration of SAO process of the current 3D HEVC (HTM 6.0) to the current HEVC (HM 9.1)’ 
(Anchor: 3D-HTM 6.0 CTC, Full-frame)

## Inter-view SAO process

Inter-view SAO process is designed on the top of migration of SAO process. Therefore, the result is compared to the result of migration of SAO process, as shown on Table 2.

Table 2. Rate-distortion and running time ratio results on 
‘Inter view-SAO process on the top of migration of SAO process’ 
(Anchor: 3D-HTM 6.0 CTC with migration of SAO process, Full-frame)

# Conclusion

In this contribution, it is reported that SAO parameters could be properly shared between neighbored views to reduce redundancies. It is recommended to adopt the proposed inter-view SAO process into HTM test model and the common test conditions.

# References

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments” , ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, Geneva, CH, January 2013.

- T. Kim, J. Heo, M. Koo, S. Yea, “CE5.h : Inter-view SAO process in 3DV coding”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0065, Geneva, CH, January 2013

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).