Error: Reference source not found

In 0.7, replace the last sentence before last paragraph with

Subclause Error: Reference source not found specifies three profiles for MVC (Multiview High, Stereo High and MFC High).

In 7.3.2.1.1, replace the syntax table with:

| seq_parameter_set_data( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| constraint_set4_flag | 0 | u(1) |
| constraint_set5_flag | 0 | u(1) |
| reserved_zero_2bits /* equal to 0 */ | 0 | u(2) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 244 ¦ ¦ profile_idc = = 44 ¦ ¦ profile_idc = = 83 ¦ ¦ profile_idc = = 86 ¦ ¦ profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139 ¦ ¦ profile_idc = = 134 ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| separate_colour_plane_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < ( ( chroma_format_idc != 3 ) ? 8 : 12 ); i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i − 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i − 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| max_num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| } |  |  |

In 7.3.2.1.3, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139 ¦ ¦ profile_idc = = 134 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ |  |  |
| } |  |  |
| if( profile_idc = = 138 ¦ ¦ profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_3dvc_extension( ) /* specified in Annex I */ | 0 |  |
| } |  |  |
| additional_extension2_flag | 0 | u(1) |
| if( additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In 7.4.2.1.1 replace the definition of the following syntax element:

constraint_set4_flag is specified as follows:

–	If profile_idc is equal to 77, 88, or 100, constraint_set4_flag equal to 1 indicates that the value of frame_mbs_only_flag is equal to 1. constraint_set4_flag equal to 0 indicates that the value of frame_mbs_only_flag may or may not be equal to 1.

–	Otherwise, if profile_idc is equal to 118, 128 or 134, constraint_set4_flag equal to 1 indicates that the coded video sequence obeys all constraints specified in subclause Error: Reference source not found. constraint_set4_flag equal to 0 indicates that the coded video sequence may or may not obey the constraints specified in subclause Error: Reference source not found.

–	Otherwise (profile_idc is not equal to 77, 88, 100, 118, 128, or 134), the value of 1 for constraint_set4_flag is reserved for future use by ITU-T | ISO/IEC. constraint_set4_flag shall be equal to 0 for coded video sequences with profile_idc not equal to 77, 88, 100, 118, 128, or 134 in bitstreams conforming to this Recommendation | International Standard. Decoders shall ignore the value of constraint_set4_flag when profile_idc is not equal to 77, 88, 100, 118, 128, or 134.

In 7.4.2.1, replace NOTE2 with the following note:

NOTE 2 – For a coded video sequence conforming to all Multiview High, Stereo High and MFC High profiles, the profile_idc should be equal to 118 and constraint_set5 flag should be equal to 1.

In H.3, replace the following definition:

H.3.44	view component: A coded representation of a view in a single access unit. When profile_idc is equal to 134, a view contains samples of two distinct spatially packed constituent frames that are packed into one frame using one of the frame packing arrangement schemes as specified in subclause D.2.25.

In H.3, add the following definition:

H.3.x	reference processing unit: A unit which processes an inter-view prediction reference before it is used for inter-view prediction in the decoding process of subsequent view components in decoding order.

H.3.x	left view: An interpretation of left view is defined as content_interpretation_type equal to 1 as specified in subclause D.2.25.

H.3.x	right view: An interpretation of right view is defined as content_interpretation_type equal to 1 as specified in subclause D.2.25.

In H.4, add the following additional abbreviations:

MFC	Multi-resolution Frame Compatible stereo coding

RPU	Reference Processing Unit

In H.7.3.2.1.4, replace the syntax table with:

| seq_parameter_set_mvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| num_views_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_views_minus1; i++ ) |  |  |
| view_id[ i ] | 0 | ue(v) |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| num_level_values_signalled_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) { |  |  |
| level_idc[ i ] | 0 | u(8) |
| num_applicable_ops_minus1[ i ] | 0 | ue(v) |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) { |  |  |
| applicable_op_temporal_id[ i ][ j ] | 0 | u(3) |
| applicable_op_num_target_views_minus1[ i ][ j ] | 0 | ue(v) |
| for( k = 0; k <= applicable_op_num_target_views_minus1[ i ][ j ]; k++ ) |  |  |
| applicable_op_target_view_id[ i ][ j ][ k ] | 0 | ue(v) |
| applicable_op_num_views_minus1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| if( profile_idc = = 134 ) { |  |  |
| reserved_zero_6bits | 0 | u(6) |
| rpu_format | 0 | u(6) |
| if( rpu_format = = 0 ¦ ¦ rpu_format = = 1 ) { |  |  |
| default_grid_position_flag | 0 | u(1) |
| if( !default_grid_position_flag ) { |  |  |
| view0_grid_position_x | 0 | u(4) |
| view0_grid_position_y | 0 | u(4) |
| view1_grid_position_x | 0 | u(4) |
| view1_grid_position_y | 0 | u(4) |
| } |  |  |
| } |  |  |
| rpu_filter_enabled_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| rpu_field_processing_flag | 0 | u(1) |
| } |  |  |
| } |  |  |

In H.7.4.2.1.4, add the following note after the semantics of num_views_minus1:

NOTE 3 – When profile_idc is equal to 134, num_views_minus1 equal to 0 indicates that the bitstream is a frame compatible bitstream. num_views_minus1 equal to 1 indicates that the base view bitstream (corresponding to VOIdx equal to 0) comprises a frame compatible bitstream and the enhancement view (corresponding to VOIdx equal to 1) comprises a resolution enhancement to the frame compatible bitstream.

In H.7.4.2.1.4, add the following semantics:

reserved_zero_6bits shall be equal to 0. Other values of reserved_zero_6bits may be specified in the future by ITU T | ISO/IEC. Decoders shall ignore the value of reserved_zero_6bits.-.

rpu_format specifies the frame packing arrangement type for view components in the base view and the corresponding frame packing arrangement type for view components in the non-base view. The semantics of rpu_format equal to 0 and 1 are as specified in Table H-X. In bitstreams conforming to this version of this Specification, the value of rpu_format shall be equal to 0 or 1. Values of rpu_format in the range of 2..63 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the coded video sequence with reserved values of rpu_format.

Table H-X – Association between frame packing arrangement type and syntax elements

| rpu_format | frame packing arrangement type for view components in the base view | frame packing arrangement type for view components in the non-base view |
| --- | --- | --- |
| 0 | frame_packing_arrangement_type equal to 3 and quincunx_sampling_flag equal to 0: side-by-side | frame_packing_arrangement_type equal to 4 and quincunx_sampling_flag equal to 0: top-bottom |
| 1 | frame_packing_arrangement_type equal to 4 and quincunx_sampling_flag equal to 0: top-bottom | frame_packing_arrangement_type equal to 3 and quincunx_sampling_flag equal to 0: side-by-side |

default_grid_position_flag equal to 0 specifies that the syntax elements view0_grid_position_x, view0_grid_position_y, view1_grid_position_x, and view1_grid_position_y are present. default_grid_position_flag equal to 1 specifies that view0_grid_position_x, view0_grid_position_y, view1_grid_position_x, and view1_grid_position_y are not present.

view0_grid_position_x has the same semantics as specified in subclause D.2.25 for the frame0_grid_position_x syntax element. The value of view0_grid_position_x shall be equal to 4, 8 or 12.

view0_grid_position_y has the same semantics as specified in subclause D.2.25 for the frame0_grid_position_y syntax element. The value of view0_grid_position_y shall be equal to 4, 8 or 12.

view1_grid_position_x has the same semantics as specified in subclause D.2.25 for the frame1_grid_position_x syntax element. The value of view1_grid_position_x shall be equal to 4, 8 or 12.

view1_grid_position_y has the same semantics as specified in subclause D.2.25 for the frame1_grid_position_y syntax element. The value of view1_grid_position_y shall be equal to 4, 8 or 12.

When default_grid_position_flag is equal to 1, the values of view0_grid_position_x, view0_grid_position_y, view1_grid_position_x, and view1_grid_position_y are inferred as follows.

–	If rpu_format is equal to 0, the following applies:

–	view0_grid_position_x is inferred to be equal to 4,

–	view0_grid_position_y is inferred to be equal to 8,

–	view1_grid_position_x is inferred to be equal to 12,

–	view1_grid_position_y is inferred to be equal to 8.

–	Otherwise (rpu_format is equal to 1), the following applies:

–	view0_grid_position_x is inferred to be equal to 8,

–	view0_grid_position_y is inferred to be equal to 4,

–	view1_grid_position_x is inferred to be equal to 8,

–	view1_grid_position_y is inferred to be equal to 12.

rpu_filter_enabled_flag equal to 1 specifies that a set of downsampling filter and upsampling filter are used to generate each color component of the inter-view prediction reference. rpu_filter_enabled_flag equal to 0 specifies that the pixel value of each color component of the inter-view prediction reference is set to 128.

rpu_field_processing_flag equal to 0 specifies that an inter-view prediction reference is processed as a frame. rpu_field_processing_flag equal to 1 specifies that the two fields of an inter-view prediction reference are processed separately. When not present, the value of rpu_field_processing_flag is inferred to be equal to 0.

NOTE – For backward compatibility, frame packing SEI message is required to be present. The frame packing related syntax elements view0_grid_position_x, view0_grid_position_y, view1_grid_position_x and view1_grid_position_y are expected to match the syntax elements frame0_grid_position_x, frame0_grid_position_y, frame1_grid_position_x and frame1_grid_position_y, respectively, in the frame packing SEI message.

In H.8, add the following paragraph after the last paragraph:

When profile_idc is equal to 134, subclause H.8.4.1 specifies the additional processing for an inter-view prediction reference, and subclause H.8.6 describes the recommended enhanced-resolution picture reconstruction process.

In H.8.4, add the following sentence after the last sentence of the first paragraph:

When profile_idc is equal to 134, subclause H.8.4.1 specifies the additional processing for an inter-view prediction reference.

Add a new subclause H.8.4.1

- H.8.4.1	Processing of the luma and chroma components of an inter-view prediction reference

This process is invoked to modify each colour component of an inter-view prediction reference used in decoding view components of the non-base view in an MFC coded video sequence. The modification process consists of a one-dimensional downsampling filtering process followed by a one-dimensional upsampling filtering process. The modification process converts an inter-view prediction reference from the frame packing arrangement format of view components in the base view to the frame packing arrangement format of view components in the non-base view.

The derivation of the modified inter-view prediction reference shall obey the following constraints:

- The processing shall be limited within either the left or the right view only,

- During filtering, samples outside the view boundary are replicated with the same value as the boundary sample.

Inputs of this process are:

- two variables refW and refH specifying the width and height, respectively, of the inter-view prediction reference picture sample array,

- a (refW)x(refH) inter-view prediction reference picture sample array refPicture for either the luma or a chroma component,

Output of this process is a (refW)x(refH) modified inter-view prediction reference picture sample array RpuPicture for either the luma or a chroma component.

Depending on rpu_field_processing_flag, the following applies:

- If rpu_field_processing_flag is equal to 1, the inter-view prediction reference picture refPicture is a field, and the modified inter-view prediction reference picture RpuPicture is a field. The inter-view prediction reference modification is applied to the individual inter-view prediction reference fields separately.

- Otherwise (rpu_field_processing_flag is equal to 0), the inter-view prediction reference picture refPicture is a frame, and the modified inter-view prediction reference picture RpuPicture is a frame. The inter-view prediction reference modification is applied to the inter-view prediction reference frame.

The mathematical function Clip1( ) is defined with Clip1( ) being substituted with Clip1Y( ) for the luma component and Clip1( ) being substituted with Clip1C( ) for the chroma components, respectively.

The variables refW and refH are derived as follows:

- For the luma component, refW is set equal to PicWidthInSamplesL and refH is set equal to PicHeightInSamplesL.

- For the chroma components, refW is set equal to PicWidthInSamplesC and refH is set equal to PicHeightInSamplesC.

The variables subW and subH are derived as follows:

- For the luma component, subW is set equal to SubWidthC and subH is set equal to SubHeightC.

- For the chroma components, subW is set equal to 1 and subH is set equal to 1.

The variable view0OffsetX is derived as follows:

- If view0_grid_position_x is equal to 4 or 8, view0OffsetX is set equal to 0,

- Otherwise (view0_grid_position_x is equal to 12), view0OffsetX is set equal to 1.

The variable view1OffsetX is derived as follows:

- If view1_grid_position_x is equal to 4 or 8, view1OffsetX is set equal to 0,

- Otherwise (view1_grid_position_x is equal to 12), view1OffsetX is set equal to 1.

The variable view0OffsetY is derived as follows:

- If view0_grid_position_y is equal to 4 or 8, view0OffsetY is set equal to 0,

- Otherwise (view0_grid_position_y is equal to 12), view0OffsetY is set equal to 1.

The variable view1OffsetY is derived as follows:

- If view1_grid_position_y is equal to 4 or 8, view1OffsetY is set equal to 0,

- Otherwise (view1_grid_position_y is equal to 12), view1OffsetY is set equal to 1.

The variable RpuW specifying the width of an active area of the reference picture is derived as specified by the following ordered steps:

- leftOffset = frame_crop_left_offset* subW

- rightOffset = frame_crop_right_offset * subW

- RpuW = refW − leftOffset − rightOffset 	(H-XX)

The variable RpuH specifying the height of an active area of the reference picture is derived as specified by the following ordered steps:

- topOffset = frame_crop_top_offset * subH * ( 2 − frame_mbs_only_flag )

- botOffset = frame_crop_bottom_offset * subH * ( 2 − frame_mbs_only_flag )

- if ( rpu_field_processing_flag )

topOffset = topOffset >> 1, botOffset = botOffset >> 1

- RpuH = refH − topOffset − botOffset 	(H-XX)

The variable SbsV is set equal to RpuW >> 1. In the side-by-side arrangement, the view boundary between the left and right views is set to SbsV+ leftOffset.

The variable TabV is set equal to RpuH >> 1. In the top-bottom arrangement, the view boundary between the left and right views is set to TabV+ topOffset.

The filtered samples of picture sample array rpuPicture[ x, y ] with x = 0..(refW − 1) and, y = 0..(refH − 1), are derived as follows:

- If rpu_filter_enabled_flag is equal to 0,

RpuPicture[ x, y ] = 128 with x = 0..(refW − 1) and y = 0..(refH − 1) 	(H-XX)

- Otherwise (rpu_filter_enabled_flag is equal to 1),

Let tempPicture[ x, y ] be a (SbsV)x(TabV) array of samples with x = 0..SbsV − 1 and y = 0..TabV − 1.

  - If rpu_format is equal to 0,

Let tempRefPic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

Let tempRpuPic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

- Otherwise (rpu_format is equal to 1),

Let tempRefPic[ x, y ]  be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

Let tempRpuPic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

Let gMin be a variable.

The filtered samples of picture sample array RpuPicture[ x, y ] with x = leftOffset..RpuW − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset are derived as specified by the following ordered steps:

- If rpu_format is equal to 0,

- RpuPicture[ x, y ] with x = leftOffset.. RpuW − 1 + leftOffset and y = topOffset..TabV − 1 + topOffset is derived from the input of the array refPicture[ x, y ] with x = leftOffset..SbsV − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset as specified by the following ordered steps:

- tempRefPic[ x, y ] = refPicture[ x + leftOffset , y + topOffset ]

with x = 0..SbsV − 1 and y = 0..RpuH − 1	(H-XX)

- Apply one-dimensional downsampling process:

tempPicture[ x, y ] = Clip1 ( ( 4 * tempRefPic[ x, 2 * y − 2 ] + 7 * tempRefPic[ x, 2 * y − 1 ] +                                          10 * tempRefPic[ x, 2 * y ] + 7 * tempRefPic[ x, 2 * y + 1 ] +
4 * tempRefPic[ x, 2 * y + 2 ] + 32 ) >> 6) 	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV − 1

- Apply one-dimensional upsampling process:

gMin = − 2 – View0OffsetX,	(H-XX)

tempRpuPic[ 2 * x + view0OffsetX, y ] = tempPicture[ x, y ],	(H-XX)

tempRpuPic[ 2 * x + ( 1 – view0OffsetX ), y ] = Clip1( ( 3 * tempPicture[ x + gMin, y ] −
 17 * tempPicture[ x + gMin + 1, y ] + 78 * tempPicture[ x + gMin + 2, y ] +
 78 * tempPicture[ x + gMin + 3, y ] − 17 * tempPicture[ x + gMin + 4, y ] +
 3 * tempPicture[ x + gMin + 5, y ] + 64) >> 7)	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV − 1

- RpuPicture[ x + leftOffset, y  + topOffset] = tempRpuPic[ x, y ]

with x = 0..RpuW − 1 and y = 0..TabV − 1	(H-XX)

- RpuPicture[ x, y ] with x = leftOffset..RpuW − 1 + leftOffset and y = TabV + topOffset..RpuH − 1 + topOffset is derived from the input of the array refPicture[ x, y ] with x = SbsV + leftOffset..RpuW − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset as specified by the following ordered steps:

- tempRefPic[ x, y ] = refPicture[ x + SbsV + leftOffset , y + topOffset ] with x = 0..SbsV − 1 and y = 0..RpuH − 1	(H-XX)

- Apply one-dimensional downsampling process:

tempPicture[ x, y ] = Clip1 ( ( 4 * tempRefPic[ x, 2 * y − 2 ] + 7 * tempRefPic[ x, 2 * y − 1 ] +                                          10 * tempRefPic[ x, 2 * y ] + 7 * tempRefPic[ x, 2 * y + 1 ] +
 4 * tempRefPic[ x, 2 * y + 2 ] + 32 ) >> 6) 	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV − 1

- Apply one-dimensional upsampling process:

gMin = − 2 – View1OffsetX,	(H-XX)

tempRpuPic[ 2 * x + view1OffsetX, y ] = tempPicture[ x, y ],	(H-XX)

tempRpuPic[ 2 * x + ( 1 – view1OffsetX ), y ] = Clip1( ( 3 * tempPicture[ x + gMin, y ] −
 17 * tempPicture[ x + gMin + 1, y ] + 78 * tempPicture[ x + gMin + 2, y ] +
 78 * tempPicture[ x + gMin + 3, y ] − 17 * tempPicture[ x + gMin + 4, y ] +
 3 * tempPicture[ x + gMin + 5, y ] + 64) >> 7)	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV − 1

- RpuPicture[ x + leftOffset, y + TabV + topOffset ] = tempRpuPic[ x, y ]  with x = 0..RpuW − 1 and y = 0..TabV − 1	(H-XX)

- Otherwise ( rpu_format is equal to 1),

- RpuPicture[ x, y ] with x = leftOffset..SbsV − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset is derived from the input of the array refPicture[ x, y ] with x = leftOffset..RpuW − 1 + leftOffset and y = topOffset..TabV − 1 + topOffset as specified by the following ordered steps:

- tempRefPic[ x, y ] = refPicture[ x + leftOffset, y + topOffset ] with x = 0..RpuW − 1 and y = 0..TabV − 1		(H-XX)

- Apply one-dimensional downsampling process:

tempPicture[ x, y ] = Clip1 ( ( 4 * tempRefPic[ 2 * x − 2, y ] + 7 * tempRefPic[2 * x − 1, y ] +                                          10 * tempRefPic[ 2 * x, y ] + 7 * tempRefPic[ 2 * x + 1, y ] +
4 * tempRefPic[ 2 * x + 2, y ] + 32 ) >> 6) 	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV − 1

- Apply one-dimensional upsampling process:

gMin = − 2 – View0OffsetY,	(H-XX)

tempRpuPic[ x, 2 * y + view0OffsetY ] = tempPicture[ x, y ],	(H-XX)

tempRpuPic[ x, 2 * y + ( 1 – view0OffsetY ) ] = Clip1( ( 3 * tempPicture[ x, y + gMin ] −
 17 * tempPicture[ x, y + gMin + 1 ] + 78 * tempPicture[ x, y + gMin + 2 ] +
 78 * tempPicture[ x, y + gMin + 3 ] − 17 * tempPicture[ x, y + gMin + 4 ] +
 3 * tempPicture[ x, y + gMin + 5 ] + 64 ) >> 7)	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV − 1

- RpuPicture[ x + leftOffset, y + topOffset ] = tempRpuPic[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1	(H-XX)

- RpuPicture[ x, y ] with x = SbsV + leftOffset..RpuW − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset is derived from the input of the array refPicture[ x, y ] with x = leftOffset..RpuW − 1 + leftOffset and y = TabV + topOffset..RpuH − 1 + topOffset as specified by the following ordered steps:

- tempRefPic[ x, y ] = refPicture[ x + leftOffset, y + topOffset + TabV ]

with x = 0..RpuW − 1 and y = 0..TabV − 1	(H-XX)

- Apply one-dimensional downsampling process:

tempPicture[ x, y ] = Clip1 ( ( 4 * tempRefPic[ 2 * x − 2, y ] + 7 * tempRefPic[2 * x − 1, y ] +                                          10 * tempRefPic[ 2 * x, y ] + 7 * tempRefPic[ 2 * x + 1, y ] +
4 * tempRefPic[ 2 * x + 2, y ] + 32 ) >> 6) 	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV – 1

- Apply one-dimensional upsampling process:

gMin = − 2 – View1OffsetY,	(H-XX)

tempRpuPic[ x, 2 * y + view1OffsetY ] = tempPicture[ x, y ],	(H-XX)

tempRpuPic[ x, 2 * y + ( 1 – view1OffsetY ) ] ] = Clip1( ( 3 * tempPicture[ x, y + gMin ] −
 17 * tempPicture[ x, y + gMin + 1 ] + 78 * tempPicture[ x, y + gMin + 2 ] +
 78 * tempPicture[ x, y + gMin + 3 ] − 17 * tempPicture[ x, y + gMin + 4 ] +
 3 * tempPicture[ x, y + gMin + 5 ] + 64 ) >> 7)	(H-XX)

with x = 0..SbsV − 1 and y = 0..TabV – 1

- RpuPicture[ x + SbsV + leftOffset, y + topOffset ] = tempRpuPic[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1	(H-XX)

The padded filtered samples of picture sample array RpuPicture[ x, y ] outside frame cropping rectangle with x = 0..leftOffset − 1 or x = RpuW + leftOffset..refW – 1or y = 0..topOffset − 1 or y = RpuH + topOffset..refH – 1 are derived as specified by the following ordered steps:

- RpuPicture[ x, y ] = RpuPicture[ leftOffset, y ] with x = 0.. leftOffset − 1 and y = topOffset..RpuH − 1+ topOffset	(H-XX)

- RpuPicture[ x, y ] = RpuPicture[ RpuW − 1 + leftOffset, y ] with x = RpuW+ leftOffset..refW − 1 and y = topOffest..RpuH − 1+ topOffset	(H-XX)

- RpuPicture[ x, y ] = RpuPicture[ x, topOffset ] with x =  0..refW − 1 and y = 0..topOffset − 1	(H-XX)

- RpuPicture[ x, y ] = RpuPicture[ x, RpuH − 1 + topOffset] with x =  0..refW − 1 and y = RpuH+ topOffset..refH – 1	(H-XX)

NOTE – If each view component in the base view is a side-by-side frame packing arrangement picture, the inter-view reference picture is first vertically downsampled and then horizontally upsampled in a conversion to the top-bottom format. Otherwise (each view component in the base view is a top-bottom frame packing arrangement picture), the inter-view reference picture is first horizontally downsampled and then vertically upsampled in a conversion to the side-by-side format.

Add a new subclause H.8.6 “MFC enhanced resolution picture reconstruction”

- H.8.6	MFC enhanced resolution picture reconstruction

This subclause does not form an integral part of this Recommendation | International Standard.

This informative subclause describes a process for reconstructing the enhanced resolution stereo views using MFC coded video sequences  The process is applied to each colour component independently.

Inputs of this process are:

- a (refW)x(refH) modified inter-view prediction reference picture sample array RpuPicture derived in subclause H.8.4.1,

- a (RpuW)x(RpuH) cropped decoded picture sample array decBasePicture for a luma or a chroma component of the base view component with VOIdx equal to 0,

- a (RpuW)x(RpuH) cropped decoded picture sample array decEnhPicture for a luma or a chroma component of the view component with VOIdx equal to 1.

Outputs of this process are:

- a (RpuW)x(RpuH) enhanced resolution picture sample array leftPicture for a luma or a chroma component of the left view,

- a (RpuW)x(RpuH) enhanced resolution picture sample array rightPicture for a luma or a chroma component of the right view.

Depending on rpu_field_processing_flag, the following applies:

- If rpu_field_processing_flag is equal to 1, the modified inter-view prediction reference picture RpuPicture is a field, the cropped decoded view component of the base view, decBasePicture, is a field, the cropped decoded view component of the non-base view, decEnhPicture, is a field, the enhanced resolution left view picture leftPicture is a field, and the enhanced resolution right view picture rightPicture is a field. The enhanced resolution picture reconstruction is applied to the individual fields separately.

- Otherwise (rpu_field_processing_flag is equal to 0), the modified inter-view prediction reference picture RpuPicture is a frame, the cropped decoded view component of the base view, decBasePicture, is a frame, the cropped decoded view component of the non-base view, decEnhPicture, is a frame, the enhanced resolution left view picture leftPicture is a frame, and the enhanced resolution right view picture rightPicture is a frame. The enhanced resolution picture reconstruction is applied to the frame.

The mathematical function Clip1( ) is defined with Clip1( ) being substituted with Clip1Y( ) for the luma component and Clip1( ) being substituted with Clip1C( ) for the chroma components, respectively.

The variable tVal is set equal to ( 1 << ( BitDepthY − 1 ) ) for the luma component and tVal is set equal to ( 1 << ( BitDepthC − 1 ) ) for a chroma component, respectively.

Let upBasePic[ x, y ] be an (RpuW)x(RpuH) array of samples with x = 0..RpuW − 1 and y = 0..RpuH − 1.

Let resPicture[ x, y ] be an (RpuW)x(RpuH) array of samples with x = 0..RpuW − 1 and y = 0..RpuH − 1.

Let upResPic[ x, y ] be an (RpuW)x(RpuH) array of samples with x = 0..RpuW − 1 and y = 0..RpuH − 1.

Let gMin be a variable.

- If rpu_format is equal to 0,

Let tempDecBasePic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

Let tempDecEnhPic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

Let tempRpuPic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

- Otherwise (rpu_format is equal to 1),

Let tempDecBasePic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

Let tempDecEnhPic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

Let tempRpuPic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

The samples of enhanced resolution picture sample array for the left view leftPicture[ x, y ] and the right view rightPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 are derived as follows:

- If rpu_format is equal to 0,

- leftPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1, decEnhPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1 and RpuPicture [ x, y ] with x = leftOffset..RpuW − 1 + leftOffset and y = topOffset..TabV − 1 + topOffset as specified by the following ordered steps:

- tempDecBasePic[ x, y ] = decBasePicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- tempDecEnhPic[ x, y ] = decEnhPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1.

- tempRpuPic[ x, y ] = RpuPicture[ x + leftOffset, y + topOffset ] with x = 0..RpuW− 1 and y = 0..TabV − 1.

- Apply one-dimensional upsampling process:

gMin = − 2 – View0OffsetX,	(H-XX)

upBasePic[ 2 * x + view0OffsetX, y ] = tempDecBasePic[ x, y ],	(H-XX)

upBasePic[2 * x + ( 1 – view0OffsetX ), y ] = Clip1( ( 3 * tempDecBasePic[ x + gMin, y ] −
 17 * tempDecBasePic[ x + gMin + 1, y ] + 78 * tempDecBasePic[ x + gMin + 2, y ] +
 78 * tempDecBasePic[ x + gMin + 3, y ] − 17 * tempDecBasePic[ x + gMin + 4, y ] +
 3 * tempDecBasePic[ x + gMin + 5, y ] + 64 ) >> 7)	(H-XX)

with x = 0..SbsV − 1 and y = 0..RpuH − 1

- resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )	(H-XX)

with x = 0..RpuW − 1 and y = 0..TabV − 1

- Apply one-dimensional upsampling process:

upResPic[ x, 2 * y] = resPicture[ x, y ]	(H-XX)

upResPic[ x, 2 * y + 1 ] = Clip3( − tVal, tVal − 1, ( 3 * resPicture[ x, y − 2 ] −
 17 * resPicture[ x, y − 1 ] + 78 * resPicture[ x, y ] + 78 * resPicture[ x, y + 1 ] −
 17 * resPicture[ x, y + 2 ] + 3 * resPicture[ x, y + 3 ] + 64 ) >> 7 ) )	(H-XX)

with x = 0..RpuW − 1 and y = 0..TabV − 1

- leftPicture[ x, y ] = Clip1( upBasePic[ x, y ] + upResPic[ x, y ] )	(H-XX)

with x = 0..RpuW − 1 and y = 0..RpuH − 1

- rightPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1, decEnhPicture[ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1 and RpuPicture [ x, y ] with x = leftOffset..RpuW − 1 + leftOffset and y = TabV + topOffset..RpuH − 1 + topOffset as specified by the following ordered steps:

- tempDecBasePic[ x, y ] = decBasePicture[ x + SbsV, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- tempDecEnhPic[ x, y ] = decEnhPicture[ x, y + TabV ] with x = 0..RpuW − 1 and y = 0..TabV − 1.

- tempRpuPic[ x, y ] = RpuPicture[ x + leftOffset, y + TabV + topOffset ] with x = 0..RpuW− 1 and y = 0..TabV − 1.

- Apply one-dimensional upsampling process:

gMin = − 2 – View1OffsetX,	(H-XX)

upBasePic[ 2 * x + view1OffsetX, y ] = tempDecBasePic[ x, y ],	(H-XX)

upBasePic[2 * x + ( 1 – view1OffsetX ), y ] = Clip1( ( 3 * tempDecBasePic[ x + gMin, y ] −
 17 * tempDecBasePic[ x + gMin + 1, y ] + 78 * tempDecBasePic[ x + gMin + 2, y ] +
 78 * tempDecBasePic[ x + gMin + 3, y ] − 17 * tempDecBasePic[ x + gMin + 4, y ] +
 3 * tempDecBasePic[ x + gMin + 5, y ] + 64 ) >> 7)	(H-XX)

with x = 0..SbsV − 1 and y = 0..RpuH − 1

- resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )	(H-XX)

with x = 0..RpuW − 1 and y = 0..TabV − 1

- Apply one-dimensional upsampling process:

upResPic[ x, 2 * y] = resPicture[ x, y ]	(H-XX)

upResPic[ x, 2 * y + 1 ] = Clip3( − tVal, tVal − 1, ( 3 * resPicture[ x, y − 2 ] −
 17 * resPicture[ x, y − 1 ] + 78 * resPicture[ x, y ] + 78 * resPicture[ x, y + 1 ] −
 17 * resPicture[ x, y + 2 ] + 3 * resPicture[ x, y + 3 ] + 64 ) >> 7 ) )	(H-XX)

with x = 0..RpuW − 1 and y = 0..TabV − 1

- rightPicture[ x, y ] = Clip1( upBasePic[ x, y ] + upResPic[ x, y ] )	(H-XX)

with x = 0..RpuW − 1 and y = 0..RpuH − 1

- Otherwise ( rpu_format is equal to 1 ),

- leftPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1, decEnhPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1 and RpuPicture [ x, y ] with x = leftOffset..SbsV − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset as specified by the following ordered steps:

- tempDecBasePic[ x, y ] = decBasePicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1.

- tempDecEnhPic[ x, y ] = decEnhPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- tempRpuPic[ x, y ] = RpuPicture[ x + leftOffset, y + topOffset ] with x = 0..SbsV− 1 and y = 0..RpuH − 1.

- Apply one-dimensional upsampling process:

gMin = − 2 – View0OffsetY,	(H-XX)

upBasePic[ x, 2 * y + view0OffsetY ] = tempDecBasePic[ x, y ],	(H-XX)

upBasePic[ x , 2 * y + ( 1 – view0OffsetY ) ] = Clip1( ( 3 * tempDecBasePic[ x, y + gMin ] −
 17 * tempDecBasePic[ x, y + gMin + 1 ] + 78 * tempDecBasePic[ x, y + gMin + 2 ] +
 78 * tempDecBasePic[ x, y + gMin + 3 ] − 17 * tempDecBasePic[ x, y + gMin + 4 ] +
 3 * tempDecBasePic[ x, y + gMin + 5 ] + 64) >> 7)	(H-XX)

with x = 0..RpuW − 1 and y = 0..TabV − 1.

- resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )	(H-XX)

with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- Apply one-dimensional upsampling process:

upResPic[ 2 * x, y ] = resPicture[ x, y ]	(H-XX)

upResPic[ 2 * x + 1, y ] = Clip3(− tVal, tVal − 1, ( 3 * resPicture[ x − 2, y ] −
 17 * resPicture[ x − 1, y ] + 78* resPicture [ x, y ] + 78* resPicture [ x + 1, y ] −
 17 * resPicture[ x + 2, y ] + 3* resPicture [ x + 3, y ] + 64 ) >> 7 ) )	(H-XX)

with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- leftPicture[ x, y ] = Clip1( upBasePic[ x, y ] + upResPic[ x, y ] )	(H-XX)

with x = 0..RpuW − 1 and y = 0..RpuH − 1

- rightPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1, decEnhPicture[ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1 and RpuPicture [ x, y ] with x = SbsV + leftOffset..RpuW − 1 + leftOffset and y = topOffset..RpuH − 1 + topOffset as specified by the following ordered steps:

- tempDecBasePic[ x, y ] = decBasePicture[ x, y + TabV ] with x = 0..RpuW − 1 and y = 0..TabV − 1.

- tempDecEnhPic[ x, y ] = decEnhPicture[ x + SbsV, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- tempRpuPic[ x, y ] = RpuPicture[ x + SbsV + leftOffset, y + topOffset ] with x = 0..SbsV− 1 and y = 0..RpuH − 1.

- Apply one-dimensional upsampling process:

gMin = − 2 – View1OffsetY,	(H-XX)

upBasePic[ x, 2 * y + view1OffsetY ] = tempDecBasePic[ x, y ],	(H-XX)

upBasePic[ x , 2 * y + ( 1 – view1OffsetY ) ] = Clip1( ( 3 * tempDecBasePic[ x, y + gMin ] −
 17 * tempDecBasePic[ x, y + gMin + 1 ] + 78 * tempDecBasePic[ x, y + gMin + 2 ] +
 78 * tempDecBasePic[ x, y + gMin + 3 ] − 17 * tempDecBasePic[ x, y + gMin + 4 ] +
 3 * tempDecBasePic[ x, y + gMin + 5 ] + 64) >> 7)	(H-XX)

with x = 0..RpuW − 1 and y = 0..TabV − 1.

- resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )	(H-XX)

with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- Apply one-dimensional upsampling process:

upResPic[ 2 * x, y ] = resPicture[ x, y ]	(H-XX)

upResPic[ 2 * x + 1, y ] = Clip3(− tVal, tVal − 1, ( 3 * resPicture[ x − 2, y ] −
 17 * resPicture[ x − 1, y ] + 78* resPicture [ x, y ] + 78* resPicture [ x + 1, y ] −
 17 * resPicture[ x + 2, y ] + 3* resPicture [ x + 3, y ] + 64 ) >> 7 ) )	(H-XX)

with x = 0..SbsV − 1 and y = 0..RpuH − 1.

- rightPicture[ x, y ] = Clip1( upBasePic[ x, y ] + upResPic[ x, y ] )	(H-XX)

with x = 0..RpuW − 1 and y = 0..RpuH − 1

Add a new subclause H.10.1.3 “MFC High profile”

- H.10.1.3	MFC High profile

Bitstreams conforming to the MFC High profile shall obey the following constraints:

–	The base view bitstream as specified in subclause H.8.5.4 shall obey all constraints of the High profile specified in subclause A.2.4 and all active sequence parameter sets shall fulfil one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	MVC sequence parameter sets shall have chroma_format_idc in the range of 0 to 1 inclusive.

–	MVC sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	MVC sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	MVC sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	When num_views_minus1 is present in an MVC sequence parameter set, its value shall be less than 2.

–	For each access unit, the value of level_idc for all active view MVC sequence parameter set RBSPs shall be the same as the value of level_idc for the active MVC sequence parameter set RBSP.

–	The level constraints specified for the Stereo High profile in subclause H.10.2 shall be fulfilled.

Conformance of a bitstream to the MFC High profile is indicated by profile_idc being equal to 134.

Decoders conforming to the MFC High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active MVC sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 134,

–	profile_idc is equal to 128,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active MVC sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

In H.10.2.1, replace the title with the following title:

H.10.2.1	Level limits common to Multiview High, Stereo High and MFC High profiles

In H.10.2.1, replace the paragraph

Error: Reference source not found specifies the limits for each level. A definition of all levels identified in the "Level number" column of Error: Reference source not found is specified for the Multiview High and Stereo High profiles. Error: Reference source not found specifies limits for each level that are specific to bitstreams conforming to the Multiview High and Stereo High profiles. Each entry in Tables Error: Reference source not found and Error: Reference source not found indicates, for the level corresponding to the row of the table, the absence or value of a limit that is imposed by the variable corresponding to the column of the table, as follows:

with the following paragraph:

Error: Reference source not found specifies the limits for each level. A definition of all levels identified in the "Level number" column of Error: Reference source not found is specified for the Multiview High, Stereo High amd MFC High profiles. Error: Reference source not found specifies limits for each level that are specific to bitstreams conforming to the Multiview High, Stereo High and MFC High profiles. Each entry in Tables Error: Reference source not found and Error: Reference source not found indicates, for the level corresponding to the row of the table, the absence or value of a limit that is imposed by the variable corresponding to the column of the table, as follows:

In H.10.2.1, replace the paragraph

For coded video sequences conforming to the Multiview High or Stereo High profile, the level_idc value is specified as follows:

with the following paragraph:

For coded video sequences conforming to the Multiview High, Stereo High  or MFC High profile, the level_idc value is specified as follows:

In H.10.2.1, replace the paragraph

In bitstreams conforming to the Multiview High or Stereo High profiles, the conformance of the bitstream to a specified level is indicated by the syntax element level_idc or level_idc[ i ] as follows:

with the following paragraph:

In bitstreams conforming to the Multiview High, Stereo High or MFC High profile, the conformance of the bitstream to a specified level is indicated by the syntax element level_idc or level_idc[ i ] as follows:

In H.10.2.1, replace NOTE4 with the following note:

NOTE 4 – When profile_idc is equal to 118 128, or 134 and level_idc is equal to 0, there may exist a level indicated by level_idc[ i ] that is applicable to the coded video sequence operating with all the views being target output views. This may occur when a subset of views or temporal subsets are removed from a coded video sequence according to the sub-bitstream extraction process specified in subclause Error: Reference source not found, and a particular value of level_idc[ i ] corresponds to the resulting coded video sequence.

In H.10.2.2, replace the paragraph a) with the following:

- In bitstreams conforming to the Stereo High profile or MFC High profile, MVC sequence parameter sets shall have frame_mbs_only_flag equal to 1 for the levels specified in Error: Reference source not found.