| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0102 M28933 |
| --- | --- |

| Title: | AHG8 GVD (Global View and Depth) Coding using 3D-ATM 7.0 and Request |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Masayuki Tanimoto, Kazuyoshi Suzuki Yotsuya-dori 1-13, Chikusa-ku, Nagoya 464-0819, Japan Takanori Senoh Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan | Tel: Email: | +81-52-781-1883 tanimototentative3@yahoo.co.jp kazuyoshi-suzuki@cb3.so-net.ne.jp +81-42-327-7262 senoh@nict.go.jp |
| Source: | Nagoya Industry Science Research Institute(NISRI), National Institute of Information and Communications Technology(NICT) |  |  |

_____________________________

# Abstract

This contribution proposes an additional specification for 3D-AVC, which supports GVD data format. When the current ATM is applied to GVD, it needs dummy data to generate residual depth and residual view. The authors do hope ATM support one depth with no texture view (0-texture, 1-depth). It makes GVD software workable with ATM easily.

# Introduction

In the 3rd JCT-3V meeting, a signalling method of Global View and Depth (GVD) data has been adopted to the 3D-AVC as an alternative data format. The detail of description of the SEI message is presented in JCT3V-C1002 [1] and JCT3V-C0048 [2].

GVD data needs a local coding for generating residual data [3]. In order to avoid mismatch between hole mask for residual depth generation at a sender side and that at a receiver side, locally decoded depth map is used to detect the residual area in non-base views or depth maps. In the local coding process, it is sufficient that only a base depth or a residual depth is encoded and decoded by the 3D-ATM. However, the 3D-ATM 7.0 supports only N-view plus N-depth as input data.

# Problem of GVD Coding using 3D-ATM 7.0

In the process of converting multi-view plus multi-depth (MVD) data to GVD data, global depth, base depth or residual depth has to be locally encoded and decoded at global depth generator for avoiding mismatch between a residual mask for residual view or depth at a sender side and that at a receiver side.

Local coding for residual view of Type-1 GVD data is done at global depth generator that is illustrated in Fig. 1. Local coding for residual depth and residual view are done at base depth generator and residual depth generator that are illustrated in Fig2, respectively.

The 3D-ATM 7.0 supports only N-view plus N-depth input data, although it is sufficient that only the global depth, base depth or residual depth is independently encoded or decoded for residual data generation.

Type-1 GVD data needs only one local coding at the global depth generator. On the other hand, Type-2 GVD data needs local coding at both the base depth generator and the residual depth generator.

Figure 1: System configuration of Type-1 GVD format

Figure 2: System configuration of Type-2 GVD format

# Coding of Type-2 GVD with Dummy Data

A solution for solving this problem is to add two dummy views or a dummy residual depth to a base depth. Then, the 3D-ATM 7.0 encodes or decodes the obtained two-view plus two-depth data.

## Coding at base depth generator

Base depth is locally encoded and decoded in base depth generator for generating residual depth map or residual view as shown in Figure 2. A dummy residual depth and two dummy views are generated in the base depth generator. Here, the size of dummy views is not necessary to be full. The smaller the size of dummy views is, the more the encoding time of the two-view plus two-depth is short. However, it is impossible to get rid of the dummy views from the two-view plus two-depth data for encoding. Figure 3 shows an example of two-view plus two-depth data for encoding at the base depth generator. This data is encoded and decoded by using the 3D-ATM 7.0.

Figure 3: Dummy view 1, dummy view 2, base depth, dummy depth 1

## Coding at residual depth generator

Residual depth map is also locally encoded and decoded in the same way as that for the base depth. That is, a two-view plus two-depth data is encoded and decoded by using the 3D-ATM 7.0. Figure 4 shows an example of two-view plus two-depth data for the encoding at the residual depth generator.

Figure 4: Dummy view 1, dummy view 2, dummy depth 1, residual depth

# Experiment and Discussion

Figure 5 (a), (b), (c) and (d) show a base view, a residual view, base depth and a residual depth of Type-2 GVD data of Newspaper sequence, respectively.

Figure 5: Example of Type-2 GVD data for Newspaper

Figure 6 shows synthesized views at view 3.0. Figure 6 (a), (b) and (c) are a view synthesized from MVD by using the HTM view synthesizer without coding, a view synthesized from Type-2 GVD without coding, and a view synthesized from Type-2 GVD encoded/decoded by the 3D-ATM 7.0 with QP26.

The quality of view in Fig. 6(c) is visually almost same as that of anchor view in Fig. 6(a). The quality of obtained views is good.

(a) View 3.0 anchor without coding (b) View 3.0 GVD without coding (c) View 3.0 GVD with QP26

Figure 6: Example of synthesized views for Newspaper

An experiment for estimating encoding time for Type-2 GVD data has been done.

Main codec needed 2897.35 sec and 402.42 sec for texture coding of Type-2 GVD and depth coding of Type-2 GVD with QP26, respectively. The summation of them is 3299.77 sec. The time for decoding Type-2 GVD was 11.04 sec. On the other hand, the encoding time for the textures and depth maps of Newspaper 3view+3depth MVD data by the 3D-ATM 7.0 with QP26 are 5432.29 sec and 752.90 sec, respectively. The sum of them is 6185.19 sec. The decoding time was 17.34 sec. The encoding time for Type-2 GVD data is about 53% of that for MVD data with the 3D-ATM 7.0 in QP26. The amount of a Type-2 GVD data is 2/3 of that of a three-view plus three-depth MVD data.

Experimental environment of the above computation is as follows: CPU: Intel Core i7 2700k 3.50GHz; Memory: 16GB RAM (DDR3 4GB x 4); OS: Windows 7 Professional SP1; Compiler: Microsoft Visual Studio 2010 Professional; Executable: x64.

The authors have also done another experiment for comparing the encoding times for dummy data at base depth generator and residual depth generator using the 3D-ATM 7.0 with QP26 and no correlation between two views to those for only base depth and residual depth at base depth generator and residual depth generator using Join Model 18.4, respectively. Main encoding times are also presented for reference. It should be noted that the main encodings were done by only the 3D-ATM 7.0 in both cases.

(1-a) Encoding the dummy data for the base depth generator using the 3D-ATM took 3495.088 sec for texture and 3613.868 sec for depth. (1-b) That for the residual depth generator using the 3D-ATM took 3285.687 sec for texture 3295.327 sec for depth. Main encoding using the 3D-ATM 7.0 took 14557.595 sec for texture and 2363.398 sec for depth.

(2-a) On the other hand, encoding the base depth for the base depth generator using the JM 18.4 took 912.707 sec. (2-b) Encoding the base depth using the JM 18.4 took 660.941 sec. Main encoding using the 3D-ATM 7.0 took 11525.886 sec for texture and 1878.134 sec for depth.

The ratio of the total encoding time 7108.956 sec in (1-a) to that 912.707 sec in (2-a) is about 7.89 to 1. The ratio of the total encoding time 6581.014 sec in (1-b) to that 660.941 sec in (2-b) is about 9.96 to 1.

In spite of the fact that the main coding for the Type-2 GVD data generated using JM 18.4 is completely same as that for the Type-2 GVD data generated using the 3D-ATM 7.0, the former encoding time (16920.993 sec) is greatly different from the latter one (13404.02 sec). The cause is unknown.

The experimental environment of this computation is as follows: CPU: Intel Core2 Duo U9400 1.40GHz; Memory: 4GB; OS: Windows XP Professional Service Pack 3; Compiler: Microsoft Visual Studio 2010 Express; Executable: x86.

When different types of codec are used for local coding and for Type-2 GVD main coding, it is not guaranteed that hole masks for generating residual data at the sender side are completely same as those in the receiver side. From this result, the authors hope that one-depth plus zero-view coding mode is supported by the 3D-ATM.

# Conclusion

In this contribution, a procedure of coding experiments for Type-2 GVD data by using the 3D-ATM 7.0 was presented. There exists a problem that local coding of two-view plus two-depth data using the 3D-ATM 7.0 at the global depth generator, base depth generator or residual depth generator needs much time. For the purpose of solving this problem, the authors do hope that one-depth plus zero-view mode is introduced to the 3D-ATM for GVD data generation.

# References

- JCT3V-C1002: “3D-AVC Draft Text 5,” Februarys 2013.

- JCT3V-C0048 / ISO/IEC JTC1/SC29/WG11/M27783: "AHG08: Draft Text for Signaling of Alternative 3D Format in ISO/IEC 14496-10 (ITU-T H.264)," January 2013.

- JCT3V-D0071 / ISO/IEC JTC1/SC29/WG11/M28874: “AHG8 Description of Global View and Depth Format for Depth View SEI Message,” April 2013.

# Patent rights declaration(s)

NISRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).