| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0103 |
| --- | --- |

| Title: | AHG8: Draft Text for Signaling Warp Maps as an Alternative 3D Format in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nikolce Stefanoski Aljosa Smolic | Tel: Email: | +41 446325634 stefanos@disneyresearch.com |
| Source: | Disney Research Zurich |  |  |

_____________________________

# Abstract

In this document, syntax and semantics for an alternative depth format SEI message are proposed. In particular, syntax elements are presented which allow deriving warp maps from decoded depth samples. This proposal in provided in the scope of mandate 4 of AhG8 to discuss and prepare draft text for a SEI message for 3D-HEVC which signals warping maps as an alternative 3D format. It is proposed to adopt the SEI message presented in this document in the current specification of 3D-HEVC Test Model 3.

# Introduction

Image-domain warping (IDW) was introduced in M22668 during the call for proposals on 3D Video Coding Technology (N12036) as an alternative view synthesis method to depth-image-based rendering (DIBR). Instead of using depth maps, IDW uses warp maps for view synthesis. IDW-based view synthesis software as well as warp map data which is computed from original multiview video + depth data, are provided in JCT3V- A003 and JCT3V-B0074, respectively. Subjective tests conducted during the meeting showed that the provided uncoded warp maps are of sufficiently high quality for conducting coding experiments (JCT3V-B0225). In JCT3V-C0033, coding results with MV-HEVC were provided for multi-view video plus warp (MVW) data at bit rates which matched the bit rates of the 3D-HEVC anchors (multiview-video plus depth (MVD) data). Subjective tests conducted during the meeting showed that IDW-based view synthesis from coded MVW data achieves a synthesis quality comparable to DIBR from coded MVD data, when MVW and MVD data are coded at similar bit rates (JCT3V-C0240). Based on these results, AhG8 obtained the mandate to discuss and prepare draft text for a SEI message for 3D-HEVC, which signals warping maps as an alternative 3D format.

# Proposed syntax and semantics

An SEI message is proposed which signals that decoded depth samples have to be interpreted as an alternative depth format. A syntax element of this SEI message called depth_type is used to discriminate different alternative depth types. A depth type of 0 identifies warp maps as an alternative depth format. The proposed new SEI message syntax and semantics are proposed below with respect to the current specification of 3D-HEVC Test Model 3:

H.14	3D-HEVC SEI messages

The specifications in Annex F together with the extensions and modifications specified in this subclause apply.

H.14.1	SEI message syntax

H.14.1.1	Alternative depth information SEI message syntax

| alternative_depth_info ( payloadSize ) { | Descriptor |
| --- | --- |
| depth_type | u(2) |
| if( depth_type == 0 ) { |  |
| min_x_offset_int | se(v) |
| min_x_offset_frac | u(8) |
| max_x_offset_int | se(v) |
| max_x_offset_frac | u(8) |
| y_offset_present_flag | u(1) |
| if (y_offset_present_flag){ |  |
| min_y_offset_int | se(v) |
| min_y_offset_frac | u(8) |
| max_y_offset_int | se(v) |
| max_y_offset_frac | u(8) |
| } |  |
| warp_map_size_present_flag | u(1) |
| if( warp_map_size_present_flag ) { |  |
| warp_map_width | ue(v) |
| warp_map_height | ue(v) |
| } |  |
| } |  |
| } |  |

H.14.2	SEI message semantics

H.14.2.1	Alternative depth information SEI message semantics

The alternative depth information SEI message indicates that decoded depth samples have to be interpreted as an alternative depth format. To discriminate different alternative depth formats, a depth_type syntax element is used, where a depth_type of 0 indicates that decoded depth samples have to be used to derive a warp map and view synthesis has to be performed by image-domain warping. Additional syntax elements contained in this SEI message are used to derive corresponding warp maps from decoded depth samples of each view.

depth_type	identifies an alternative depth type according to the following table:

| depth_type | interpretation |
| --- | --- |
| 0 | Warp map |
| 1-3 | Reserved |

min_x_offset_int, min_x_offset_frac specify an integer and a fractional part of a min offset for the horizontal direction of a warp map. A min_x_offset value is derived as follows:

min_x_offset = min_x_offset_int + min_x_offset_frac / 256

max_x_offset_int, max_x_offset_frac specify an integer and a fractional part of a max offset for the horizontal direction of a warp map. A max_x_offset value is derived as follows:

max_x_offset = max_x_offset_int + max_x_offset_frac / 256

y_offset_present_flag equal to 1 specifies that min and max offset values for the vertical direction are present.

min_y_offset_int, min_y_offset_frac specify an integer and a fractional part of a min offset for the vertical direction of a warp map. A min_y_offset value is derived as follows:

if ( y_offset_present_flag )

min_y_offset = min_x_offset_int + min_x_offset_frac / 256

else

min_y_offset = 0

max_y_offset_int, max_y_offset_frac specify an integer and a fractional part of a max offset for the vertical direction of a warp map. A max_y_offset value is derived as follows:

if ( y_offset_present_flag )

max_y_offset = max_x_offset_int + max_x_offset_frac / 256

else

max_y_offset = 0

warp_map_size_present_flag equal to 1 specifies that a new warp map size is present, which is valid for the current and all following warp maps in decoding order until a new warp map size is again specified. warp_map_size_present_flag  equal to 0 specifies that the warp map size is not changed.

warp_map_width, warp_map_height specify width and height of a warp map, respectively. Valid values are constraint as follows:

0 <= warp_map_width <= pic_width_in_luma_samples

if ( ! y_offset_present_flag )

0 <= warp_map_height <= pic_height_in_luma_samples

else

0 <= warp_map_height <= pic_height_in_luma_samples / 2

Warp map derivation and interpretation

Decoded depth samples of each view are used to derive a corresponding warp map for that view. A warp map is derived as follows from decoded depth samples d[x][y] and syntax elements provided by this SEI message:

deltax =  pic_width_in_luma_samples / (warp_map_width - 1)

deltay =  pic_height_in_luma_samples / (warp_map_height - 1)

for (x=0;x<warp_map_width;++x)

for (y=0;y<warp_map_height;++y){

w[x][y][0] = x * deltax + min_x_offset + (max_x_offset-min_x_offset) / 255 * d[x][y]

if (y_offset_present_flag) {

w[x][y][1] =  y * deltay + min_y_offset

+ (max_y_offset-min_y_offset) / 255 * d[x][y + pic_height_in_luma_samples/2]

} else

w[x][y][1] = y * deltay

}

}

Each warp map derived by this process is assigned to its associated input view and has the property to map the input view to a central camera position as illustrated in Figure 1. Note that only the warp map of the most left input view maps the view to a central camera position on the right, while all other warps maps map their corresponding input views to a central camera position on the left.

Figure 1 – Camera positions and associated warps.

Warp map samples w[x][y][0] and w[x][y][1] are interpreted as a 2-dimensional sub-pel position in the synthesized view, to which the color samples of the input view at the sub-pel position (x*deltax, y*deltax) are mapped. A warp map also represents a mapping of color samples of quads of the input view, which are defined by the four positions (x*deltax, y*deltay), ((x+1)*deltax, y*deltay), ((x+1)*deltax, (y+1)*deltay), and (x*deltax, (y+1)*deltay), to corresponding quads in the synthesized view, which are defined by the corresponding positions (w[x][y][0], w[x][y][1]), (w[x+1][y][0], w[x+1][y][1]), (w[x+1][y+1][0], w[x+1][y+1][1]), und (w[x][y+1][0], w[x][y+1][1]).

# Conculsion

This document proposes a new SEI message for signaling warp maps as an alternative depth format. It is proposed to adopt this SEI message in the current specification of 3D-HEVC Test Model 3 (JCT3V-C1005).

# References

- N12036, “Call for Proposals on 3D Video Coding Technology“, ISO/IEC MPEG, Geneva, Switzerland, March 2011

- M22668, “Description of 3D Video Coding Technology Proposal by Disney Research Zurich and Fraunhofer HHI”, Nikolce Stefanoski, Pedro Espinosa, Oliver Wang, Manuel Lang, Aljoscha Smolic, Sebastian Bosse, Miquel Farre, Karsten Müller, Heiko Schwarz, Martin Winken, Thomas Wiegand,  ISO/IEC MPEG, Nov 2011, Geneva, Switzerland

- JCT3V-A0003, “3DV: Test data and view synthesis software for warp coding and IDW-based view synthesis experiments”, Nikolce Stefanoski, Aljoscha Smolic, JCT-3V, July 2012, Stockholm, Sweden

- JCT3V-B0074, “AHG8: New warp data for IDW-based view synthesis experiments”, Nikolce Stefanoski, Aljoscha Smolic, JCT-3V, Oct 2012, Shanghai, China

- JCT3V-B0225, “Report: Subjective testing results on comparing synthesis quality of IDWR and VSRS 1D Fast”, N. Stefanoski, JCT-3V, Oct 2012, Shanghai, China

- JCT3V-C0033, “Multiview video + warp coding with a MVC+D-like MV-HEVC-based coding system”, N. Stefanoski, A. Smolic, JCT-3V, Jan 2013, Geneva, Switzerland

- JCT3V-C0240, “Subjective testing results on comparing warp-based with depth-based synthesis from coded data at same bit-rate”, N. Stefanoski, JCT-3V, Jan 2013, Geneva, Switzerland

- JCT3V-C1005, “3D-HEVC Test Model 3”, JCT-3V, Jan 2013, Geneva, Switzerland

# Patent rights declaration(s)

Disney Research Zurich may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).