| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0104 |
| --- | --- |

| Title: | 3D-CE2.h related: Problem fix and simplification for DV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen1, Na Zhang2,3, Jian-Liang Lin1 Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 2MediaTek Beijing, North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 3 Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, na.zhang, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-HEVC, depth oriented neighboring block disparity vector (DoNBDV) is utilized to enhance the accuracy of the disparity vector (DV) derived from the neighboring block disparity vector (NBDV). During the process of NBDV, when no DV could be derived from the neighboring blocks, a zero vector will be used as a default vector. However, this zero vector is not used by DoNBDV to derive a refined DV. In this contribution, it is proposed to apply the DoNBDV after the derivation process of NBDV is completed. Based on this fix, it is also proposed to omit the checking of the temporal neighbouring blocks for the derivation of DV to simplify the process of DV derivation. Experimental results reportedly show that the fix of DoNBDV could bring 0.1% BD-rate reduction and the proposed simplification has no overall coding performance change compared to HTM-6.0, while the DV derivation process is simplified and the memory access bandwidth is also reduced.

# Problem statement

In 3D-HEVC, the disparity vector (DV) derivation is critical for the inter-view motion prediction, inter-view residual prediction, and view synthesis prediction (VSP) to indicate the correspondence between views. In HTM-6.0 [1], NBDV is used to derive a DV from the spatial or temporal neighboring blocks based on a predefined order. If no DV could be derived from the neighboring blocks, a zero vector is used as a default vector. DoNBDV [2] further utilizes the DV derived by NBDV to identify a depth block in an already coded depth view to derive a refined DV. However, when no DV could be derived from the neighboring blocks, this zero vector is not used for DoNBDV to derive a refined DV in current design.

# Solution to the problem and simplification for DoNBDV

In this contribution, we propose to apply the DoNBDV after the derivation process of NBDV is completed. To be more specific, it is proposed to use a zero vector to locate the reference depth block in the reference view to derive a refined DV when no DV can be derived from the neighboring blocks.

Based on the fix of the DoNBDV, we also propose to omit the checking of the temporal neighbouring blocks during the DV derivation to reduce the bandwidth to access the temporal collocated data.

# Experimental results

Both the fix and the proposed simplification have been integrated into HTM-6.0 [1], and the tests are conducted under the common test conditions [3]. As shown in Table1, the fix of DoNBDV brings 0.2%, 0.2% and 0.1% coding gains for video 1, video 2, and overall results, respectively. Table 2 shows the results of the proposed simplification based on the fix, as shown in Table 2, it has no overall coding performance change compared to HTM-6.0, while the DV derivation process is simplified and the memory access bandwidth is also reduced.

Table 1.  The results of bug fix of DoNBDV

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 101.2% | 99.5% | 101.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 98.5% | 100.0% |
| Newspapercc | -0.4% | -0.2% | -0.1% | -0.1% | -0.1% | 100.7% | 104.4% | 103.6% |
| GhostTownFly | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 101.4% | 99.5% | 104.2% |
| PoznanHall2 | -0.8% | -0.5% | -0.3% | -0.3% | -0.2% | 100.7% | 105.0% | 105.5% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.1% | 101.5% | 97.7% |
| UndoDancer | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% | 100.6% | 98.3% | 98.1% |
| 1024x768 | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% | 100.8% | 100.8% | 101.6% |
| 1920x1088 | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 100.9% | 101.1% | 101.4% |
| average | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 100.9% | 100.9% | 101.5% |

Table 2.  The results of the proposed simplification to DoNBDV

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 100.8% | 104.7% | 100.8% |
| Kendo | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 100.5% | 98.6% | 103.5% |
| Newspapercc | 0.1% | 0.6% | 0.1% | 0.1% | 0.0% | 100.6% | 103.5% | 102.2% |
| GhostTownFly | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% | 101.4% | 95.2% | 102.0% |
| PoznanHall2 | -0.5% | -0.6% | -0.2% | -0.2% | -0.1% | 101.3% | 96.3% | 103.8% |
| PoznanStreet | 0.4% | 0.4% | 0.1% | 0.1% | 0.1% | 100.9% | 105.7% | 99.9% |
| UndoDancer | -0.4% | -0.1% | -0.1% | -0.1% | -0.2% | 100.6% | 98.8% | 95.9% |
| 1024x768 | 0.0% | 0.3% | 0.1% | 0.1% | 0.0% | 100.7% | 102.3% | 102.2% |
| 1920x1088 | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 101.0% | 99.0% | 100.4% |
| average | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.9% | 100.4% | 101.2% |

# Conclusion

This contribution proposes to make a fix for DoNBDV by using a zero vector to locate the reference depth block in the reference view to derive a refined DV while no DV can be derived by NBDV. It is reported that the fix can bring 0.1% BD-rate reduction. We also propose to simplify DV derivation process by omitting the checking of temporal neighbouring blocks based on the bug fix. Experimental results show that the proposed simplification has no overall coding change compared to HTM-6.0, while the DV derivation process is simplified and the memory access bandwidth is also reduced.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] HTM-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0

[2] Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

[3] D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.