- H.8.5.4 Derivation process for a disparity vector

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a flag deriveFromDepthFlag specifying whether the disparity vector is derived form a depth component

- Outputs of this process are:

- a view order index refViewIdx specifying a reference view.

- a flag availableDV specifying whether the disparity vector is available,

- a disparity vector mvDisp.

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows.

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - PredMode[ xC ][ yC ] is equal to MODE_SKIP

  - PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

//this part is only disabled when DoNBDV is enabled

The derivation process for a disparity vector from temporal neighbour block as specified in Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH as inputs, and flag availableDV, and disparity vector mvDisp outputs.

When availableDV is equal to 0, for each N being A1, B1, B0, A0, or B2, and ( xN, yN ) being ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW − 1,  yP − 1 ), ( xP + nPSW,  yP − 1 ), ( xP − 1,  yP + nPSH  ), or ( xP − 1,  yP − 1 ) the following ordered steps apply.

  - When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

        - xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(Error: Reference source not found-1)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(Error: Reference source not found-2)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(Error: Reference source not found-3)

  - The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the partition index partIdx, and the luma location ( xN, yN ) as inputs and the output is assigned to the prediction block availability flag availableN.

  - When all of the following conditions are true, availableA1 is set equal to 0.

    - N is equal to A1

    - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N

    - partIdx is equal to 1

  - When all of the following conditions are true, availableB1 is set equal to 0.

    - N is equal to B1

    - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD

    - partIdx is equal to 1

  - When all of the following conditions are true, availableN is set equal to 0.

    - checkParallelMergeFlag is equal to 1

    - (xP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to
(xN >> (log2_parallel_merge_level_minus2 + 2) )

    - (yP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2) ).

  - The flag availableIvpMvSearchFlagN is set equal to availableN.

  - When one of the following conditions is true, availableIvpMvSearchFlagN is set equal to 0.

    - N is equal to B0 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

    - N is equal to B1 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

    - N is equal to B2 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

  - The flags availableFlagIvpMvN0 and availableFlagIvpMvN1 are set equal to 0.

  - For each X from 0 to 1, the following applies:

    - When availableDV is equal to 0, availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefIdxLX[ xN ][ yN ] is greater than or equal to 0, the following applies:

        - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

            - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(Error: Reference source not found-4)
mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-5)
availableDV = 1		(Error: Reference source not found-6)

        - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

          - When availableIvpMvSearchFlagN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

              - ivpMvDispLXN = IvpMvDispLX[ xN ][ yN ]	(Error: Reference source not found-7)
availableFlagIvpMvLXN = 1		(Error: Reference source not found-8)

      - Otherwise ( RefIdxLX[ xN ][ yN ] is less than 0 ), the following applies:

        - When VspModeFlag[ xN ][ yN ] is equal to 1, the following applies:

            - refViewIdx = RefViewIdx[ xN ][ yN ]	(Error: Reference source not found-9)
mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-10)
mvDisp[ 1 ] = 0		(Error: Reference source not found-11)
availableDV = 1		(Error: Reference source not found-12)

When availableDV is equal to 0 for each N being A1, B1, B0, A0, and B2, the following applies.

  - For each X from 0 to 1, the following applies.

    - When availableDV is equal to 0 and availableFlagIvpMvLXN is equal to 1, the following applies:

        - mvDisp = ivpMvDispLXN		(Error: Reference source not found-13)
availableDV = 1		(Error: Reference source not found-14)

When availableDV is equal to 0, the flag availableDV is set equal to1, and both components of the disparity vector mvDisp are set equal to 0.

When availableDV is equal to 1 and deriveFromDepthFlag is equal to 1, the following ordered steps apply:

  - The derivation process for disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xP, yP, the disparity vector mvDisp, the view identifier refViewIdx, the variables nPSW, nPSH, the variable nSubBlkW equal to nPSW, the variable nSubBlkH equal to nPSH, and the flag restMaxSearchFlag being equal to 1 as the inputs, and the output is the array disparitySamples of size (nPSWDs)x(nPSHDs).

  - The horizontal component of the disparity vector mvDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].