| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0106 |
| --- | --- |

| Title: | 3D-CE5.h related: On complexity reduction of bi-prediction for illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Tzu-Der Chuang, Jian-Liang Lin Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, peter.chuang, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-HEVC reference software, HTM-6.0, when the two pieces of motion information used for bi-prediction refer a same reference block, encoder/decoder non-normatively changes the inter prediction direction from bi-prediction to uni-prediction to avoid the unnecessary computation caused by the redundant motion compensation process. However, with this non-normative change, HTM-6.0 could generate a bitstream which does not conform to the 3D-HEVC text when illumination compensation (IC) is enabled due to the unequal clipping operations between uni-prediction and bi-prediction. To resolve this issue, in this contribution, it is proposed to normatively align the operations between uni-prediction and bi-prediction when IC is enabled. Experimental results reportedly show that the proposed method causes no coding efficiency loss while the unnecessary computation of bi-prediction can be removed regardless of whether IC is enabled.

# Introduction

Illumination compensation (IC) is a technique to reduce the sample prediction errors caused by the different light fields of two views captured by different cameras at different locations. In HTM-6.0 [1], a linear IC model is used to compensate the illumination discrepancy between different views. The linear IC model is shown in Equ. (1), where p(i, j) is the predicting sample at location (i, j), (dx, dy) is the disparity vector, p’(i, j) is the reference sample from neighboring view, and a and b are parameters of the linear IC model.

p(i,j)=a⋅p'i+dx,j+dy+b                                (1)

In current 3D-HEVC, when the inter-prediction is uni-prediction, the clipping is firstly applied to the interpolated samples output by the DCT based interpolation filter (DCTIF) to clip the sample value within a valid range. The clipped samples are then applied with the linear IC model to perform illumination compensation. After the IC operations, the sample values are again clipped within a valid range and are output as predictors.

For the bi-prediction cases, the interpolated samples of each list (L0 and L1) are not clipped but applied the linear IC model directly. The illumination compensated samples for each list are then averaged and clipped within a valid range.

When motion information of list 0 is the same as that of list 1, i.e., two motion vectors point to a same reference block, an HEVC encoder or decoder can non-normatively change the inter prediction direction from bi-prediction to uni-prediction in order to avoid the unnecessary computation caused by the redundant motion compensation process. However, this non-normative flexibility cannot be supported in 3D-HEVC when IC is enabled due to the unequal clipping operations between uni-prediction and bi-prediction.

# Proposed method

We propose to align the operations between the uni-prediction and bi-prediction when IC is enabled. Specifically, for both uni-prediction and bi-prediction, the interpolated samples output by the interpolation filter are applied with the clipping operation. Compared to the bi-prediction in the original design, the dynamic range of the samples output by the interpolation filter is also reduced. The clipped samples are then applied with IC operations. For the uni-prediction, the illumination compensated samples are again applied with the clipping operation as the output. For the bi-prediction, the illumination compensated samples are averaged and applied with the clipping operation as the output. The modifications to the draft text are illustrated in Table 1, and the modifications are highlighted.

Table 1.  The modified text of predictor calculation for illumination compensation

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

    - clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 ) (H-1)

    - predSamples[ x ][ y ] = !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 )  (H-2)

- Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

    - clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 ) (H-3)

    - predSamples[ x ][ y ] = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 )   (H-4)

- Otherwise,

    - predVal0 = !puIcFlagL0 ? predSamplesL0[ x ][ y ] : 
( ( predSamplesL0[ x ][ y ] * icWeightL0 ) >> icShiftL0  ) + ( icOffsetL0 << shift1 ) )   (Error: Reference source not found-5)

    - clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 )  (H-6)

    - predVal0= !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 )   (H-7)

    - predVal1 = !puIcFlagL1 ? predSamplesL1[ x ][ y ] : 
( ( predSamplesL1[ x ][ y ] * icWeightL1 ) >> icShiftL1 ) + ( icOffsetL1 << shift1 ) )   (Error: Reference source not found-8)

    - clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 )  (H-9)

    - predVal1 = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 ) H-10)

- predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, ( predVal0 + predVal1 + offset2 ) >> shift2 ) (H-11)

# Experimental results

The proposed scheme is integrated into HTM-6.0 [1], and the tests are conducted under the common test conditions [2]. As shown in Table 2, the proposed scheme introduces no coding efficiency loss for overall results.

Table 2.  The results of bug fix of DoNBDV

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.6% | 94.3% | 99.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 96.5% | 99.9% |
| Newspapercc | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 102.0% | 101.1% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 99.2% | 103.5% |
| PoznanHall2 | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% | 100.7% | 93.1% | 99.4% |
| PoznanStreet | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 102.1% | 99.4% |
| UndoDancer | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.5% | 105.6% | 96.0% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.6% | 97.6% | 100.2% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.5% | 100.0% | 99.5% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 98.9% | 99.8% |

# Conclusion

In this contribution, a scheme is proposed for illumination compensation to avoid unnecessary computation of bi-prediction when the two lists share the same motion information. The experimental results show no coding efficiency loss is introduced by the proposed scheme.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] HTM-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0

[2]D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.