J.8.3.1.8.1 Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	the location of a top-left sample (mbx1, mby1 ) of a current macroblock,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- If reduced_resolution_flag is equal to 1, the variable partHeight and partWidth is derived as follows.

partHeight = 7

partWidth = 7

mbx1= mbx1>>1

mby1= mby1>>1

Otherwise, the variable partHeight and partWidth is derived as follows.

partHeight = 15

partWidth = 15

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j += ( partHeight – 1 ) )
	for( i = 0; i < partWidth; i += ( partWidth – 1 ) )		(J-8-XX)
		if( depthPic[mbx1 + i, mby1 + j ] > maxDepth ) 
			maxDepth = depthPic[mbx1 + i, mby1 + j ]

- The variable mv is specified as follows:

srcIndex = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

mv[ 0 ] = Disparity( maxDepth, srcIndex, refIndex) 
mv[ 1 ] = 0

J.8.3.1.9.1 Derivation process for the disparity vector and the inter-view reference

Inputs to this process are depth reference view component depthPic, the location of a top-left sample ( mbx1, mby1 ) of a current macroblock and the listSuffixFlag.

Outputs of this process are a picture InterViewPic, an offset vector dv and a variable InterViewAvailable

The variable InterViewAvailable is set equal to 0.

The following applies to derive an inter-view reference picture or inter-view only reference picture, InterViewPic, with X set to 1 when listFuffixFlag is 1 or 0 otherwise:

for( cIdx = 0;cIdx<num_ref_idx_l0_active_minus1 + 1 && !InterViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]
		InterViewAvailable = 1
	}

When InterViewAvailable is equal to 1, the following steps apply in order.

- If reduced_resolution_flag is equal to 1, the variable partHeight and partWidth is derived as follows.

partHeight = 7

partWidth = 7

mbx1= mbx1>>1

mby1= mby1>>1

Otherwise, the variable partHeight and partWidth is derived as follows.

partHeight = 15

partWidth = 15

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j+=(partHeight-1) )
	for( i = 0; i < partWidth; i+=(partWidth-1))
		if( depthPic[mbx1 + i, mby1 + j ] > maxDepth ) maxDepth = depthPic[mbx1 + i, mby1 + j ]

- The variable dv is specified as follows:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dv[ 0 ] = Disparity(maxDepth, currIndex, refIndex )
dv[ 1 ] = 0