| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0108 |
| --- | --- |

| Title: | 3D-CE2.a related: Simplified disparity vector derivation in Skip mode and Direct mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-AVC, the disparity vector used for the inter-view MVP derivation in Skip mode and Direct mode is derived from the motion vectors of the neighboring blocks and from the disparity vector converted from the maximum of four corner depth samples in the associated depth block. This contribution proposes to simplify the disparity vector derivation by directly deriving the disparity from the maximum of four corner depth samples. The experimental results reportedly show that this proposed simplification brings no coding efficiency loss compared to ATM-7.0 while the complexity of the inter-view candidate derivation and memory access for the motion data of the neighboring blocks are reduced.

# Introduction

In 3D-AVC, the derivation process of the inter-view MVP in Skip mode and Direct mode is shown in Figure 1 [1]. To derive the disparity vector (DV) for locating a corresponding block in the inter-view MVP derivation, a derivation method using neighboring blocks is applied. The disparity vector is derived from the motion vectors of the neighboring blocks and from the depth samples in the associated depth block of current texture block as shown in Figure 2.

Specifically, the disparity vector is derived from the list 0 or list 1 motion vectors of neighboring blocks A, B, and C (D). If only one of the neighboring blocks was coded with inter-view prediction, its motion vector is interpreted as the disparity vector utilized in the inter-view MVP derivation. If more than one inter-view prediction vectors are available in block A, B, C (D), the disparity vector is derived as the median of the three neighboring blocks and any one of the neighboring blocks which was not coded with inter-view prediction is replaced by the disparity vector converted from the maximum of four corner depth samples in the associated depth block [2].

Figure 1.  The inter-view candidate in Skip mode and Direct mode

Figure 2.  The disparity vector derivation procedure of inter-view MVP [2]

# Simplification for disparity vector derivation

To simplify the disparity vector derivation process in the inter-view MVP, we propose to directly drive the disparity vector from the depth samples instead of from both the motion vectors of the neighboring blocks and the depth samples in the associated depth block. Figure 3 shows the simplified disparity vector derivation process in which the disparity vector is directly converted from the maximum of four corner depth samples in the associated depth block.

Figure 3.  The simplified disparity vector derivation procedure of inter-view MVP

# Experimental results

The proposed simplification is integrated into ATM-7.0 [3], and the simulations are run under the common test conditions [4]. Table 1 shows the results of the simplified disparity vector derivation procedure for the inter-view MVP in Skip mode and Direct mode. The experimental results show that this proposed simplification brings no coding efficiency loss while the complexity of the inter-view candidate derivation and memory access bandwidth for the motion data access of the neighboring block are reduced.

Table 1. Results of the simplified disparity vector derivation

# Conclusion

This contribution proposed a simplification of the disparity vector derivation process in Skip mode and Direct mode. It is proposed to directly derive the disparity vector from the depth samples instead of from both the motion vectors of the neighboring blocks and the depth samples in the associated depth block. The results show that the proposed simplification brings no coding efficiency loss while the complexity is reduced and the memory access of neighboring blocks’ motion information is avoided in the derivation process of the inter-view MVP in Skip mode and Direct mode.

# Patent rights declaration(s)

MediaTek Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “3D-CE2.a results on simplified inter-view candidate derivation,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0133, January, 2013.

[2] D. Rusanovskyy, F.-C. Chen, L. Zhang, T. Suzuki, “3D-AVC test model 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1003, January, 2013.

[3] ATM-7.0, http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv7.0/

[4] D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.