| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0109 |
| --- | --- |

| Title: | 3D-CE5.h related: Additional inter-view merging candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an additional inter-view merging candidate is proposed to be added in the merging candidate list for the texture coding in a dependent view. For the pruning process, the additional inter-view candidate is only compared with the original inter-view candidate for redundancy removal. The simulation results reportedly show that the proposed additional inter-view merging candidate could achieve 0.2%, 0.1% and 0.1% BD-rate savings on average for video1, video2, and overall results, respectively, compared to HTM-6.0 while the encoding and decoding time are almost unchanged.

# Introduction

In current 3D-HEVC [1], the inter-view merging candidate is inserted into the first position of the merging candidate list for texture coding. As listed in the left part of Table 1, the merging candidate list includes four spatial merging candidates, one inter-view candidate, one disparity candidate, one VSP candidate, and one temporal merging candidate with the size fixed to 6. The positions of those merging candidates are shown in Figure 1.

Table 1.  The original and proposed merging candidate lists

| Original merging candidate list | Proposed merging candidate list |
| --- | --- |
| - Inter-view candidate | - Inter-view candidate |
| - A1 | - A1 |
| - B1 | - B1 |
| - B0 | - B0 |
| - A0 | - A0 |
| - Disparity candidate (DV) | - Disparity candidate (DV) |
| - B2 | - B2 |
| - VSP candidate | - VSP candidate |
| - Temporal candidate | - Additional inter-view candidate |
|  | - Temporal candidate |

# Proposed method

In this contribution, we propose to add an additional inter-view candidate to improve the coding efficiency. As shown in the right part of Table 1, this additional inter-view candidate is inserted into the position before the temporal candidate in the merging candidate list and the size of merge candidate set is still fixed to 6. The additional inter-view candidate is derived from a corresponding block which is located as the H position shifted by a disparity vector as shown in Figure 1. The disparity vector is the same as the one used in the derivation of the original inter-view candidate in the merging candidate list. As shown in Figure 2, in the pruning process, the additional inter-view candidate is only compared to the first inter-view candidate.

Figure 1. The additional inter-view candidate (I2).

Figure 2. The pruning process of the additional inter-view candidate (I2). Arrows indicate which merging candidates are to be compared.

# Experimental results

The experiments were conducted according to the common test conditions defined in [2], and the software is based on HTM-6.0 [3]. As shown in Table 2, the proposed additional inter-view candidate can achieve 0.2%, 0.1%, and 0.1% BD-rate savings on average for video1, video2 and overall results, respectively. The encoding time and the decoding time are roughly unchanged.

Table 2.  The results of the proposed scheme

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | 98.3% | 102.7% | 94.7% |
| Kendo | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | 98.5% | 100.0% | 99.2% |
| Newspapercc | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 98.7% | 100.6% | 100.9% |
| GhostTownFly | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.1% | 101.6% | 98.7% |
| PoznanHall2 | -0.6% | -0.1% | -0.2% | -0.2% | -0.1% | 100.3% | 99.3% | 102.2% |
| PoznanStreet | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.2% | 99.7% | 96.9% |
| UndoDancer | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | 99.3% | 101.4% | 98.8% |
| 1024x768 | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 98.5% | 101.1% | 98.3% |
| 1920x1088 | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 99.5% | 100.5% | 99.1% |
| average | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 99.1% | 100.8% | 98.8% |

# Conclusion

In this contribution, an additional inter-view candidate is proposed in the merging candidate list for texture coding. The redundancy checking of this additional candidate is only compared with the original inter-view candidate to save the computational complexity. The average coding gain of the proposed scheme is around 0.2%, 0.1% and 0.1% for video1, video2 and overall results, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, and S. Yea, “Draft of 3D-HEVC test model description,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1005, January, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.

- HTM-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0