| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0111 M27831 |
| --- | --- |

| Title: | JCT3V – The rate control schemes for 3D multi-view video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document Approved by the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Lim, Hyunho Jo, Jonghoon Yoo, Donggyu Sim 512 Bima-building, Kwangwoon Univ. Wolgye-dong, Seoul, Korea. Ivan V. Bajić Applied sciences 9823, Simon Fraser Univ. 8888 University dr., Burnaby, BC, Canada | Tel: Email: | +82-10-2744-6098 limwoong@kw.ac.kr idjhh@kw.ac.kr mmyjh@kw.ac.kr dgsim@kw.ac.kr +1-604-612-8115 ibajic@ensc.sfu.ca |
| Source: | Kwangwoon University (KWU) and Simon Fraser University (SFU) |  |  |

_____________________________

# Abstract

A rate control scheme is an essential part of the practical video encoder for services such as VoD (Video on Demand), video conference, etc. The latest reference software of HEVC, HM-10.0, includes two rate control schemes based on URQ (Unified Rate Quantization) and R-lambda model [1-3]. In contrast, the test model for multi-view video coding based on HEVC does not include a rate control scheme. As the base view of HEVC-based multi-view video coding is compatible to HEVC, the rate control schemes in HM-10.0 can be used for both the base view and extended views immediately. However, a rate control scheme for the extended views can be improved rather than the rate control schemes in HM-10.0 by using the inter-view correlation. Before thorough analysis about a rate control scheme for the extended views, the former rate control schemes in HM-10.0 should be implemented for the performance evaluation. In this proposal, we implemented the current rate control schemes in HM-10.0 on HTM6.1 to extend those single-view rate control scheme to multi-view and compared the performance of the rate control schemes for each view to the anchor (HTM6.1 under CTC) under the CBR (Constant Bit Rate) condition.

# Introduction

URQ model-based rate control scheme, described in JCTVC-H0213 [1], was adopted in the HM reference software at the 8th San Jose meeting in February 2012. Some improvements were reported in JCTVC-I0094 [2]. At the meeting in Shanghai, China, another rate control scheme based on R-lambda model was newly adopted in the HM reference software for single-view coding [3]. We implemented both these two rate control schemes from HM-10.1 on HTM6.1 for HEVC-based multi-view extension.

# Experimental results

We implemented both URQ model and R-lambda model-based rate control schemes not only for the base view but also for the extended views. For the extended views, the rate control schemes are same as for the based view before improvement for the first step. The implemented two rate control schemes for multi-view video coding were evaluated under the CBR condition. In table 1, the target bitrates for each view is from the encoding results of the encoder without a rate control scheme under CTC configuration. Table 1 shows the average percentage error in the actual generated bitrate compared to the target bitrate of URQ model-based rate control scheme.

Table 1. Percentage error in the actual bitrate (%) of URQ model-based rate control

| Sequences | Target bitrates (kbps) |  |  | Actual generated Bitrates (kbps) |  |  | Δ kbps accuracy for each View (%) |  |  | Average of total Δ kbps accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | view0 | view1 | view2 | view0 | view1 | view2 | view0 | view1 | view2 |  |
| Balloons | 876.00 | 301.00 | 330.00 | 889.27 | 306.15 | 335.51 | 1.51 | 1.71 | 1.67 | 1.59 |
|  | 481.00 | 146.00 | 159.00 | 488.42 | 149.29 | 161.69 | 1.54 | 2.25 | 1.69 | 1.70 |
|  | 278.00 | 79.00 | 87.00 | 282.32 | 80.55 | 88.65 | 1.55 | 1.96 | 1.90 | 1.69 |
|  | 168.00 | 45.00 | 50.00 | 170.66 | 45.68 | 50.97 | 1.58 | 1.52 | 1.93 | 1.64 |
| Kendo | 844.00 | 299.00 | 319.00 | 856.59 | 303.02 | 324.33 | 1.49 | 1.34 | 1.67 | 1.50 |
|  | 451.00 | 141.00 | 149.00 | 458.35 | 143.38 | 151.57 | 1.63 | 1.69 | 1.73 | 1.66 |
|  | 257.00 | 76.00 | 80.00 | 261.42 | 77.49 | 81.61 | 1.72 | 1.97 | 2.01 | 1.82 |
|  | 155.00 | 44.00 | 46.00 | 158.03 | 44.99 | 46.98 | 1.96 | 2.25 | 2.13 | 2.04 |
| Newspaper | 950.00 | 325.00 | 369.00 | 961.22 | 329.18 | 373.27 | 1.18 | 1.29 | 1.16 | 1.20 |
|  | 500.00 | 145.00 | 166.00 | 505.86 | 143.87 | 164.66 | 1.17 | 0.78 | 0.81 | 0.42 |
|  | 276.00 | 74.00 | 86.00 | 279.35 | 74.56 | 86.60 | 1.21 | 0.75 | 0.70 | 1.03 |
|  | 160.00 | 42.00 | 48.00 | 161.33 | 42.04 | 48.15 | 0.83 | 0.09 | 0.32 | 0.61 |
| Gtfly | 3667.00 | 623.00 | 645.00 | 3740.33 | 633.42 | 654.96 | 2.00 | 1.67 | 1.54 | 1.90 |
|  | 1611.00 | 233.00 | 242.00 | 1642.53 | 252.50 | 245.95 | 1.96 | 8.37 | 1.63 | 2.64 |
|  | 755.00 | 106.00 | 111.00 | 770.60 | 106.89 | 111.56 | 2.07 | 0.84 | 0.50 | 1.75 |
|  | 366.00 | 51.00 | 54.00 | 372.15 | 52.45 | 55.66 | 1.68 | 2.84 | 3.07 | 1.97 |
| PoznanHall2 | 809.00 | 268.00 | 279.00 | 819.67 | 270.75 | 281.78 | 1.32 | 1.03 | 1.00 | 1.19 |
|  | 344.00 | 110.00 | 112.00 | 350.95 | 113.92 | 121.67 | 2.02 | 3.56 | 8.63 | 3.63 |
|  | 177.00 | 58.00 | 59.00 | 180.57 | 58.87 | 59.29 | 2.02 | 1.51 | 0.50 | 1.61 |
|  | 100.00 | 32.00 | 34.00 | 101.71 | 33.21 | 33.80 | 1.71 | 3.78 | 0.60 | 1.63 |
| PoznanStreet | 2427.00 | 753.00 | 739.00 | 2472.68 | 768.33 | 753.94 | 1.88 | 2.04 | 2.02 | 1.94 |
|  | 956.00 | 224.00 | 220.00 | 975.61 | 229.34 | 225.54 | 2.05 | 2.38 | 2.52 | 2.18 |
|  | 461.00 | 92.00 | 93.00 | 471.18 | 98.06 | 93.74 | 2.21 | 6.59 | 0.79 | 2.63 |
|  | 240.00 | 43.00 | 42.00 | 245.79 | 45.01 | 43.35 | 2.41 | 4.68 | 3.21 | 2.82 |
| Dancer | 4594.00 | 992.00 | 938.00 | 4703.63 | 1012.74 | 955.89 | 2.39 | 2.09 | 1.91 | 2.27 |
|  | 1995.00 | 378.00 | 357.00 | 2029.35 | 386.19 | 364.28 | 1.72 | 2.17 | 2.04 | 1.82 |
|  | 917.00 | 172.00 | 163.00 | 933.43 | 179.51 | 166.13 | 1.79 | 4.37 | 1.92 | 2.16 |
|  | 429.00 | 82.00 | 79.00 | 437.58 | 84.20 | 80.83 | 2.00 | 2.68 | 2.32 | 2.14 |
|  |  |  |  |  |  | Avg. | 1.74 | 2.44 | 1.85 | 1.83 |

As shown in table 1, the average bitrate accuracy of URQ model-based rate control scheme for 3 views is 1.83%.

Table 2. PSNR (dB) degradation for all views of URQ model-based rate control

| Sequences | PSNR without rate control (dB) |  |  | PSNR with the URQ model-based rate control (dB) |  |  | View-wise Δ PSNR |  |  | Average of Δ PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | view0 | view1 | view2 | view0 | view1 | view2 | view0 | view1 | view2 |  |
| Balloons | 43.10 | 42.31 | 41.97 | 42.03 | 40.88 | 40.66 | -1.07 | -1.42 | -1.31 | -1.27 |
|  | 41.07 | 40.02 | 39.77 | 39.35 | 38.30 | 37.91 | -1.72 | -1.72 | -1.86 | -1.77 |
|  | 38.51 | 37.44 | 37.22 | 36.29 | 35.53 | 35.40 | -2.22 | -1.91 | -1.82 | -1.98 |
|  | 35.68 | 34.70 | 34.51 | 33.14 | 32.49 | 32.38 | -2.54 | -2.22 | -2.13 | -2.29 |
| Kendo | 43.61 | 42.77 | 42.39 | 41.70 | 39.99 | 40.41 | -1.92 | -2.78 | -1.98 | -2.22 |
|  | 41.46 | 40.46 | 40.19 | 37.86 | 37.04 | 37.39 | -3.61 | -3.43 | -2.81 | -3.28 |
|  | 38.99 | 38.05 | 37.85 | 35.90 | 35.44 | 35.31 | -3.09 | -2.60 | -2.54 | -2.74 |
|  | 36.38 | 35.53 | 35.37 | 34.06 | 33.56 | 33.38 | -2.32 | -1.97 | -1.99 | -2.09 |
| Newspaper | 41.35 | 40.32 | 39.91 | 39.34 | 37.92 | 37.48 | -2.02 | -2.40 | -2.43 | -2.28 |
|  | 39.00 | 38.00 | 37.55 | 36.51 | 35.36 | 34.79 | -2.49 | -2.64 | -2.77 | -2.63 |
|  | 36.46 | 35.59 | 35.12 | 33.75 | 33.22 | 32.78 | -2.71 | -2.38 | -2.34 | -2.48 |
|  | 33.91 | 33.19 | 32.62 | 31.59 | 31.19 | 30.56 | -2.32 | -1.99 | -2.05 | -2.12 |
| Gtfly | 40.19 | 39.68 | 39.64 | 39.59 | 38.92 | 38.90 | -0.61 | -0.75 | -0.74 | -0.70 |
|  | 37.78 | 37.43 | 37.39 | 36.58 | 36.23 | 36.18 | -1.20 | -1.20 | -1.21 | -1.20 |
|  | 35.60 | 35.36 | 35.31 | 34.35 | 34.14 | 34.07 | -1.25 | -1.22 | -1.24 | -1.24 |
|  | 33.46 | 33.30 | 33.25 | 32.32 | 32.23 | 32.17 | -1.15 | -1.08 | -1.08 | -1.10 |
| PoznanHall2 | 42.13 | 41.87 | 41.72 | 41.14 | 41.19 | 41.07 | -0.99 | -0.67 | -0.64 | -0.77 |
|  | 41.15 | 40.87 | 40.78 | 40.54 | 40.33 | 40.24 | -0.61 | -0.54 | -0.54 | -0.56 |
|  | 39.76 | 39.47 | 39.44 | 39.53 | 39.25 | 39.00 | -0.23 | -0.22 | -0.43 | -0.29 |
|  | 38.05 | 37.73 | 37.73 | 37.74 | 37.50 | 37.31 | -0.30 | -0.23 | -0.42 | -0.32 |
| PoznanStreet | 39.86 | 38.99 | 38.93 | 38.59 | 37.28 | 37.07 | -1.27 | -1.71 | -1.86 | -1.61 |
|  | 37.91 | 37.15 | 36.96 | 35.55 | 35.37 | 34.82 | -2.37 | -1.78 | -2.15 | -2.10 |
|  | 35.90 | 35.28 | 35.00 | 32.79 | 32.96 | 32.64 | -3.11 | -2.32 | -2.36 | -2.60 |
|  | 33.78 | 33.32 | 33.01 | 30.88 | 30.90 | 30.59 | -2.90 | -2.41 | -2.42 | -2.58 |
| Dancer | 38.75 | 37.95 | 38.13 | 38.15 | 37.15 | 37.33 | -0.60 | -0.80 | -0.79 | -0.73 |
|  | 35.89 | 35.34 | 35.54 | 33.95 | 33.39 | 33.61 | -1.94 | -1.95 | -1.93 | -1.94 |
|  | 33.37 | 32.95 | 33.16 | 31.43 | 31.06 | 31.19 | -1.95 | -1.89 | -1.96 | -1.94 |
|  | 31.10 | 30.76 | 30.94 | 29.48 | 29.26 | 29.37 | -1.62 | -1.50 | -1.57 | -1.57 |
|  |  |  |  |  |  | Avg | -1.79 | -1.70 | -1.69 | -1.73 |

Table 2 shows PSNR degradation for each view and the average PSNR degradation for all views is 1.73dB using URQ model-based rate control scheme. Table 3 shows the bitrate accuracy for each view using R-lambda model-based rate control.

Table 3. Percentage error in the actual bitrate (%) of R-lambda model based rate control

| sequence | Target bitrates (kbps) |  |  |  | Actual generated Bitrates (kbps) |  |  | Δ kbps accuracy for each View (%) |  |  | Average of total Δ kbps accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | view0 | view1 |  | view2 | view0 | view1 | view2 | view0 | view1 | view2 |  |
| Balloons | 876.00 | 301.00 |  | 330.00 | 889.12 | 311.21 | 340.77 | 1.50 | 3.39 | 3.26 | 2.26 |
|  | 481.00 | 146.00 |  | 159.00 | 487.74 | 151.71 | 164.48 | 1.40 | 3.91 | 3.44 | 2.28 |
|  | 278.00 | 79.00 |  | 87.00 | 284.14 | 82.26 | 90.62 | 2.21 | 4.13 | 4.16 | 2.93 |
|  | 168.00 | 45.00 |  | 50.00 | 173.12 | 46.88 | 52.13 | 3.05 | 4.18 | 4.27 | 3.47 |
| Kendo | 844.00 | 299.00 |  | 319.00 | 853.57 | 305.74 | 325.54 | 1.13 | 2.25 | 2.05 | 1.56 |
|  | 451.00 | 141.00 |  | 149.00 | 455.44 | 144.80 | 152.70 | 0.98 | 2.69 | 2.49 | 1.61 |
|  | 257.00 | 76.00 |  | 80.00 | 259.74 | 78.48 | 82.22 | 1.07 | 3.27 | 2.78 | 1.80 |
|  | 155.00 | 44.00 |  | 46.00 | 157.69 | 46.00 | 47.80 | 1.73 | 4.56 | 3.91 | 2.65 |
| Newspaper | 950.00 | 325.00 |  | 369.00 | 960.62 | 334.04 | 378.40 | 1.12 | 2.78 | 2.55 | 1.77 |
|  | 500.00 | 145.00 |  | 166.00 | 508.08 | 149.99 | 171.00 | 1.62 | 3.44 | 3.01 | 2.23 |
|  | 276.00 | 74.00 |  | 86.00 | 281.83 | 77.01 | 89.01 | 2.11 | 4.06 | 3.50 | 2.72 |
|  | 160.00 | 42.00 |  | 48.00 | 165.25 | 43.90 | 49.93 | 3.28 | 4.52 | 4.01 | 3.63 |
| Gtfly | 3667.00 | 623.00 |  | 645.00 | 3713.76 | 632.20 | 654.63 | 1.28 | 1.48 | 1.49 | 1.33 |
|  | 1611.00 | 233.00 |  | 242.00 | 1639.37 | 237.29 | 246.79 | 1.76 | 1.84 | 1.98 | 1.79 |
|  | 755.00 | 106.00 |  | 111.00 | 774.87 | 112.06 | 117.56 | 2.63 | 5.72 | 5.91 | 3.34 |
|  | 366.00 | 51.00 |  | 54.00 | 384.76 | 56.05 | 60.00 | 5.13 | 9.91 | 11.11 | 6.33 |
| PoznanHall2 | 809.00 | 268.00 |  | 279.00 | 829.83 | 273.86 | 286.10 | 2.57 | 2.19 | 2.54 | 2.49 |
|  | 344.00 | 110.00 |  | 112.00 | 355.46 | 111.00 | 111.62 | 3.33 | 0.91 | 0.34 | 2.13 |
|  | 177.00 | 58.00 |  | 59.00 | 182.39 | 59.14 | 60.66 | 3.05 | 1.96 | 2.81 | 2.78 |
|  | 100.00 | 32.00 |  | 34.00 | 103.67 | 33.63 | 35.96 | 3.67 | 5.10 | 5.76 | 4.37 |
| PoznanStreet | 2427.00 | 753.00 |  | 739.00 | 2486.96 | 779.09 | 763.05 | 2.47 | 3.46 | 3.25 | 2.81 |
|  | 956.00 | 224.00 |  | 220.00 | 995.24 | 232.19 | 229.16 | 4.10 | 3.66 | 4.16 | 4.04 |
|  | 461.00 | 92.00 |  | 93.00 | 489.26 | 96.95 | 98.00 | 6.13 | 5.38 | 5.38 | 5.92 |
|  | 240.00 | 43.00 |  | 42.00 | 257.45 | 47.76 | 47.19 | 7.27 | 11.08 | 12.36 | 8.43 |
| Dancer | 4594.00 | 992.00 |  | 938.00 | 4617.89 | 1008.35 | 952.88 | 0.52 | 1.65 | 1.59 | 0.84 |
|  | 1995.00 | 378.00 |  | 357.00 | 2030.98 | 385.47 | 364.14 | 1.80 | 1.98 | 2.00 | 1.85 |
|  | 917.00 | 172.00 |  | 163.00 | 933.60 | 176.13 | 167.16 | 1.81 | 2.40 | 2.55 | 1.99 |
|  | 429.00 | 82.00 |  | 79.00 | 445.85 | 84.84 | 82.56 | 3.93 | 3.46 | 4.50 | 3.94 |
|  |  |  |  |  |  |  | Avg | 2.59 | 3.76 | 3.83 | 2.98 |

As shown in table 3, the average bitrate accuracy for all views is 2.98%. Table 4 shows the PSNR degradation for each view using R-lambda model-based rate control.

Table 4. PSNR (dB) degradation for each view of R-lambda model-based model based rate control

| sequence | PSNR without rate control (dB) |  |  |  |  | PSNR with the URQ model-based rate control (dB) |  |  | View-wise Δ PSNR |  |  | Average of Δ PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | view0 |  | view1 |  | view2 | view0 | view1 | view2 | view0 | view1 | view2 |  |
| Balloons | 43.10 | 42.31 |  | 41.97 |  | 43.01 | 41.80 | 41.52 | -0.10 | -0.50 | -0.46 | -0.35 |
|  | 41.07 | 40.02 |  | 39.77 |  | 40.93 | 39.54 | 39.31 | -0.14 | -0.48 | -0.46 | -0.36 |
|  | 38.51 | 37.44 |  | 37.22 |  | 38.42 | 37.07 | 36.88 | -0.09 | -0.37 | -0.34 | -0.27 |
|  | 35.68 | 34.70 |  | 34.51 |  | 35.59 | 34.45 | 34.28 | -0.09 | -0.25 | -0.23 | -0.19 |
| Kendo | 43.61 | 42.77 |  | 42.39 |  | 43.35 | 42.43 | 42.06 | -0.26 | -0.33 | -0.32 | -0.31 |
|  | 41.46 | 40.46 |  | 40.19 |  | 41.28 | 40.20 | 39.91 | -0.18 | -0.26 | -0.28 | -0.24 |
|  | 38.99 | 38.05 |  | 37.85 |  | 38.79 | 37.76 | 37.55 | -0.20 | -0.28 | -0.30 | -0.26 |
|  | 36.38 | 35.53 |  | 35.37 |  | 36.19 | 35.37 | 35.23 | -0.19 | -0.16 | -0.13 | -0.16 |
| Newspaper | 41.35 | 40.32 |  | 39.91 |  | 41.00 | 39.58 | 39.16 | -0.35 | -0.74 | -0.75 | -0.61 |
|  | 39.00 | 38.00 |  | 37.55 |  | 38.77 | 37.34 | 36.86 | -0.24 | -0.66 | -0.70 | -0.53 |
|  | 36.46 | 35.59 |  | 35.12 |  | 36.36 | 35.14 | 34.59 | -0.10 | -0.45 | -0.53 | -0.36 |
|  | 33.91 | 33.19 |  | 32.62 |  | 33.94 | 32.98 | 32.31 | 0.04 | -0.20 | -0.30 | -0.16 |
| Gtfly | 40.19 | 39.68 |  | 39.64 |  | 39.99 | 39.38 | 39.36 | -0.20 | -0.30 | -0.28 | -0.26 |
|  | 37.78 | 37.43 |  | 37.39 |  | 37.68 | 37.14 | 37.11 | -0.10 | -0.29 | -0.28 | -0.23 |
|  | 35.60 | 35.36 |  | 35.31 |  | 35.50 | 35.05 | 35.01 | -0.10 | -0.31 | -0.30 | -0.24 |
|  | 33.46 | 33.30 |  | 33.25 |  | 33.50 | 33.23 | 33.17 | 0.04 | -0.07 | -0.08 | -0.04 |
| PoznanHall2 | 42.13 | 41.87 |  | 41.72 |  | 41.78 | 41.22 | 41.11 | -0.35 | -0.65 | -0.61 | -0.53 |
|  | 41.15 | 40.87 |  | 40.78 |  | 40.49 | 39.86 | 39.79 | -0.67 | -1.02 | -0.99 | -0.89 |
|  | 39.76 | 39.47 |  | 39.44 |  | 39.14 | 38.62 | 38.60 | -0.62 | -0.85 | -0.83 | -0.77 |
|  | 38.05 | 37.73 |  | 37.73 |  | 37.58 | 37.15 | 37.22 | -0.46 | -0.58 | -0.51 | -0.52 |
| PoznanStreet | 39.86 | 38.99 |  | 38.93 |  | 39.78 | 38.73 | 38.65 | -0.08 | -0.26 | -0.28 | -0.20 |
|  | 37.91 | 37.15 |  | 36.96 |  | 37.75 | 36.79 | 36.60 | -0.16 | -0.35 | -0.37 | -0.29 |
|  | 35.90 | 35.28 |  | 35.00 |  | 35.66 | 34.90 | 34.64 | -0.24 | -0.38 | -0.36 | -0.33 |
|  | 33.78 | 33.32 |  | 33.01 |  | 33.74 | 33.18 | 32.89 | -0.04 | -0.13 | -0.13 | -0.10 |
| Dancer | 38.75 | 37.95 |  | 38.13 |  | 38.66 | 37.88 | 38.04 | -0.09 | -0.06 | -0.08 | -0.08 |
|  | 35.89 | 35.34 |  | 35.54 |  | 35.80 | 35.20 | 35.39 | -0.09 | -0.14 | -0.15 | -0.13 |
|  | 33.37 | 32.95 |  | 33.16 |  | 33.28 | 32.79 | 32.98 | -0.09 | -0.17 | -0.18 | -0.14 |
|  | 31.10 | 30.76 |  | 30.94 |  | 31.03 | 30.71 | 30.89 | -0.07 | -0.05 | -0.04 | -0.05 |
|  |  |  |  |  |  |  |  | Avg | -0.19 | -0.37 | -0.37 | -0.31 |

As shown in table 4, the average degradation for all views is 0.31dB.

We showed results about the bitrate accuracy and PSNR degradation for the all views for cases using URQ model-based and R-lambda model-based rate control schemes. The purpose of rate control is not only to control the total average bitrate, but also to limit fluctuations in the instantaneous bitrate. To assess this aspect of the proposed methods, we provide the frame-wise graphical accumulated virtual buffer usage for all test sequences, for all rate points and for all views in the uploaded Excel file included with this document.

# Conclusion

We proposed an initial rate control scheme for 3D multi-view video coding using both URQ and R-lambda model-based methods. We implemented those rate control schemes on HTM6.1 reference software and evaluated the bitrate accuracy and PSNR degradation compared to the results from the current HTM6.1 reference software without the rate control schemes. Most rate control schemes for single-view coding use error (usually MAD) prediction in the temporal domain to predict the to-be-generated bits for the current coding unit. Both these rate control schemes, as well as many others developed for earlier coding standards, use previously generated bits and errors (usually in the form of MAD) to predict the to-be-generated bits for the current coding unit. Also, target bits for the first frame of a GOP are usually assigned heuristically based on the average bpp (bits per pixels) for the whole sequence. This bpp-based bit assignment doesn’t consider the fact that extended views have one or more inter-view reference frames. According to those characteristics, the single-view rate control schemes in HEVC are not suitable for the extended views of multi-view extension and needs some improvement.

We recommend this proposed implementation of rate control for multi-view video coding to be implemented to the current HTM software as a starting point of multi-view rate control. After evaluation of those rate control schemes, we also recommend that our previous proposal described in the document JCT3V-C0090, depth map-based inter-view MAD prediction for the extended views [4].

# References

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Rate control based on unified RQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H0213 (m23088), San José, CA, USA, Feb. 2012.

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Improvement of the rate control based on pixel-based URQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0094 (m24333), Geneva, Switzerland, Apr.-May. 2012.

- B. Li, H. Li, L. Li and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0103, Shanghai, China, Oct. 2012.

- W. Lim, D. Sim and I. V. Bajić, “JCT3V – Improvement of the rate control for 3D multi-view video coding,” ISO/IEC JTC1/SC29/WG11, JCT3V-C0090, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

KWU and SFU do not have any current or pending patent rights relating to the technology described in this contribution.