| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0112 |
| --- | --- |

| Title: | 3D-CE2.h related: Default disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Jaewon Yoon Ho-Cheon Wey Yongjin Cho Chanyul Kim 416, Maetan 3 Yeongtong, Suwon Gyeonggi 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com jw0413.yoon@samsung.com hc.wey@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a method for deriving a default disparity vector. In the current derivation process for a disparity vector, if a disparity vector is not available after the neighbouring block disparity vector (NBDV) derivation process, the disparity vector is not refined from a coded depth map of the reference view and is set equal to (0, 0) by default. However, even though a disparity vector is not available, the default disparity vector (0, 0) can also be refined as if a disparity vector is available. The proposed method reportedly shows an average of 0.2% bit-saving for video 1 and video 2, and 0.1% bit-saving for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate.

# Introduction

In the current 3D-HEVC [1], after a neighbouring block disparity vector derivation process, the disparity vector is refined from the coded depth map of reference view in order to improve the accuracy of the disparity vector. The derivation process for a disparity vector can be depicted as shown in Figure 1 and illustrated as follows.

- availableDV, which indicates whether a disparity vector is found or not, is set to 0, and the disparity vector is set to (0, 0) as a default disparity vector.

- NBDV derivation process is performed as following steps.

  - For every temporal neighbouring block, if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For every spatial neighbouring block, if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For every spatial neighbouring block, if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

- If availableDV is equal to 1 and deriveFromDepthFlag, which indicates whether a disparity vector is derived from a coded depth map or not, is equal to 1, the disparity vector is refined from a coded depth map.

Figure 1. Derivation process for a disparity vector in the current 3D-HEVC.

However, when a disparity vector is not available, the disparity vector is always set to (0, 0) by default, and this default disparity vector is used for inter-view prediction such as temporal/disparity inter-view motion vector candidate derivation and view synthesis prediction. However, it may cause inaccurate inter-view prediction because the default disparity vector (0, 0) might be far different from a real disparity vector.

# Proposed Method

The proposed method is simply to allow a depth based refinement for the default disparity vector when a disparity vector is not found in the NBDV derivation process (i.e. when a disparity vector is not available). The proposed derivation process for a disparity vector can be depicted as shown in Figure 2 and illustrated as follows (The modification is highlighted in yellow.).

- availableDV, which indicates whether a disparity vector is found or not, is set to 0, and the disparity vector is set to (0, 0) as a default disparity vector.

- NBDV derivation process is performed as following steps.

- For every temporal neighbouring block, if availableDV is equal to 0, find the disparity vector from neighbouring block, and once the disparity vector is found availableDV is set to 1.

- For every spatial neighbouring block, if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

- For every spatial neighbouring block, if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

- If availableDV is equal to 1 and deriveFromDepthFlag, which indicates whether a disparity vector is derived from a coded depth map or not, is equal to 1, the disparity vector is refined from a coded depth map.

- If availableDV is equal to 0, the default disparity vector (0, 0) is refined from a coded depth map.

Figure 2. Proposed derivation process for a disparity vector.

In addition, in the current 3D-HEVC, if disparity vector is not found in NBDV derivation process, the residual prediction is not used. However, a refined default disparity vector can be used for the residual prediction. Therefore, this contribution also proposes to use the residual prediction in a case where using a refined default disparity vector. And when obtaining a residual data from a reference view for the residual prediction, the refined default disparity vector can also be used. It can be simply implemented by setting availableDV equal to 1 when refining a default disparity vector.

# Experimental Results

The performance evaluation is carried out based on the common test condition [2] and HTM6.0 reference software. The coding performance of the proposed methods is shown in Table 1. The proposed method achieves an average of 0.2% BD-rate gain for dependent views (video 1 and video 2) and 0.1% BD-rate gain for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate.

Table 1. Performance comparison between the proposed method and CTC anchor

# Conclusion

In this contribution, it is proposed to refine the default disparity vector when a disparity vector is not available and to enable the residual prediction when using refined default disparity vector. The proposed method shows 0.2% bit-saving for dependent views and 0.1% bit-saving for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate with minor change. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3,” JCT3V-C1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, January 2013, Geneva, Switzerland.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, January 2013, Geneva, Switzerland.