| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0113 |
| --- | --- |

| Title: | 3D-CE2.h related: Simplification of disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Jaewon Yoon Ho-Cheon Wey Byeongdoo Choi Yongjin Cho Chanyul Kim 416, Maetan 3 Yeongtong, Suwon Gyeonggi 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com jw0413.yoon@samsung.com hc.wey@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a method for simplifying a disparity vector derivation. In the current 3D-HEVC, after a disparity vector is derived from the neighbouring block disparity vector (NBDV) derivation process, the disparity vector is used to find the corresponding block in coded depth of the reference view and is refined by the representative depth value in the corresponding depth block. However, it seems to be complicated that performing NBDV derivation process just to obtain the disparity vector used to find the corresponding depth block. Therefore, it is proposed to derive a disparity vector by only using the coded depth map of the reference view, but not searching spatial or temporal neighbouring blocks. The proposed method is comprised of two steps. Firstly, the first disparity vector is derived from the corresponding depth block pointed at by the initial disparity vector corresponding to the depth value of (1 << (bit depth – 1)). And then, the final disparity vector is derived from the corresponding depth block pointed at by the first disparity vector. The proposed method reportedly shows an average of 0.2% and 0.4% bit-saving for video 1 and video 2, respectively, 0.1% bit-saving for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate.

# Introduction

In the current 3D-HEVC [1], the disparity vector, which is derived from NBDV derivation process, is used to find the corresponding block in coded depth of the reference view and is refined by the representative depth value in the corresponding depth block. The detailed derivation process for disparity vector can be illustrated as follows.

- availableDV, which indicates whether a disparity vector is found or not, is set to 0.

- The horizontal component of the disparity vector is set to 0.

- The vertical component of the disparity vector is set to 0.

- NBDV derivation process is performed as following ordered steps.

  - For every temporal neighbouring block (right bottom and center blocks in the candidate picture list), if availableDV is equal to 0, find the disparity vector from neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For every spatial neighbouring block (A1, B1, B0, A0, and B2 blocks), if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For every spatial neighbouring block (A1, B1, B0, A0, and B2 blocks), if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

- If availableDV is equal to 1 and deriveFromDepthFlag, which indicates whether a disparity vector is derived from a coded depth map or not, is equal to 1, following ordered steps apply.

  - Find the corresponding depth block of the reference view by using the disparity vector.

  - Select the representative depth value of the corresponding depth block.

  - Convert the representative depth value to the disparity vector.

However, it seems to be complicated to perform both the NBDV derivation process and the refinement process. To be specific, while the disparity vector derived from the NBDV derivation process is only used to locate the corresponding depth block of the reference view, the NBDV derivation process itself requires a relatively large amount of computing resources since it needs to search temporal and spatial neighbouring blocks.

# Proposed Method

The proposed method is to use only coded depth information of the reference view to derive the disparity vector, but not to perform the NBDV derivation process. The simplified derivation process for the disparity vector can be illustrated as follows.

- The horizontal component of the disparity vector is set to the value corresponding to the depth value (1 << (bit depth – 1)).

- The vertical component of the disparity vector is set to 0.

- Find the corresponding depth block of the reference view by using the disparity vector.

- Select the representative depth value of the corresponding depth block

- Convert the representative depth value to the disparity vector

- Repeat 3 to 5 once again.

The proposed method can be explained as two step search as shown in Figure 1. At the first step, the first disparity vector is derived from the corresponding block pointed at by the initial disparity vector. And then, the final disparity vector is derived from the corresponding block pointed at by the first disparity vector. Here, the initial converted value of the depth value (1 << (bit depth – 1)) at the first step is adaptively changed according to the camera parameters.

Figure 1. Proposed disparity derivation process

In addition, the derivation process for disparity vector can be separated according to whether a coded depth map is available as shown in Figure 2. If the coded depth information is available, the proposed simplified disparity vector derivation process can be used. Otherwise, the NBDV derivation process can be used.

Figure 2. Separation of the disparity vector derivation according to the availability of a coded depth map

# Experimental Results

The performance evaluation is carried out based on the common test condition [2] and HTM6.0 reference software. The coding performance of the proposed methods is shown in Table 1. The proposed method achieves an average of 0.2% and 0.4% BD-rate gain for video 1 and video 2, respectively, and 0.1% BD-rate gain for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate.

Table 1. Performance comparison between the proposed methods and CTC anchor

# Conclusion

In this contribution, it is proposed to simplify the disparity vector derivation by only using coded depth map of the reference view. In spite of the simplification, the proposed method shows 0.2% and 0.4% bit-saving for video 1 and video, respectively, 0.1% bit-saving for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3,” JCT3V-C1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, January 2013, Geneva, Switzerland.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, January 2013, Geneva, Switzerland.