| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0114_r1 |
| --- | --- |

| Title: | 3D-CE5.h related: Simplification of illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Ho-Cheon Wey Jaewon Yoon Byeongdoo Choi Yongjin Cho Chanyul Kim 416, Maetan 3 Yeongtong, Suwon Gyeonggi 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com hc.wey@samsung.com jw0413.yoon@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes four methods for the illumination compensation (IC) in 3D-HEVC. Firstly, the scale factor derivation in the IC seems to have relatively high complexity because there are many multiplications. Therefore, it is proposed to remove multiplicative terms from the scale factor derivation. Secondly, it is proposed to apply the IC to only 2Nx2N partition to reduce the complexity. Thirdly, when deriving the top-left sample of the reference block for IC parameter estimation, if the disparity vector points a factional sample position, left or above integer sample position is always selected. Thus, in order to improve coding efficiency, it is proposed to point the nearest integer sample position by applying the rounding offset to the top-left sample derivation. Finally, after IC parameter derivation, some combinations of the scale factor and the offset, such as the case where the scale factor is equal to 1 and the offset is equal to 0, do not need to perform the IC. Therefore, it is proposed to selectively perform IC by checking derived IC parameters. The proposed methods reportedly show an average of 0.2% and 0.6% bit-saving for video 1 and video 2, respectively, 0.2% bit-saving for video PSNR vs. video bitrate, and 0.1% bit-saving for video PSNR vs. total bitrate and synth PSNR vs. total bitrate.

# Introduction

In the current 3D-HEVC [1], the illumination compensation (IC) is used to compensate illumination mismatch between difference views for all color components and depth. And the IC is performed by the linear model expressed as following equation.

where (x, y) represents the sample position in the reference block, Pred’(x,y) represents an illumination compensated sample, Pred(x,y) represents a sample in the reference block, and a represents the scale factor and b represents the offset of the linear model.

And the parameters of linear model a (scaling factor) and b (offset) are calculated as following equations.

,

where N represents number of available neighbouring pixel, NBcurr(i) represents a the neighbouring sample of the current block, and NBref(i) represents a the neighbouring sample of the reference block.

In addition, the IC derivation is performed for all color components (Y, U and V) and depth, and all prediction directions (reference list 0 and reference list 1) of each prediction unit (PU) when the coding unit (CU) is determined to use the IC.

Although the IC could give some amount of coding gains, performing the IC seems to need a relatively large amount of computing resources. Therefore, this contribution introduces simplification methods for IC to decrease the computational complexity.

# Proposed Methods

## Method 1: Simplification of the scale factor

The scale factor derivation in the IC has relatively high complexity because there are numerous multiplications and this derivation is performed for all color component(Y, U and V) and depth, and all prediction direction (reference list 0 and reference list 1) of each PU when the CU-level IC enabling flag is equal to 1.

Thus, it is proposed to remove multiplicative terms from the scale factor derivation. The proposed derivation for the scale factor is described as following equation.

where N represents number of available neighbouring pixel, NBcurr(i) represents a the neighbouring sample of the current block, and NBref(i) represents a neighbouring sample of the reference block.

In order to apply the proposed method to the 3D-HEVC, the modified IC parameters (scaling factor and offset) derivation and the performing IC process can be explained as following equations.

,

,

,

,

where N represents number of available neighbouring pixel, NBcurr(i) represents a the neighbouring sample of the current block, NBref(i) represents a neighbouring sample of the reference block, a represents the scale factor, b represents the offset, (x, y) represents the sample position in the reference block, Pred’(x,y) represents an illumination compensated sample, Pred(x,y) represents a sample in the reference block.

The coding performance of Method 1 (Simplification of the scale factor) is shown in Table 1. From the results, it is observed that in spite of the simplification, Method 1 achieves an average of 0.1% and 0.3% bit-saving for video 1 and video 2, respectively, 0.1% bit-saving for video PSNR vs. total bitrate and synth PSNR vs. total bitrate.

Table 1. Test results for Method 1 (Simplification of the scale factor)

## Method 2: Applying IC to 2Nx2N partition only

In the current 3D-HEVC, the IC is applied to all PU partition sizes (PART_2Nx2N, PART_2NxN, PART_Nx2N, PART_NxN, PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N). In order to reduce the complexity, applying the IC can be restricted to specific block partition. Thus, this contribution proposes a method to apply the IC to 2Nx2N partition (PART_2Nx2N) only.

With Method 2, the number of neighbouring pixel is always represented as the power of 2 and PU partition size below 2Nx2N does not need to perform the mode decision for the IC at the encoder side.

The coding performance of Method 2 (Applying IC to 2Nx2N partition only) is shown in Table 2. From these results, it is observed that Method 2 can reduce the complexity without coding loss.

Table 2. Test results for Method 2 (Applying IC to 2Nx2N partition only)

## Method 3: Derivation of top-left sample in the reference block

In the current IC parameter derivation, when deriving the top-left sample of the reference block for IC parameter estimation, if the disparity vector points a factional sample position (a, b,… , o in Figure 1), above or left integer sample position (A in Figure 1) is always selected as shown in Figure 1. However, it would be expected that the coding efficiency could be increased if the nearest integer sample would be chosen.

Figure 1. Integer and fractional sample position

Therefore, in order to improve the coding efficiency, it is proposed to point the nearest integer sample position by applying the rounding offset in the top-left sample derivation. The modified derivation process in text of 3D-HEVC can be modified as following (highlighted in green).

- H.8.5.2.2.5.1 Derivation process for illumination compensation mode availability and parameters … - The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as - xRLX = xC + ( (mvLX[ 0 ] + 2 + (cIdx ? 2 : 0) ) >> (2 + (cIdx ? 1 : 0)) ) (H-193) - yRLX = yC + ( (mvLX[ 1 ] + 2 + (cIdx ? 2 : 0) ) >> (2 + (cIdx ? 1 : 0)) ) (H-194) …

The coding performance of Method 3 (Derivation of top-left sample in the reference block) is shown in Table 3. From the results, it is observed that Method 3 achieves an average of 0.1% bit-saving for video 1 and video 2.

Table 3. Test results for Method 3 (Derivation of top-left sample in the reference block)

## Method 4: Preventing useless decoding process

After IC parameter derivation, some combinations of the scale factor and the offset do not need to perform the IC. For instance, if the scale factor a is equal to 1 and the offset b is equal to 0, these parameters cannot change the sample value in the reference block at all. Table 4 shows a frequency of the cases which do not need to apply IC after the parameter derivation. As can be seen in Table 4, almost 6% of cases are performing unnecessary decoding process.

Table 4. Frequency of the (a = 1 & b = 0) cases (based on CTC)

Therefore, it is proposed to selectively apply IC by checking these cases, which do not need to perform the IC. In order to prevent these odd cases, the text of 3D-HEVC can be simply modified as following (highlighted in green). It can be changed by putting a one-line sentence which checks the value of the scale factor and the offset and sets puICFlagLX flag, which indicates whether performing the IC or not.

- H.8.5.2.2.5 Illumination compensated sample prediction process - … - If predFlagL0 is equal to 1 and predFlagL1 is equal to 0, - clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 ) (Error: Reference source not found-1) - predSamples[ x ][ y ] = !puIcFlagL0 ? clipPredVal : ( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 ) (Error: Reference source not found-2) - Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1, - clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 ) (Error: Reference source not found-3) - predSamples[ x ][ y ] = !puIcFlagL1 ? clipPredVal : ( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 ) (Error: Reference source not found-4) - Otherwise, - predVal0 = !puIcFlagL0 ? predSamplesL0[ x ][ y ] : ( ( predSamplesL0[ x ][ y ] * icWeightL0 ) >> icShiftL0 ) + ( icOffsetL0 << shift1 ) ) (Error: Reference source not found-5) - predVal1 = !puIcFlagL1 ? predSamplesL1[ x ][ y ] : ( ( predSamplesL1[ x ][ y ] * icWeightL1 ) >> icShiftL1 ) + ( icOffsetL1 << shift1 ) ) (Error: Reference source not found-6) - predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, ( predVal0 + predVal1 + offset2 ) >> shift2 ) (Error: Reference source not found-7) - H.8.5.2.2.5.1 Derivation process for illumination compensation mode availability and parameters … - 3. The derivation process for illumination compensation parameters as specified in subclause H.8.5.2.2.5.2 is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample list, the number of neighbouring samples numNeighSamlesLX and the size of the current luma coding block nCSl as inputs and the illumination parameters icWeightLX, icOffsetLX and icShiftLX as outputs. - 4. If icWeightLX is equal to (1 << icShiftLX) and icOffsetLX is equal to 0, puIcFlagLX is set to equal to 0.

In addition, if the shift factor has fixed 4 bits as described in Method 1 section, the last sentence can be changed as follows.

4. If icWeightLX is equal to 16 and icOffsetLX is equal to 0, puIcFlagLX is set to equal to 0.

The coding performance of Method 4 (Preventing useless decoding process) is shown in Table 5. From the results, it is observed that Method 4 can reduce the complexity without coding loss.

Table 5. Test results for Method 4 (Preventing useless decoding process)

# Experiment Results

The performance evaluation is carried out based on the common test condition [2] and HTM6.0 reference software. The coding performance of the proposed methods (combing methods 1, 2, 3, and 4) is shown in Table 6. The proposed methods achieves an average of 0.2% and 0.6% bit-saving for video 1 and video 2, respectively, 0.2% bit-saving for video PSNR vs. video bitrate, and 0.1% bit-saving for video PSNR vs. total bitrate and synth PSNR vs. total bitrate.

Table 6. Performance comparison between the proposed methods and anchor

Table 7 shows the summary of the simulation results. From these results, it can be observed that more coding gains are obtained when combining all methods.

Table 7. Summary of simulation results

# Conclusion

This contribution proposes the methods to simplify the illumination compensation. In spite of the simplification, the proposed methods could obtain 0.2% and 0.6% coding gain for video 1 and video 2, respectively, 0.2% gain for video PSNR vs. video bitrate, and 0.1% gain for video PSNR vs. total bitrate and synth PSNR vs. total PSNR. Therefore, it is recommended to adopt all proposed methods into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3,” JCT3V-C1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, January 2013, Geneva, Switzerland.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, January 2013, Geneva, Switzerland.