| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0117 |
| --- | --- |

| Title: | 3D-CE1.h related: Simplification of VSP insertion process |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Byeongdoo Choi, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

View synthesis prediction (VSP) was integrated on the current 3D-HTM (JCT3V-C0152). VSP is used as a new candidate in Skip and Merge modes and inserted right before a temporal candidate. However, if any of the adjacent blocks is coded with VSP, it is inserted as the candidate for the block. In the insertion process for a candidate from a potentially VSP block, the current process investigates all the neighboring blocks whether they are coded with VSP or not. In this contribution, three methods are introduced for simplification of the current VSP insertion process. Especially, the proposed VSP insertion process only based on the left and above blocks, such as the VSP skip flag signalling in 3D-AVC, achieves -0.1% gain in both the decoded and synthesized views as well as the complexity reduction.

# Current VSP Insertion Process

The current VSP insertion process investigates all the neighboring blocks including A1, B1, B0, A1, and B2 in Fig. 1 whether they are coded with VSP or not. If any of the neighboring blocks is coded with VSP, it is used as the candidate for the block in Skip and Merge modes. Otherwise, VSP is inserted right before a temporal candidate. The current Merge list is constructed as follows:

1. Add inter-view candidate

2. Add left candidate (A1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

3. Add above candidate (B1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

4. Add above-right candidate (B0)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

5. Add disparity candidate

6. Add below-left candidate (A1) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

7. Add above-left candidate (B2) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

8. Add VSP candidate if the number of the candidates is less than 5 and not added before

9. Add bottom-right or center temporal candidate (Col) if the number of the candidates is less than 6

Fig. 1. Spatially neighboring blocks and temporally collocated blocks

# Fixed VSP Insertion Process

Unlike the current VSP insertion process, this method fixes the VPS candidate position in Skip and Merge modes. It makes the Merge list construction process simple. Based on the fixed VSP insertion process, the Merge list is constructed as follows:

1. Add inter-view candidate

2. Add left candidate (A1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

3. Add above candidate (B1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

4. Add above-right candidate (B0)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

5. Add disparity candidate

6. Add VSP candidate if the number of the candidates is less than 5 and not added before

7. Add below-left candidate (A1) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

8. Add above-left candidate (B2) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

8. Add VSP candidate if not added before if the number of the candidates is less than 5

9. Add bottom-right or center temporal candidate (Col) if the number of the candidates is less than 6

# Simplified VSP Insertion Process

This method determines the VPS candidate position in Skip and Merge modes, based on the left (A1) and above (B1) blocks, such as the VSP skip flag signalling in 3D-AVC. Therefore, it makes the Merge list construction process simple. Based on the simplified VSP insertion process, the Merge list is constructed as follows:

1. Add inter-view candidate

2. Add left candidate (A1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

3. Add above candidate (B1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

4. Add above-right candidate (B0)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

5. Add disparity candidate

6. Add VSP candidate if the number of the candidates is less than 5 and not added before

7. Add below-left candidate (A1) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

8. Add above-left candidate (B2) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

8. Add VSP candidate if not added before if the number of the candidates is less than 5

9. Add bottom-right or center temporal candidate (Col) if the number of the candidates is less than 6

# Simplified VSP Insertion Process with Lowest Priority

The VSP candidate position in Skip and Merge modes is decided based on the left (A1) and above (B1) blocks, such as the VSP skip flag signalling in 3D-AVC. In this method, the VSP position is placed right after the temporal candidate when the left and above blocks are not coded with VSP. Due to its lowest priority in Skip and Merge modes, the VSP insertion strongly depends on availability of the neighboring candidates. Based on the proposed method, the Merge list is constructed as follows:

1. Add inter-view candidate

2. Add left candidate (A1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

3. Add above candidate (B1)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

4. Add above-right candidate (B0)

- But if VSP coded and VSP candidate is not added before, add VSP candidate

5. Add disparity candidate

6. Add below-left candidate (A1) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

7. Add above-left candidate (B2) if the number of the candidates is less than 5

- But if VSP coded and VSP candidate is not added before, add VSP candidate

8. Add VSP candidate if not added before if the number of the candidates is less than 5

8. Add bottom-right or center temporal candidate (Col) if the number of the candidates is less than 6

9. Add VSP candidate if the number of the candidates is less than 6 and not added before

# Test Results

Test results are summarized in Tables 1 to 3. Table 1 is about the fixed VSP insertion process. The result shows that the proposed method has the negligible coding loss in the dependent views. Table 2 is about the simplified VSP insertion process. The result illustrates that the proposed method achieves the small coding gain in the dependent views. Table 3 is about the simplified VSP insertion process with the lowest priority. The result demonstrates that the proposed method achieves the coding gain in both the decoded and synthesized views.

Table 1. Performance of the fixed VSP insertion process

Table 2. Performance of the simplified VSP insertion process

Table 3. Performance of the simplified VSP insertion process with lowest priority

# Conclusion

By integrating the proposed methods on 3D-HTM 6.0, it simplifies the current VSP insertion process as well as achieves the coding gain in both the decoded and synthesized views. Since the modifications are very simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).