| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0120 |
| --- | --- |

| Title: | 3D-CE3.h related: MPI restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Motion parameter inheritance (MPI) was integrated on the current 3D-HTM 6.0 (JCT3V-C0137). MPI is used as a new candidate in Skip and Merge modes in a depth component. This proposal uses MPI only in Skip and Merge modes of a 2Nx2N partition, because a motion vector correlation between texture and depth components is less correlated in partitioned blocks than 2Nx2N block. Also, the proposed method does not inherit view synthesis prediction (VSP) instead of MPI when a corresponding texture block is coded with VSP. Experimental results demonstrate that the proposed method simplifies the MPI process without the coding loss.

# MPI Restriction

Similar to a concept of an inter-view motion vector candidate in the texture coding, MPI, which reuses motion information in a corresponding texture block (JCT3V-C0137) as shown in Fig. 1, is employed as a new candidate in Skip and Merge modes in the depth coding.

- Depth VSP is used instead of MPI when the corresponding texture block is coded with VSP, but there is no evidence that VSP is correlated to MPI. Hence, the proposed method does not inherit VSP in this case.

- Since a motion vector correlation between texture and depth components is not strongly correlated in partitioned blocks, the proposed method uses MPI only in Skip and Merge modes of a 2Nx2N partition

Fig. 1. Derivation of corresponding texture block (JCT3V-C0137)

# Test Results

Test results are summarized in Tables 1 to 3. Table 1 is the performance of disabling the VSP inheritance in MPI. Table 2 is about only using MPI to Skip and Merge modes of a 2Nx2N partition. Finally, Table 3 is about the combined method. The results demonstrate that the proposed methods have no coding loss.

Table 1. Performance of disabling the VSP inheritance in MPI

Table 2. Performance of only using MPI to Skip and Merge modes of a 2Nx2N partition

Table 3. Performance of the combined method

# Conclusion

By integrating the proposed method on the current 3D-HTM 6.0, it simplifies the MPI process without the coding loss. Since the modifications are very simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).