| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0121 |
| --- | --- |

| Title: | 3D-CE3.h related: Removal of depth quadtree prediction and simple early depth CU decision for fast encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Depth quadtree prediction was integrated on the current 3D-HTM (JCT3V-B0068). The depth quadtree prediction forces an encoder to limit the depth quadtree at the same level as the texture quadtree, so the encoding time can be significantly reduced by about 75%. Also for the coding improvement, a split flag and a partition size in the depth CU are not transmitted to a decoder, but predicted by referring the coding information of the collocated texture CU. Hence, there exists a parsing dependency between the texture and depth components. To solve this parsing problem while maintaining low encoding complexity, this proposal introduces early depth CU determination instead of the depth quadtree prediction. The proposed method does not split a depth CU further when the best mode of the CU is Skip as well as the collocated texture CU is coded with Skip. The result shows that the proposed method maintains the current encoding complexity without the parsing dependency.

# Early Depth CU Decision

With the depth quadtree prediction, a depth CU cannot be split more than its collocated texture CU, and 2NxN, Nx2N, and NxN partition sizes are not allowed when the texture partition size is 2Nx2N, 2NxN or Nx2N. These limitations significantly reduce the encoding time. However, the depth quadtree prediction allows the parsing dependency between texture and depth components, because the split flag and partition size in the depth CU are predicted by the collocated texture CU coding information.

To solve the parsing problem while maintaining the low encoding complexity, the proposal introduces a simple early depth CU decision method. The proposed method does not split the depth CU further when the best mode of the current depth CU is Skip as well as the collocated texture CU is encoded with Skip.

# Test Results

Test results are summarized in Tables 1 and 2. Table 1 is the performance when the prediction part, which makes the parsing dependency, is removed in the depth quadtree prediction. Table 2 is the performance when the proposed early depth CU decision method is used instead of the depth quadtree prediction. The result shows that giving up the prediction part allowing the parsing dependency results in the coding loss of 0.3%. However, the proposed method maintains the low encoding complexity with the less coding loss and without the parsing dependency.

Table 1. Performance when the prediction part is removed in the depth quadtree prediction

Table 2. Performance of the early CU decision method

# Conclusion

By integrating the proposed early CU decision method on the current 3D-HTM 6.0, it solves the parsing dependency in the depth quadtree prediction. Since the proposed fast depth coding method is very simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).