| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0122 |
| --- | --- |

| Title: | 3D-CE5.h related: AMVP candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

One inter-view motion vector candidate, five spatial candidates, and two temporal candidates are used for AMVP in 3D-HEVC. Three candidates among them are finally selected during the AVMP candidate list construction. In order to distinguish the three candidates, a motion vector predictor index (mvp_l0_idx and mvp_l1_idx) is defined in 3D-HEVC, instead of employing a conventional flag (mvp_l0_flag and mvp_l1_flag) in HEVC. This proposal fixes the number of the candidates in AMVP to two, so we do not have to change the syntax in HEVC for the 3D extension parts. In order to reduce the number of the candidates from three to two, the first option in the proposed method is to remove the inter-view motion vector candidate and then choose two candidates among the spatial and temporal candidates. The second option is to use the inter-view motion vector candidate and then choose one candidate among the spatial and temporal candidates. The results demonstrate that both options have no coding loss and reduce both the encoding and decoding times. In addition, this proposal introduces separate flags to decide a usage of the inter-view motion vector candidate in Merge and AMVP, respectively.

# Current AMVP List Construction

One inter-view motion vector candidate, five spatial candidates, and two temporal candidates are used for AMVP. Three candidates among them are finally selected during the AVMP candidate list construction, as show in Fig. 1. A motion vector predictor index (mvp_l0_idx and mvp_l1_idx) is used to distinguish the candidates in 3D-HEVC.

Fig. 1. Current AMVP list construction

# Proposed AMVP List Construction

-  Method #1: Not using inter-view motion vector candidate

The proposed method employs only two candidates for AMVP. In order to reduce the number of the candidates from three to two, it does not use the inter-view motion vector candidate. Hence, five spatial candidates and two temporal candidates are only used, which is the same AMVP list construction process as HEVC, as displayed in Fig. 2. A motion vector predictor flag (mvp_l0_flag and mvp_l1_flag) defined in HEVC is used to distinguish two candidates, so we do not have to change the syntax related to AMVP in HEVC for the 3D extension parts

Fig. 2. Proposed AMVP list construction #1

-  Method #2: Using inter-view motion vector candidate

The proposed method employs only two candidates. In order to reduce the number of the candidates from three to two, it uses one inter-view motion vector candidate and chooses one candidate among five spatial candidates and two temporal candidates, as shown in Fig. 3. A motion vector predictor flag (mvp_l0_flag and mvp_l1_flag) defined in HEVC is used to distinguish two candidates, so we do not have to change the syntax related to AMVP in HEVC for the 3D extension parts

Fig. 3. Proposed AMVP list construction #2

# Test Results

Test results are summarized in Tables 1 and 2. Table 1 is the performance when the inter-view motion vector candidate is not used in two AMVP candidates. Table 2 is the performance when the inter-view motion vector candidate is used in two AMVP candidates. Both results demonstrate that reducing the number of the candidates for AMVP from three to two does not affect the coding performance. Also, it reduces the encoding and decoding times. In the specification text, we do not have to change the syntax elements related to the AMVP motion vector predictor in HEVC for 3D extension parts

Table 1. Method #1: Not using inter-view motion vector candidate

Table 2. Method #2: Using inter-view motion vector candidate

# Tool Enabling Flag

According to Table 1, the inter-view motion vector candidate in AVMP does not give any influence on the coding performance, so it should be turned off. However, the inter-view motion vector candidates are enabled or disabled in Merge and AMVP with one flag, iv_mv_pred_flag, so it is impossible to enable or disable the inter-view motion vector candidate separately in Merge and AMVP. Therefore, we propose to define two flags for enabling or disabling the inter-view motion vector candidate in Merge and AMVP separately (merge_iv_mv_pred_flag and amvp_iv_mv_pred_flag) as follows.

- H.7.3.2.1.2 Video parameter set extension 2 syntax

if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) {

merge_iv_mv_pred_flag[layered] amvp_iv_mv_pred_flag[layered]

iv_res_pred_flag[ layerId ]

}

merge_iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of merge mode of the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction in merge mode is not used for the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with layer_id equal to layerId.

When merge_iv_mv_pred_flag[ layerId ] is not present, its value shall be inferred to be equal to 0.

amvp_iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of amvp mode of the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction in amvp mode is not used for the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with layer_id equal to layerId.

When amvp_iv_mv_pred_flag[ layerId ] is not present, its value shall be inferred to be equal to 0.

# Conclusion

By integrating the proposed method on 3D-HTM 6.0, it reduces the encoding and decoding times. Also, the syntax elements related to AMVP in HEVC are not changed for the 3D extension parts. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).