| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0122 |
| --- | --- |

| Title: | Proposed text for JCT3V-D0122 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

- H.7.3.2.1.2 Video parameter set extension 2 syntax

if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) {

merge_iv_mv_pred_flag[layered] amvp_iv_mv_pred_flag[layered]

iv_res_pred_flag[ layerId ]

}

merge_iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of merge mode of the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction in merge mode is not used for the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with layer_id equal to layerId.

When merge_iv_mv_pred_flag[ layerId ] is not present, its value shall be inferred to be equal to 0.

amvp_iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of amvp mode of the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction in amvp mode is not used for the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with layer_id equal to layerId.

When amvp_iv_mv_pred_flag[ layerId ] is not present, its value shall be inferred to be equal to 0.