| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0123 |
| --- | --- |

| Title: | 3D-CE6.h related: DMM simplification and signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Ho-Cheon Wey, Min Woo Park, Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In this proposal, the performance of a depth modelling mode (DMM) is evaluated. For the efficient edge coding, four DMMs, which consist of explicit wedgelet partitioning (DMM1), intra-predicted wedgelet partitioning (DMM2), inter-component wedgelet partitioning (DMM3), and inter-component-predicted contour partitioning (DMM4), were developed. Since each DMM has two sub-modes based on a usage of a delta constant partition value (CPV), the total number of the DMMs is eight. In order to distinguish these eight DMMs, a mode index is signalled. However, it is observed that only DMM1 is enough to code the edge and the remaining modes are redundant. Hence, the proposed method uses only DMM1 for the depth edge coding and signals a flag to indicate whether the delta CPV is used or not. Results demonstrate that the proposed method reduces the encoding and decoding times without the coding loss in CTC. In AI (all intra) condition, the proposed method reduces the encoding time by 74% with the minor coding loss. Furthermore, the specification text related to DMM is significantly cleaned up.

# DMM Simplification and Signalling

Four DMMs have been added to the conventional intra modes for the efficient depth edge coding. Four DMMs are as follows:

- Mode 1: Explicit wedgelet signalling;

- Mode 2: Intra-predicted wedgelet partitioning;

- Mode 3: Inter-component prediction of wedgelet partitions;

- Mode 4: Inter-component-predicted contour partitioning.

Each DMM has two sub-modes based on the usage of CPV, so total eight DMMs are used. Since DMMs have been developed with a similar concept, they are significantly redundant. Through the exhaustive experiments, it is observed that DMM1 is very dominant in terms of the coding performance. Hence, the proposed method removes the remaining modes and uses only DMM1.

In order to distinguish two sub-modes of DMM1 in the proposed method, delta_dc_flag is newly defined. delta_dc_flag equal to zero specifies that the delta CPV is not used for DMM1. Equal to one specifies that the delta CPV is used. When delta_dc_flag is not present, its value is inferred to be equal to zero.

This DMM simplification and signaling makes the specification text much simpler than the current one, as shown in the proposed specification text.

# Test Results

Test results are summarized in Tables 1 and 2. Table 1 shows the performance when DMM1 and DMM2 are only used in (a) CTC and (b) AI conditions, respectively. Table 2 illustrates the performance when DMM1 is only used in (a) CTC and (b) AI conditions, respectively. The result displays that using DMM1 and DMM2 achieves the minor gain with the small encoding and decoding time reduction in CTC. In AI, it has the coding loss of 0.2% in the synthesized views, but significantly reduces the encoding time by 81.5% and the decoding time by 98.4%. When DMM1 is only employed, the encoding time is reduced by 94.7% and 74.0% in CTC and AI, respectively. The decoding time is also reduced by 98.4% and 98.7% in CTC and AI, respectively, with the minor coding loss.

Table 1 (a). Performance when DMM1 and DMM2 are only used in CTC

Table 1 (b). Performance when DMM1 and DMM2 are only used in AI

Table 2 (a). Performance when DMM1 is only used in CTC

Table 2 (b). Performance when DMM1 is only used in AI

# Conclusion

By integrating the proposed method on the current 3D-HTM 6.0, it significantly reduces not only the encoding and decoding times but also the related specification text. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).