| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0123 |
| --- | --- |

| Title: | Proposed text for JCT3V-D0123 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Ho-Cheon Wey, Min Woo Park, Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

- H.7.3.9.1 General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( vps_dmm_flag[ nuh_layer_id ] && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + i ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDDIRDELTA ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && !sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ) ? 2 : 1 |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

    - Coding unit semantics

      - General coding unit semantics

dmm_flag[ x0 ][ y0 ] equal to 0 specifies that depth map model modes are not used. dmm_flag[ x0 ][ y0 ] equal to 1 specifies that depth map model modes are used that the dmm_mode syntax element is present. When the dmm_flag[ x0 ][ y0 ] is not present, its value should be inferred to be equal to 0.

dmm_mode[ x0 ][ y0 ] shall be one of the values shown in Table Error: Reference source not found-1. dmm_mode is used to derive the intra prediction mode when the dmm_flag[ x0 ][ y0 ] is equal to 1.

Table Error: Reference source not found-1 – Interpretation of dmm_mode

| dmm_mode | Method identifier |
| --- | --- |
| 0 | MODE_DMM_WFULL |
| 1 | MODE_DMM_WFULLDELTA |
| 2 | MODE_DMM_WPREDDIR |
| 3 | MODE_DMM_WPREDDIRDELTA |

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list when dmm_mode[ x0 ][ y0 ] is equal to MODE_DMM_WFULL or equal to MODE_DMM_WFULLDELTA.

dmm_delta_end_flag[ x0 ][ y0 ] equal to 0 specifies that dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign[ x0 ][ y0 ] syntax elements are not present. dmm_delta_end_flag equal to 1 specifies that dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign[ x0 ][ y0 ] syntax elements are present.

dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign_flag[ x0 ][ y0 ] are used to derive DmmDeltaEnd[ x0 ][ y0 ] as follows:

- DmmDeltaEnd[ x0 ][ y0 ] = 
( 1 − 2 *dmm_delta_end_sign_flag[ x0 ][ y0 ] ) *	( dmm_delta_end_abs_minus1[ x0 ][ y0 ] + 1)	(Error: Reference source not found-1)

dmm_dc_0_abs[ x0 ][ y0 ], dmm_dc_0_sign_flag[ x0 ][ y0 ], dmm_dc_1_abs[ x0 ][ y0 ], dmm_dc_1_sign_flag[ x0 ][ y0 ] are used to derive DcOffsetP0[ x0 ][ y0 ] and DcOffsetP1[ x0 ][ y0 ] values as follows:

- DcOffsetP0[ x0 ][ y0 ]  = ( 1 − 2 *dmm_dc_0_sign_flag[ x0 ][ y0 ] ) * dmm_dc_0_abs[ x0 ][ y0 ]	(Error: Reference source not found-2)
DcOffsetP1[ x0 ][ y0 ]  = ( 1 − 2 *dmm_dc_1_sign_flag[ x0 ][ y0 ] ) * dmm_dc_1_abs[ x0 ][ y0 ]	(Error: Reference source not found-3)

- H.8.4.2 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PbSize specifying the size of the current luma prediction block.

Table Error: Reference source not found-2 specifies the value for the intra prediction mode and the associated names.

Table Error: Reference source not found-2 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_DC |
| 2..34 | Intra_Angular |
| 35...38 | Intra_DepthPartition (used only for depth) |
| Otherwise (39, 40) | Intra_Chain (used only for depth) |

[Ed. (GT): Since the Intra_FromLuma mode has been removed in the HEVC text spec used as base for this document, the dmm mode and intra chain numbers are here decremented by 1 compared to SW.]

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

- If sdc_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows.

- If sdc_pred_mode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 1.

- Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 35.

- Otherwise, if sdc_pred_mode [ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 0.

- Otherwise, if dmm_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows:

- IntraPredMode[ xB ][ yB ] = dmm_mode[ xB ][ yB ] + 35	(Error: Reference source not found-4)

- Otherwise (dmm_flag[ xB ][ yB ] is equal to 0 and sdc_flag[ xB ][ yB ] is equal to 0), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- H.8.4.4.2.7  Specification of Intra_DepthPartition (35, 36) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 35 or 36.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]	(Error: Reference source not found-5)

  - The depth partition value derivation and assignment process as specified in subclause  is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern [ xB ][ yB ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(36) ), intraChainFlag set equal to 0, and the DC Offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

  - For x, y = 0..nT − 1, inclusive the following applies:

    - WedgeIdx[ xB +x ][ yB +y] is set equal to wedge_full_tab_idx[ xB ][ yB ].

- -

        - -

        - -

  - -

      - -

- -

          - -

          - -

- H.8.4.4.2.10 Specification of Intra_DepthPartition (37, 38) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 41 or 42.

Sample location ( xRef, yRef ) specifies the top-left sample of a reference block relative to the top-left sample of the current picture.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The flag refAvailableFlag specifying whether a neighbouring block is available for prediction is set equal to 0.

  - The luma location ( xRefA, yRefA ) is set equal ( xB, yB − 1 ) and the luma location ( xRefL, yRefL ) is set equal to the ( xB − 1, yB ).

  - For N being replace by A and L, the following applies:

    - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location of the current block ( xB, yB ) and the location ( xRefN, yRefN) as the input and the output is assigned to flag availableFlagN.

    - When refAvailableFlag is equal to 0, availableFlagN is equal to 1 and PredMode[ xRefN ][ yRefN ] is equal to MODE_INTRA and IntraPredMode[ xRefN ][ yRefN ] is in the range of 35 to 38 or 41 to 42, the following applies:

      - The flag refAboveFlag is set equal to ( N = = A )

      - The variable refWedgeIdx is set to the Wedgelet pattern index value WedgeIdx[ xRefN ][ yRefN ].

      - The Wedgelet direction derivation process as specified in subclause Error: Reference source not found is invoked with sample location ( xB, yB ), the transform size nT, the flag refAboveFlag and the reference Wedgelet index refWedgeIdx as inputs and the output is assigned to refAvailableFlag, refEndPos, refDeltaHor and refDeltaVer.

  - When refAvailableFlag is equal to 0, the following applies:.

    - The reference partition end position refEndPos and the flag refAboveFlag are derived as specified in the following:

maxSlopeAbove = 0 
			posMaxSlopeAbove = 0
			maxSlopeLeft = 0
			posMaxSlopeLeft = 0
			for( k = 0; k < nT − 1; k++ ) {
				if( Abs( p[ k+1 ][ −1 ] − p[ k ][ −1 ] ) > maxSlopeAbove ) {
					maxSlopeAbove = Abs( p[ k+1 ][ −1 ] − p[ k ][ −1 ] )
					posMaxSlopeAbove = Min( k+1, nT−2 )
				}
				if( Abs( p[ −1 ][ k+1 ] - p[ −1 ][ k ] ) > maxSlopeLeft ) {
					maxSlopeLeft = Abs( p[ −1 ][ k+1 ] - p[ −1 ][ k ] )
					posMaxSlopeLeft = Min( k+1, nT−2 )
				}
			}
			refEndPos = ( maxSlopeAbove > maxSlopeLeft ) ? posMaxSlopeAbove : posMaxSlopeLeft
			refAboveFlag = ( maxSlopeAbove > maxSlopeLeft ) ? 1 : 0

    - When availableFlagA is equal to 1 or availableFlagL is equal to 1, the following applies:

      - The luma position ( x, y ) is set equal to ( availableFlagA ? ( xRefA , yRefA ) : ( xRefL , yRefL ).

      - When PredMode[ x ][ y ] is equal to MODE_INTRA and IntraPredMode[ x ][ y ] is in the range of 2 to 34, the following applies:

      - The flag refAvailableFlag is set equal to 1.

      - The variable predModeIntra is set equal to IntraPredMode[ x ][ y ].

      - Depending on predModeIntra, the variable intraPredAng is derived as specified in 8-4.

      - Depending on predModeIntra, refAboveFlag and intraPredAng, the variables refDeltaHor and refDeltaVer are derived as specified in Table Error: Reference source not found-3.

Table Error: Reference source not found-3 – Specification of refDeltaHor, refDeltaVer 
depending on predModeIntra, refAboveFlag, intraPredAng

| predModeIntra < 18 | refAboveFlag | refDeltaHor | refDeltaVer |
| --- | --- | --- | --- |
| 1 | 1 | ( intraPredAng > 0 ) ? −32 : 32 | Abs( intraPredAng ) |
|  | 0 | 32 | − intraPredAng |
| 0 | 1 | − intraPredAng | 32 |
|  | 0 | Abs( intraPredAng ) | ( intraPredAng>0 ) ? −32 : 32 |

  - The variable resShift specifying a shift for wedgelet sampling, the variable wedgeOri specifying a wedgelet orientation. and the xS, yS, xE and yE specifying the horizontal and vertical position of the start and end of the wedgelet partition line are derived as specified in the following:

    - The variable resShift is set as specified in Error: Reference source not found with log2BlkSize equal to Log2( nT ).

    - If refAvailableFlag is equal to 1, the wedgelet direction prediction process specified in subclause Error: Reference source not found is invoked with sample location ( xB, yB ), the transform block size nT, the reference block direction flag refAboveFlag, reference partition end position refEndPos, and reference partition horizontal and vertical slope refDeltaHor and refDeltaVer as inputs and the output is assigned to wedgeOri, xS, yS, xE and yE.

    - Otherwise (refAvailableFlag is not equal to 1), the variables wedgeOri, xS, yS, xE and yE are set equal to 0.

  - The Wedgelet pattern generation process as specified in subclause Error: Reference source not found is invoked with the transform size nT, the variable, resShift, the Wedgelet orientation identifier wedgeOri, the line start horizontal position xS, the line start vertical position yS, the line end horizontal position xE and the line end vertical position yE as inputs and the output is assigned to wedgePattern[ x ][ y ]. [ Ed. (PM) Software uses wedgelet lookup list for partition derivation. (3D-I0017) ]

  - The depth partition value derivation and assignment process as specified in subclause  is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(42) ), intraChainFlag set equal to 0, and the DC Offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

  - The Wedgelet pattern index WedgeIdx[ xB ][ yB ] is derived as specified in the following:

    - For k = 0..NumWedgePattern[ Log2( nT ) ] − 1, inclusive,  the following applies:

      - The flags patIdenticalFlag and patInvIdenticalFlag are set equal to 1.

      - For x, y = 0..nT − 1, inclusive the following applies.

      - If wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ Log2( nT ) ][ k ][ x ][ y ], patIdenticalFlag is set to 0.

      - Otherwise ( wedgePattern[ x ][ y ] is equal to WedgePatternTable[ Log2( nT ) ][ k ][ x ][ y ] ), patInvIdenticalFlag is set to 0.

      - When patIdenticalFlag is equal to 1 or patInvIdenticalFlag is equal to 1, the following applies

      - For x, y = 0..nT − 1, inclusive the following applies:

      - WedgeIdx[ xB +x ][ yB +y] is set equal to k.

Table Error: Reference source not found-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | dmm_flag |  | 0 | 1 | 2 |
|  | dmm_mode | Table Error: Reference source not found-6 | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Table Error: Reference source not found-7 | 0 | 1 | 2 |
|  |  | Table Error: Reference source not found-8 |  |  |  |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Table Error: Reference source not found-9 | 0 | 1 | 2 |
|  | dmm_dc_1_abs dmm_dc_2_abs | Table Error: Reference source not found-10 | 0 | 1 | 2 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | mvp_l0_idx mpv_l1_idx | Error: Reference source not found |  | 0..1 | 2..3 |
|  | sdc_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Error: Reference source not found | 0..19 | 20..39 | 40..59 |
|  | sdc_pred_mode | Error: Reference source not found | 0..2 | 3..5 | 6..8 |

Table Error: Reference source not found-5 – Values of variable initValue for dmm_flag ctxIdx

| Initialization variable | dmm_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-6 – Values of variable initValue for dmm_mode ctxIdx

| Initialization variable | dmm_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-7 – Values of variable initValue for wedge_full_tab_idx ctxIdx

| Initialization variable | wedge_full_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

- ctxIdx

Table Error: Reference source not found-9– Values of variable initValue for wedge_predtex_tab_idx ctxIdx

| Initialization variable | dmm_delta_end_flag dmm_delta_end_abs_minus1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-10 – Values of variable initValue for dmm_dc_1_abs and dmm_dc_2_abs ctxIdx

| Initialization variable | dmm_dc_1_abs dmm_dc_2_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-11 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| dmm_flag | 0 | FL, cMax = 1 | 0 |  | 0 |
|  | 1 |  | 0 |  | 1 |
|  | 2 |  | 0 |  | 2 |
| dmm_mode | 0 | FL, cMax = 7 | 0 | Table Error: Reference source not found-6 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-6 | 1 |
|  | 2 |  | 0 | Table Error: Reference source not found-6 | 2 |
| wedge_full_tab_idx | 0 | FL, cMax = wedgeFullTabIdxBits[ log2PbSize ] (defined in Table Error: Reference source not found-12) | 0 | Table Error: Reference source not found-7 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-7 | 1 |
|  | 2 |  | 0 | Table Error: Reference source not found-7 | 2 |
|  |  | Table Error: Reference source not found-13 |  | Table Error: Reference source not found-8 |  |
|  |  |  |  | Table Error: Reference source not found-8 |  |
|  |  |  |  | Table Error: Reference source not found-8 |  |
| dmm_delta_end_flag | 0 | FL, cMax = 1 | 0 | Table Error: Reference source not found-9 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-9 | 1 |
|  | 2 |  | 0 | Table Error: Reference source not found-9 | 2 |
| dmm_delta_end_abs_minus1 | 0 | FL, cMax = 3 | 0 | Table Error: Reference source not found-9 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-9 | 1 |
|  | 2 |  | 0 | Table Error: Reference source not found-9 | 2 |
| dmm_delta_end_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | UEG0 [Ed. (GT) To be specified] | 0 | Table Error: Reference source not found-10 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-10 | 1 |
|  | 2 |  | 0 | Table Error: Reference source not found-10 | 2 |
| dmm_dc_1_sign_flag dmm_dc_2_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| ic_flag | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 2 |  | 0 | Error: Reference source not found | 1 |
| mvp_l0_idx mvp_l1_idx | 1 | FL, cMax = 1 + iv_mv_pred_flag[ nuh_layer_id ] | 1 | Error: Reference source not found | 0 |
|  | 2 |  | 1 | Error: Reference source not found | 2 |
| sdc_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| sdc_residual_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 2 |
|  | 2 |  | 0 | Error: Reference source not found | 4 |
| sdc_residual_sign_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 2 |
|  | 2 |  | 0 | Error: Reference source not found | 4 |
| sdc_residual_abs_minus1 | 0 | FL, cMax = dltFlag ? Floor( Log2( num_depth_values_in_dlt[ nuh_layer_id ] ) : BitDepthY | 9 | Error: Reference source not found | 0 |
|  | 1 |  | 9 | Error: Reference source not found | 20 |
|  | 2 |  | 9 | Error: Reference source not found | 40 |
| sdc_pred_mode | 0 | U [Ed. (GT): in implementation 0 and 1 are switched. ] [Ed. (GT): Could be TU, cMax = 3] | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |

Table Error: Reference source not found-12 – Values of wedgeFullTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 7 | 10 | 11 | 11 | 13 |

-]

Table Error: Reference source not found-14 – Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| dmm_flag | 0 | na | na | na | na | na |
| dmm_mode | 0 | 0 | 0 | 0 | 0 | 0 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_delta_end_flag | 0 | na | na | na | na | na |
| dmm_delta_end_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| mvp_l0_idx, mvp_l1_idx | 0 | 1 | na | na | na | na |
| sdc_flag | 0,1,2 (subclause G.9.3.3.1.1) | na | na | na | na | na |
| sdc_residual_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_sign_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_abs_minus1 | 0,10 (subclause Error: Reference source not found) |  |  |  |  |  |
| sdc_pred_mode | 0 | 1 | 2 | na | na | na |

Table G-15 – Values of variable initValue for dmm_flag ctxIdx

| Initialization variable | dmm_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-16 – Values of variable initValue for dmm_mode ctxIdx

| Initialization variable | dmm_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-17 – Values of variable initValue for wedge_full_tab_idx ctxIdx

| Initialization variable | wedge_full_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

--ctxIdx