| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0124 |
| --- | --- |

| Title: | 3D-CE6.h related: SDC with DLT in intra slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Ho-Cheon Wey, Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Simplified depth coding (SDC) with depth lookup table (DLT) was integrated on the current 3D-HTM (JCT3V-B0036). SDC predicts a depth block with DMM1, DC, and Planar modes of a 2Nx2N partition. When the block is coded with SDC, it signals an index with DLT instead of a residual value. DLT is a mapping table from a residual value to an index, and generated once based on the first several depth frames. DLT is transmitted to a decoder for the inverse mapping from an index to a value in a sequence parameter set (SPS) level. At the last meeting, DLT signalling in intra slices instead of SPS and its simplification was proposed (JCT3V-C0093 and JCT3V-C0142). In addition, it was proposed that DLT is generated in every intra slice to avoid the performance degradation by scene changes in the middle of a sequence. In this proposal, SDC with DLT is only used in intra slices. If DLT is generated per every intra slice, only using DLT in intra slices is more reasonable. Experimental result shows that the proposed method has no coding loss.

# Proposed SDC with DLT

With SDC, a depth block is intra predicted by DMM1, DC, and Planar modes of a 2Nx2N partition. Since the partition size of SDC is always 2Nx2N, only one flag is sent to a decoder at a CU level to indicate whether the block is coded with SDC or not. Before coding, a residual value is mapped to an index by DLT. It is generated once by analyzing the first several depth frames and signalled in SPS. In order to generate more robust DLT to scene changes in the middle of a sequence, it was proposed that DLT is generated in every intra slice and then signalled in intra slices. Also, an efficient coding approach for DLT was proposed. If DLT is generated, coded, and signalled in every intra slice, it is reasonable to employ SDC with DLT only in intra slices. Therefore, in the proposed method, SDC with DLT is used only in intra slices.

# Test Results

Test results are summarized in Tables 1 and 2. Table 1 is the performance of the proposed method on the top of the current SW, which means that DLT is generated once with the first several depth frames and signalled in SPS. Table 2 is the performance of the proposed method on the top of SW integrated based on JCT3V-C0093 and JCT3V-C0142, which means that DLT is generated, coded and signalled in every intra slice. The result shows that the proposed method in both cases has no coding loss.

Table 1. Performance of the proposed method on the top of the current SW

Table 2. Performance of the proposed method on the top of SW integrated based on JCT3V-C0093 and JCT3V-C0142

# Conclusion

The proposed method enables SDC with DLT only in intra slices, in the context of the previous proposals (JCT3V-C0093 and JCT3V-C0142). Since the modifications are very simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).