| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0135 |
| --- | --- |

| Title: | 3D-CE5.h related: Unification of disparity vector rounding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Sehoon Yea | Email: | junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This proposal unifies rounding operation of disparity vector with respect to rounding offset. The modification consist of adding rounding offset for 3 parts; DoNBDV, B-VSP, IC process. Experimental results showed an average coding gain of 0.1% compared to HTM-6.0.

# Introduction

In HTM-6.0, a disparity vector (DV) is used to find a virtual depth block or a corresponding texture block in reconstructed base view. For example, first, a DV derived by NBDV process is exploited to find a virtual depth block for depth-oriented NBDV refinement (DoNBDV) process. Second, a refined DV from DoNBDV process is used to find a corresponding PU from which motion vector for inter-view motion prediction (IVMP) is derived. Third, DV in B-VSP process is utilized to get a corresponding virtual depth block to select PU-wise max depth value. Finally, when parameters of illumination compensated sample prediction (IC) are calculated, DV is used to get reference samples.

The DV for various inter-view predictions has quarter-pel resolution. So, to find a virtual depth block or a corresponding texture block needs rounding operation on full-pel resolution. In general, when quarter-pel to full-pel converting, it is mapped to the nearest full-pel position with rounding offset. However, both software and draft specification round either with offset or without offset for some rounding operations.

# Proposed

In this contribution, we propose unification of DV rounding with rounding offset. In HTM-6.0, DV rounding for DoNBDV, B-VSP, and derivation of IC parameters is conducted without rounding offset. However, DV rounding for IVMP is conducted with rounding offset. So, we apply DV rounding with rounding offset for NBDV, B-VSP, and derivation of IC parameters.

In this contribution, DV rounding for DoNBDV and B-VSP is modified by H-291 and H-292 as follows:

Let refDepPels be an array of reconstructed depth samples of the depth view component with ViewIdx equal to refViewIdx. The luma location (xTL, yTL) of top-left luma sample of refDepPels is derived by xTL = Clip3( 0, pic_width_in_luma_samples – nPSW – 1, xP + ( (mvDisp[ 0 ] +2) >> 2 ) ) (H 291) yTL = Clip3( 0, pic_height_in_luma_samples – nPSH – 1, yP + ( (mvDisp[ 1 ] +2) >> 2 ) ) (H 292)

DV rounding for derivation of IC parameters is modified by H-193 and H-194 as follows:

- The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as - xRLX = xC + ( (mvLX[ 0 ] + (cIdx ? 4 : 2)) >> (2 + (cIdx ? 1 : 0)) ) (Error: Reference source not found-1) - yRLX = yC + ( (mvLX[ 1 ] + (cIdx ? 4 : 2)) >> (2 + (cIdx ? 1 : 0)) ) (Error: Reference source not found-2)

There is no change for DV rounding for IVMP process which is conducted by H-161 and H-162 as follows:

The reference layer luma location ( xRef, yRef ) is derived by - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) ) (Error: Reference source not found-3) - yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 )) (Error: Reference source not found-4)

# Experimental results

The results indicate that the proposal has a slightly coding gains compared to HTM-6.0 using common test conditions [1]. Encoding and decoding time are measured on a Linux cluster and is expected to be similar as for HTM-6.0.

Table 1 Performance of proposed DV rounding offset compared to HTM-6.0

|  | video 0 | video 1 | video 2 | video PSNR /video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% | 99.5% | 98.0% |
| Kendo | 0.0% | -0.5% | 0.0% | -0.1% | -0.1% | -0.1% | 99.7% | 100.6% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 99.9% | 106.3% |
| GhostTownFly | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.7% | 99.7% |
| PoznanHall2 | 0.0% | 0.3% | -0.2% | 0.0% | -0.1% | 0.0% | 99.7% | 100.5% |
| PoznanStreet | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 99.9% | 100.2% |
| UndoDancer | 0.0% | -0.4% | 0.2% | 0.0% | 0.0% | -0.4% | 99.9% | 99.1% |
| 1024x768 | 0.0% | -0.3% | 0.0% | -0.1% | 0.0% | -0.1% | 99.7% | 101.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.8% | 99.9% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 99.8% | 100.6% |

Table 2 Performance of DV rounding offset for DoNBDV compared to HTM-6.0

|  | video 0 | video 1 | video 2 | video PSNR /video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 98.1% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 102.1% |
| Newspapercc | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | 99.6% | 107.5% |
| GhostTownFly | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 102.3% |
| PoznanHall2 | 0.0% | -0.7% | 0.0% | -0.2% | -0.2% | -0.1% | 99.7% | 99.0% |
| PoznanStreet | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 99.7% |
| UndoDancer | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 100.4% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.6% | 102.6% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 100.4% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 101.3% |

Table 3 Performance of DV rounding offset for B-VSP compared to HTM-6.0

|  | video 0 | video 1 | video 2 | video PSNR /video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 95.2% |
| Kendo | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 99.7% | 96.2% |
| Newspapercc | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 99.8% | 98.5% |
| GhostTownFly | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.6% | 98.9% |
| PoznanHall2 | 0.0% | -0.3% | -0.5% | -0.2% | -0.2% | -0.1% | 99.9% | 100.2% |
| PoznanStreet | 0.0% | 0.3% | 0.1% | 0.1% | 0.0% | 0.0% | 100.1% | 104.8% |
| UndoDancer | 0.0% | -0.4% | 0.1% | 0.0% | 0.0% | -0.1% | 100.1% | 94.5% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 96.7% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 99.6% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 98.4% |

Table 4 Performance of DV rounding offset for IC parameters derivation compared to HTM-6.0

|  | video 0 | video 1 | video 2 | video PSNR /video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 100.0% | 101.8% |
| Kendo | 0.0% | -0.4% | -0.2% | -0.1% | -0.1% | -0.1% | 100.1% | 96.9% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 100.1% | 101.6% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 98.6% |
| PoznanHall2 | 0.0% | 0.0% | -0.4% | -0.1% | -0.1% | 0.0% | 100.2% | 94.8% |
| PoznanStreet | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 95.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 98.0% |
| 1024x768 | 0.0% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 100.1% | 100.1% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 96.8% |
| average | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100.2% | 98.2% |

# Conclusion

This contribution proposes unification of disparity vector rounding with offset. The proposal achieved 0.1% gains under common test conditions.

# Reference

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-B1100, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).