| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0136 |
| --- | --- |

| Title: | 3D-CE6.h related: Improvement of SDC coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Hongbin Liu, Sehoon Yea | Email: | junghak.nam@lge.com hongbin.liu@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, modification of depth value reconstruction process is proposed to improve simplified depth coding (SDC) coding. Reconstructed samples in SDC are calculated by adding SDC residuals with intra predicted samples in themselves instead of DC of predicted samples. By utilizing to predicted samples, fidelity of reconstructed samples could be improved. The experimental results show that the proposed method achieved 0.2% and 0.2% BD-Rate gains for total video and synthesized views in CTC, respectively. The results also yield 0.2% and 0.1% BD-Rate gains for total video and synthesized views in AI, respectively.

# Introduction

SDC for depth intra coding was adopted into 3D-HEVC [1]. SDC bypasses transformation/quantization process and represents a reconstructed block with DC. The reconstructed block can be reconstructed with or without depth lookup table (DLT). For example, when a PU block is reconstructed by SDC without DLT, reconstructed samples are calculated by adding SDC residuals and DC of intra predicted samples. Otherwise, reconstructed samples based on DLT are obtained from DLT index which is calculated by adding DLT index residual with DLT index for predicted samples. However, it is the same as adding SDC residuals with DC of intra predicted samples in terms of implementation.

# Proposed

This contribution proposes a method to improve the SDC coding efficiency by using predicted samples generated by intra prediction. In current 3D-HEVC, DC of intra predicted samples are exploited to obtain reconstructed samples in depth value reconstruction process. However, each of predicted samples for DC and Planar modes in SDC is not equal to DC of predicted samples. So, we propose using predicted samples instead of DC of predicted samples. The proposed method can improve quality of reconstructed samples by maintaining shape of the predicted samples. The predicted samples are already available to calculate DC of predicted samples in the conventional SDC coding. Therefore, there is no additional memory access for modification in this proposal. Note that each of predicted samples for DMM1 mode is  equal to DC of predicted samples. So, there is no effect for DMM1 mode in SDC.

# Modification of decoding process

H.8.4.4.3	Depth value reconstruction process

Inputs to this process are:

- a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nT specifying the prediction size

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1

- a variable intraPredMode specifying the prediction mode of the current prediction block

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = −1..2*nT−1.

The values of the prediction samples predSamples[ x ][ y ], are derived as specified by the following ordered steps.

  - Depending on intraPredMode the array wedgePattern[ x ][ y ] with x, y =0..nT−1 specifying the binary segmentation pattern is derived as follows.

    - If intraPredMode is equal to Intra_DepthPartition( 35 ), the following applies.

        - wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]

    - Otherwise ( intraPredMode is not equal to Intra_DepthPartition( 35 ) ), the following applies.

      - For x, y = 0..nT−1 wedgePattern[ x ][ y ] is set equal to 0.

  - The variable log2SubSample is set equal to ( nT < 32 ) ? 0 : 1.

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		dcPred[ p ] = ( numPred > 0 ) ? ( sumPred / numPred ) : 0

[ Ed. (GT): Is this averaging necessary for DMM or DC mode? ]

  - For x, y = 0..nT−1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following.

    - If dlt_flag[ nuh_layer_id ] is equal to 0, the following applies:

        - resSamples[ x ][ y ] = dcPred[ wedgePattern[ x ][ y ] ] +	
	SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]

    - Otherwise ( dlt_flag[ nuh_layer_id ] is equal to 1 ), the following applies.

        - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-1)
dltIdxResi = SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-2)
resSamples[ x ][ y ] = Idx2DepthValue[ dltIdxPred + dltIdxResi ]	(Error: Reference source not found-3)

    - If dlt_flag[ nuh_layer_id ] is equal to 0, the following applies:

        - resSamples[ x ][ y ] = predSamples[ x ][ y ] +	
	SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]

    - Otherwise ( dlt_flag[ nuh_layer_id ] is equal to 1 ), the following applies.

        - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-4)
dltIdxResi = SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-5)
resSamples[ x ][ y ] = predSamples[ x ][ y ] +
     (Idx2DepthValue[ dltIdxPred + dltIdxResi ] - dcPred[ wedgePattern[ x ][ y ] ] )	(Error: Reference source not found-6)

# Experimental results

The results indicate that the proposal has a coding gains compared to HTM-6.0 using common test conditions [2]. Encoding and decoding time are measured on a Linux cluster and is expected to be similar as for HTM-6.0.

Table 1 Performance of the proposed method compared to HTM-6.0 RA

|  | video 0 | video 1 | video 2 | video PSNR /video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | -0.3% | -0.1% | 101.3% | 98.0% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.0% | -0.5% | -0.3% | 101.4% | 98.6% |
| Newspapercc | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% | 100.8% | 99.4% |
| GhostTownFly | 0.0% | -0.6% | -0.2% | -0.1% | -0.4% | -0.4% | 100.8% | 98.4% |
| PoznanHall2 | 0.0% | -0.3% | 0.2% | 0.0% | -0.1% | 0.1% | 100.9% | 97.5% |
| PoznanStreet | 0.0% | 0.0% | 0.1% | 0.0% | -0.3% | -0.3% | 100.4% | 97.6% |
| UndoDancer | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.4% | 100.8% | 97.7% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | -0.3% | -0.1% | 101.2% | 98.6% |
| 1920x1088 | 0.0% | -0.3% | 0.0% | 0.0% | -0.2% | -0.2% | 100.7% | 97.8% |
| average | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% | 100.9% | 98.2% |

Table 2 Performance of the proposed method compared to HTM-6.0 AI

|  | video 0 | video 1 | video 2 | video PSNR /video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 103.4% | 98.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 103.7% | 96.1% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 102.4% | 100.8% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 97.7% | 100.7% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 104.7% | 99.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 99.4% | 98.7% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.0% | 99.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 103.2% | 98.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 100.2% | 99.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 101.5% | 99.1% |

# Conclusion

In this contribution, modification of depth value reconstruction process is proposed to improve SDC coding. To use predicted samples instead of DC of predicted samples is proposed for Planar mode. The experimental results show 0.2% (total videos), 0.2% (synthesized videos) BD-rate in in CTC and 0.2% (total videos), 0.1% (synthesized videos) BD-rate in all intra condition when compared to HTM-6.0.

# Reference

- Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, JCT-3V 2nd Meeting, Shanghai, CN, 13-19 Oct. 2012.

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-C1100, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).