| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0138 |
| --- | --- |

| Title: | 3D-CE2.h related: Simplified DV derivation for DoNBDV and BVSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Na Zhang, Jicheng An, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, na.zhang, jicheng.an, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-HEVC, two different depth-to-DV conversion methods are performed for depth oriented neighbor block disparity vector (DoNBDV) and backward view synthesis prediction (BVSP). A simplified depth-to-DV conversion is proposed in this contribution. The proposed conversion method uses the maximum among four corners of the depth samples to extract a disparity vector from the virtual depth block in DoNBDV and BVSP. The proposed method not only unifies the depth-to-DV conversion method between DoNBDV and BVSP but also aligns the depth-to-DV conversion method with 3D-AVC. The experimental results reportedly show that the proposed depth-to-DV conversion has no overall coding efficiency loss and introduces minor coding efficiency gain in the synthesized view, and the computational complexity and memory access of depth samples are reduced.

# Introduction

In current 3D-HEVC [1], if the decoded depth of the base view is already available while coding the texture of a dependent view, the disparity derivation needed for the coding of the texture of the dependent view might be improved by utilizing the depth map of the base view. A disparity vector can be extracted by using an estimated disparity vector from neighbor block disparity vector (NBDV) to retrieve a depth block from base view. The values in the depth block can be converted to a refined disparity vector for inter-view motion vector, disparity vector predictor, or backward view synthesis prediction [4][5].

The refined disparity vector (depth oriented neighbor block disparity vector, DoNBDV) is used in AMVP and merge mode [4] to generate an inter-view motion vector predictor or disparity vector predictor for a prediction unit (PU). A simplified depth-to-DV conversion was proposed to reduce the access of the depth block and introduce almost no coding efficiency loss [6].

The backward view synthesis prediction (BVSP) [5] uses a similar scheme to extract its disparity vector for each sub-prediction unit (sub-PU). The steps in BVSP which fetch a depth block from base view are similar with the retrieval of the virtual depth block in AMVP and merge mode. The fetched depth block would have the same size of the current prediction unit (PU), and it will then be used to do a sub-PU level BVSP for the current PU. For example, the warping operation may be performed at a sub-PU level precision, like 2x2 or 4x4 blocks. A maximum depth value is picked for a sub-PU block and used for warping all the pixels in the sub-PU block.

The depth-to-DV conversion in DoNBDV uses a maximum among five points scheme within a PU. And the depth-to-DV conversion in BVSP uses the maximum within a sub-PU block. Similar tools such as DMVP and BVSP in 3D-AVC [3] already utilized the maximum among four corners scheme for the depth-to-DV conversion. In this contribution, the maximum among four corners scheme is proposed to unify the conversion method and reduce the memory access of the depth samples in DoNBDV and BVSP.

# Algorithm description

The maximum among four corners scheme is proposed to unify the conversion method and reduce the memory access of the depth samples in DoNBDV and BVSP.

As shown in Fig. 2(a), the maximum among five points scheme is used in the current 3D-HEVC. The maximum depth value among four corners in a PU is proposed to be used for DoNBDV as shown in Fig. 2(b). The refinement of the disparity vector used by AMVP and merge mode is simplified from using maximum among five points to maximum among four corners.

As shown in Fig. 3(a), the maximum among the whole sub-PU block is used in the current 3D-HEVC. The maximum depth value among four corners in a sub-PU block is proposed to be used for BVSP to warp all the samples in the sub-PU block as shown in Fig. 3(b). In the current software of 3D-HEVC, the sub-PU block size is 4x4 by default.

Fig. 2.  Disparity vector extraction by maximum depth value of four corner samples in the virtual depth block (a) Original scheme (b) Proposed scheme

Fig. 3.  Disparity vector extraction by maximum depth value of four corner samples in the Sub-PU of a virtual depth block (a) Original scheme (b) Proposed scheme

In DoNBDV and BVSP, the maximum depth value among four corners is proposed to be used in the depth-to-DV conversion. In 3D-AVC [3], the maximum among four corners is also used for all the depth-to-DV conversion. This unifies the conversion method and also simplifies the original maximum among the five points and maximum of the sub-PU block scheme. The proposed method also aligns with the method adopted in 3D-AVC.

# Experiment results

The proposed simplification and unification are integrated into 3D-HTM 6.0 [1], and the simulations are run under the common test conditions [2]. Two tests are carried out: test 1 enables the maximum among four corners only in DoNBDV; test 2 enables the maximum among four corners both in DoNBDV and BVSP. The results of test 1 and test 2 are illustrated in Table 1 and Table 2 respectively. The experimental results show that this proposed simplification brings no coding loss while the computational complexity and memory access bandwidth are reduced.

Table 1.  Coding results for enabling maximum among four corners in DoNBDV

Table 2.  Coding results for enabling maximum among four corners in DoNBDV and BVSP

# Conclusion

A simplified depth-to-DV conversion is proposed in this contribution. The proposed conversion method uses the maximum among four corners of the depth samples to extract a disparity vector from the virtual depth block in DoNBDV and BVSP. The proposed method not only unifies the depth-to-DV conversion method among DoNBDV and BVSP but also aligns the depth-to-DV conversion method with 3D-AVC. The experimental results reportedly show that the proposed depth-to-DV conversion introduces some coding gain in the synthesized view, and the computational complexity and memory access of depth samples are reduced.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1005, January, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “3D-CE2.a related: MB-level depth-to-DV conversion in ATM,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0134, January, 2013.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

- D. Tian, F. Zou, A. Vetro, “CE1.h: Backward view synthesis prediction using neighbouring blocks,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0152, January, 2013.

- J. Jung, J. Sung, S. Yea, “3D-CE1.h related: Adaptive method for depth-oriented neighboring block disparity vector,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0112, January, 2013.