| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 April 2013 | Document: JCT3V-D0141 |
| --- | --- |

| Title: | 3D-CE6.h: Concatenate binarization for residual index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Sehoon Yea, | Email: | jin78.heo@lge.com, sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents a depth intra coding method for Simplified Depth Coding (SDC). The proposed method uses the concatenation of basic binarization schemes for residual index coding. The concatenate binarization uses only three context models to code residual index and it can reduce the number of context models for residual index from 20 to 1. Under both common test conditions and all-intra configuration this method provides 0.1% BD-rate gain with no impact on the encoder/decoder complexity.

# Introduction

This contribution represents a reduction of the number of CABAC contexts for residual index coding and was proposed to the 3rd JCT-3V meeting in JCT3V-C0067 [1]. Since the Simplified Depth coding (SDC) descrited in JCT3V-B0036 [2] uses a fixed length binarization to code the residual index generated by a depth lookup table (LUT), it required the number of CABAC contexts for residual index coding. In this contribution, a concatenbation of the unary binarization for prefix part and the fixed length binarization for suffix part is applied to signal the residual index. Since the unary binarization can be easily skewed the distribution of a certain bin value, the proposed method can remove the number of contexts for residual index coding.

# Proposed Method

Instead of coding a residual depth value directly, it is mapped to its corresponding residual index in a depth lookup table which is constructed by analyzing a certain number of frames of the input sequence. The calculated residual index is coded with a sign flag and a magnitude of the residual index using the fixed length (FL) binarization without transform and quantization process.

Typically FL binarization is applied to symbol with a nearly uniform distribution. However, since the statistical distribution of the residual index is highly skewed on small residual index, as shown in Figure 1, it is not suitable for coding the residual index. Considering this observation, this contribution uses the unary code to generate the binary sequence of the residual index. Consequently, this proposal assigns shorter codeword to a small residual index that occurs more frequently instead of using uniformly assigned n bits for all residual indexes. Therefore, the unary binarization generates smaller number of bins to encode by the entropy coder than the FL binarization.

Figure 1 Statistical distribution of residual index, QP=25

However, the statistical distribution of some sequences; Newspaper, GT_Fly, and Poznan_Street have the different statistical distribution of unary binarization, as shown in Figure 1. The statistical distribution of them is quite wide; note the large variation and wide tails. Therefore, this contribution proposes a concatenation of basic binarization schemes; unary binarization and FL binarization. For residual index binarization, the concatenate binarization is constructed by the prefix part of the unary code and the suffix part of the FL code.

Since the probability of bin ‘0’ and ‘1’ in the binary sequence generated by FL binarization has random characteristics, several adequately designed sets of context models for bin at each position are used. As a result, 20 context models are employed to code bin at each position of the binary sequence. However, in the prefix part generated by the unary binarization, it is easy to expect the bin value because most of bins except for the last bin have the same bin value. Therefore, only one kind of context model for residual index coding is applied to the prefix part.

Since the suffix part of the concatenate binarization uses the fixed length code and the occurrence probability of the suffix part is lower, context modeling seems to be unnecessary. Therefore, contexts to code bin of the suffix part are removed.

Table 1 Comparison of the number of context models for residual index

# Experimental Results

According to the CE6.H description [3] and the common test conditions [4], three tests are conducted under random access (CTC) and intra-only configuration. First test is the result of the proposed concatenate binarization method, and second test is the comparison result for JCT3V-D0032 [5] and the concatenate binarization based on JCT3V-D0032 which means that JCT3V-D0032 replaces residual coding scheme with concatenate binarization. The last test is the result of concatenate binarization based on JCT3V-D0032.

## Test 1: Results of concatenate binarization

Table 2 Result for 3-view scenario under CTC

Table 3 Result for 3-view scenario under intra-only configuration

## Test 2: Comparison results for JCT3V-D0032 and concatenate binarization based on JCT3V-D0032

Table 4 Result for 3-view scenario under CTC

Table 5 Result for 3-view scenario under intra-only configuration

## Test 3: Results of concatenate binarization based on JCT3V-D0032

Table 6 Result for 3-view scenario under CTC

Table 7 Result for 3-view scenario under intra-only configuration

# Working Draft Modifications

## Modifications to Coding Unit

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( vps_dmm_flag[ nuh_layer_id ] && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| wedge_predtex_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + j ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + j ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + j ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + j ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + j ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + j ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ x0 + i ][ y0 + j ][ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && !sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ) ? 2 : 1 |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_prefix[ x0 ][ y0 ][ i ] | ae(v) |
| if ( prefix_count = = PrefixThreshold ) |  |
| sdc_residual_suffix[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

sdc_residual_prefix[ x0 ][ y0 ][ i] and sdc_residual_suffix[ x0 ][ y0 ][ i ] specify the value of a residual index.

sdc_residual_prefix[ x0 ][ y0 ][ i ], and sdc_residual_suffix[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

The parsing process for these syntax elements begins with reading the bits starting at the current location in the bitstream up to PrefixThreshold and including the first zero bit, and counting the number of leading bits (prefix_count) that are equal to 1.

PrefixThreshold = ( (num_depth_values_in_dlt * 3) >> 2 )

If prefix_count is not equal to PrefixThreshold, the parsing process for SdcResidual[ x0 ][ y0 ][ i ] terminates. Otherwise, the parsing process for sdc_residual_suffix[ x0 ][ y0 ][ i ] begins with reading the bits starting at the current location in the bitstream up to BitsPerSuffix and suffix_value is derived.

BitsPerSuffix = ceil( Log2( num_depth_values_in_dlt – prefixThreshold) )

–	If prefix_count is not equal to PrefixThreshold

SdcResidual[ x0 ][ y0 ][ i ] = ( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( prefix_count + 1 )

–	Otherwiese prefix_count is equal to PrefixThreshold

SdcResidual[ x0 ][ y0 ][ i ] = ( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( prefix_count + suffix_value + 1 )

# Decoding Process

## Decoding Process for Simplified Depth Coding

### Initialization process for context variables

Table G-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| coding_unit() | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
|  | sdc_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Table G-2 | 0..19 | 20..39 | 40..59 |
|  | sdc_residual | Table G-2 | 0 | 1 | 2 |
|  | sdc_pred_mode | Error: Reference source not found | 0..2 | 3..5 | 6..8 |

Table G-2 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 255 | 255 | 64 | 0 | 2 | 0 | 64 | 0 | 6 | 0 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 67 | 0 | 8 | 0 | 61 | 0 | 7 | 0 | 47 | 0 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 243 | 255 | 12 | 0 | 14 | 0 | 33 | 0 | 243 | 255 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 2 | 0 | 66 | 0 | 0 | 0 | 63 | 0 | 1 | 0 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| initValue | 12 | 0 | 50 | 0 | 14 | 0 | 27 | 0 | 0 | 0 |
|  | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 239 | 255 | 14 | 9 | 27 | 0 | 0 | 0 | 239 | 255 |

Table G-38 – Values of variable initValue for sdc_residual ctxIdx

| Initialization variable | sdc_residual |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 155 | 155 | 155 |

# Conclusion

This contribution proposes concatenate binarization scheme for residual index coding in Simplified Depth Coding (SDC). The improved scheme reduces the number of contexts for residual index from 20 to 1, besides achieves 0.1% and 0.1% gains under common test conditions and all-intra configuration, respectively.

# Reference

- Jin Heo, Sehoon Yea, “3D-CE6.h: Improved Simplified Depth Coding,” JCT3V-C0067, Geneva, CH, Jan. 2013.

- Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, Shanghai, CN, Oct. 2012.

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-A1100, Stockholm, Sweden, July 2012.

- Philipp Merkle, “Description of Core Experiment 6 (CE6) on Depth Intra Coding,” JCT3V-A1106, Stockholm, July 2012.

- Fabian Jäger, Jin Heo, “CE6.h: results on CABAC context reduction for simplified depth coding (SDC),” JCT3V-D0032, Incheo, KR, Apr. 2013.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).