| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 April 2013 | Document: JCT3V-D0142 |
| --- | --- |

| Title: | 3D-CE6.h related: Improved DLT coding method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Sehoon Yea, | Email: | jin78.heo@lge.com, sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents an improvement of depth lookup table (DLT) used in simplified depth coding (SDC) for depth map coding. In the current SDC, DLT was employed to encode the residual index and all valid depth values in DLT are encoded with Exp-Golomb code at the sequence parameter set (SPS) level. However since each value is coded in itself without considering the correlation between sequential values, the number of bits used to code a valid depth value in DLT is redundant. Therefore, this contribution proposes an improved DLT coding method. Under common test condition this method provides 0.0% BD-rate gain with no impact on the encoder/decoder complexity.

# Introduction

A depth lookup table (DLT) in simplified depth coding (SDC) was proposed to 2rd JCT-3V meeting in JCT3V-B0036 [1]. In SDC, by skipping the transform and quantization, residual indices are directly encoded by the DLT. The DLT is constructed based on an initial analysis of the input depth map and the value in the DLT is encoded with Exp-Golomb code. However, since the sequential values in the DLT are similar in general, the difference between the consecutive values in the DLT is encoded with Exp-Golomb code.

# Proposed Method

In order to improve DLT coding, a differential pulse code modulation (DPCM) method is proposed. If the syntax use_dlt_flag is equal to 1, DLT is used and the syntax num_depth_values_in_dlt specified the number of elements in the DLT is encoded. Both use_dlt_flag and num_depth_values_in_dlt coding processes are exactly same as the conventional methods. The proposed method only modified the depth value coding method in the DLT.

First, min_dlt_depth_value which is the smallest value in the DLT is encoded. Second, diff_dlt_depth_value_minus1 which means the differential value between the consecutive values in the DLT is encoded as many as (the number of elements – 1) in the DLT. These two syntaxes relative with the depth value coding in the DLT are encoded with Exp-Golomb code.

# Experimental Results

According to the CE6.H description [2] and the common test conditions [3], test is conducted under random access (CTC).

Table 3 Result for 3-view scenario under CTC

# Working Draft Modifications

## Modifications to Coding Unit

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 |  |
| ViewId[ layerId ] = layerId >> 1 |  |
| view_id[ layerId ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| if ( j = = 0 ) |  |
| min_dlt_depth_value[ layerId][ j ] | ue(v) |
| else |  |
| delta_dlt_depth_value_minus1[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

min_dlt_depth_value[ layerId ][ j ] specified the smallest value in the depth lookup table for depth view components with layer_id equal to layerId.

delta_dlt_depth_value_minus1[ layerId ][ j ] + 1 specified the difference between the dlt_depth_value[ j − 1 ] and dlt_depth_value[ j ] in the depth lookup table for depth view components with layer_id equal to layerId.

# Decoding Process

## Decoding Process for a depth lookup table

For j = 0 .. num_depth_values_in_dlt – 1 the elements in Idx2DepthValue ares derived as follows.

- When j = 0, Idx2DepthValue[ j ] is set equal to min_dlt_depth_value[ 0 ].

- When j = 1 .. num_depth_values_in_dlt – 1, Idx2DepthValue[ j ] = Idx2DepthValue [ j – 1] + delta_dlt_depth_value_minus1 [ j ] + 1

# Conclusion

This contribution proposes an improved depth lookup table (DLT) scheme for Simplified Depth Coding (SDC). The improved scheme achieves 0.0% gains under common test condition.

# Reference

- Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, Shanghai, CN, Oct. 2012.

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-A1100, Stockholm, Sweden, July 2012.

- Philipp Merkle, " Description of Core Experiment 6 (CE6) on Depth Intra Coding", JCT3V-A1106, Stockholm, July 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).