| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20-26 Apr. 2013 | Document: JCT3V-D0147 |
| --- | --- |

| Title: | CE6.h: Results on Signalling of Depth Lookup Table |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ilsoon Lim Hocheon Wey Jin Young Lee Byeongdoo Choi Chanyul Kim | Tel: Email: | ilsoon.lim@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In this proposal, we moved SPS signalling to intra slice-layer signalling in Depth Lookup Table in 3DV-HTM. The current DLT is sent in the Sequence Parameter Set (SPS) but slice layer signalling is more appropriate. Because, the current DLT in SPS is decided through the initial analysis and the DLT is applied to the whole sequence. The initial analysis is not good solution to DLT construction in the varying scene and can cause coding loss in the various test sequences. In this proposal, we make and send new DLT either at every intra slice.

The DLT intra slice-layer signalling was implemented and the test results in CTC show 0.0% (video PSNR / video bitrate), 0.0% (video PSNR / total bitrate), 0.0% (synth PSNR / total bitrate) BD-rate. The test results in all intra condition show 0.0% (video PSNR / video bitrate), 0.3% (video PSNR / total bitrate), 0.3% (synth PSNR / total bitrate) BD-rate when compared with 3DV-HTM ver. 6.0 [2]. The coding loss in all intra conditions is negligibly small compared to DLT’s own coding gain.

Finally, it is more appropriate for the intra slice-layer signalling to be adopted into 3DV-HTM.

# Introduction

In this section, we will overview the present DLT and then explain the improvement. The DLT is constructed based on an initial analysis of depth map histogram and is then coded in the SPS. DLT is not longer optional and should be mandatory, because the coding loss of SDC without DLT is significant and we had better disable both SDC and DLT than SDC enabling without DLT.

To make the DLT, the first 24 frames (default setting) or some number of frames is analyzed and then the constructed DLT is written to SPS. Such initial analysis is prune to the scene change where the histogram is totally different. Fig. 1 shows the present DLT construction and usage. The DLT is decided at the beginning at the sequence and is applied to all scenes.

To make the DLT robust to varying scene, we propose the intra-slice layer signalling instead of the SPS signalling. In the proposal, DLT is constructed every intra-slice and is coded at the intra slice header. In the P- and B- slice, we don’t construct and code the new DLT. Instead, we use the DLT already constructed in intra slice. Since the neighboring frames are highly similar in histogram analysis, we don’t need to make and send the similar DLT at every frame.

(a) The present DLT signalling

(b) The DLT intra-slice signalling

Fig. 1. The comparison of the present DLT and DLT intra-slice signalling

# Test results – Intra slice-layer signalling

Tests were based on the common test condition and all-intra conditions for the 3 view configuration [2]. The results are summarized in Table 1(CTC) and Table 2(All-intra).

## Common test conditions

Table 1 BD rate results for 3-view case in common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 100.5% | 100.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 100.3% | 99.6% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 100.9% | 100.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.5% | 99.8% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.4% | -0.2% | -0.2% | -0.2% | 99.9% | 99.4% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 101.8% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 100.4% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 100.6% | 100.3% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 100.3% | 100.3% |
| average | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100.4% | 100.3% |

## All intra conditions

Table 2 BD rate results for 3-view case in all intra conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 0.6% | 99.3% | 94.6% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 0.4% | 101.2% | 101.5% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.4% | 100.8% | 100.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.0% | 99.8% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 0.5% | 97.7% | 95.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 96.9% | 92.7% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 98.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 0.4% | 100.4% | 99.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 98.8% | 96.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.3% | 99.5% | 97.6% |

# Conclusion

In this proposal, we moved SPS signalling to intra slice-layer signalling in Depth Lookup Table in 3DV-HTM. The current DLT is sent in the Sequence Parameter Set (SPS) but slice layer signalling is more appropriate. Because, the current DLT in SPS is decided through the initial analysis and the DLT is applied to the whole sequence. The initial analysis is not good solution to DLT construction in the varying scene and can cause coding loss in the various test sequences. In this proposal, we make and send new DLT either at every intra slice.

The DLT intra slice-layer signalling was implemented and the test results in CTC show 0.0% (video PSNR / video bitrate), 0.0% (video PSNR / total bitrate), 0.0% (synth PSNR / total bitrate) BD-rate. The test results in all intra condition show 0.0% (video PSNR / video bitrate), 0.3% (video PSNR / total bitrate), 0.3% (synth PSNR / total bitrate) BD-rate when compared with 3DV-HTM ver. 6.0 [2]. The coding loss in all intra conditions is negligibly small compared to DLT’s own coding gain.

Finally, it is more appropriate for the intra slice-layer signalling to be adopted into 3DV-HTM.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, JCT-3V 2nd Meeting, Shanghai, CN, 13-19 Oct. 2012

- “Common Test Conditions for 3DV experimentation,” JCT2-C1100, JCT-3V 3rd Meeting, Geneva, CH, 17-23 Jan. 2013.

- “3D-CE6 Related: Improved Depth Lookup Table,” JCT3V-C0093, JCT-3V 3rd Meeting, Geneva, CH, 17-23 Jan. 2013.

# Appendix

In this section, we just show test result on all-slice signalling mandated at CE 6 output document (JCT3V-C1106). Tests were based on the common test condition and all-intra conditions for the 3 view configuration [2]. The results are summarized in Table 3(CTC) and Table 4(All-intra).

## Common test conditions

Table 3 BD rate results for 3-view case in common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% | 8.8% | 8.8% | 99.3% | 101.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 7.7% | 7.6% | 99.7% | 100.4% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | 8.9% | 8.9% | 99.8% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.1% | 96.7% |
| Poznan_Hall2 | 0.0% | -0.3% | -0.4% | -0.2% | 7.3% | 7.3% | 99.7% | 103.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 99.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.4% | 98.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 8.5% | 8.5% | 99.6% | 100.5% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | 0.0% | 1.8% | 1.8% | 99.2% | 99.7% |
| average | 0.0% | 0.0% | -0.1% | 0.0% | 4.7% | 4.7% | 99.4% | 100.0% |

## All intra conditions

Table 4 BD rate results for 3-view case in all intra conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 0.6% | 99.3% | 94.6% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 0.4% | 101.2% | 101.5% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.4% | 100.8% | 100.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.0% | 99.8% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 0.5% | 97.7% | 95.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 96.9% | 92.7% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 98.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 0.4% | 100.4% | 99.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 98.8% | 96.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.3% | 99.5% | 97.6% |

# Appendix(Tool Experiment on SDC and DLT)

In this section, we report the coding gain of the current SDC and DLT.

## SDC Enabled and DLT Disabled

Anchor: HTM ver. 6.0

Compared conditions: SDC is enabled and DLT is disabled.

Table 5 BD rate results for 3-view case in common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.0% | 0.5% | 0.8% | 99.1% | 106.1% |
| Kendo | 0.0% | 0.0% | 0.2% | 0.0% | 0.7% | 1.0% | 98.9% | 100.9% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.6% | 1.3% | 98.6% | 105.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.5% | 99.1% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.1% | 0.1% | 0.4% | 98.7% | 100.7% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.5% | 93.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.3% | 98.8% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.6% | 1.0% | 98.9% | 104.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 97.8% | 98.1% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.3% | 0.5% | 98.2% | 100.7% |

Table 6 BD rate results for 3-view case in all intra conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.9% | 1.9% | 100.0% | 94.4% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 1.0% | 2.0% | 104.7% | 101.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.7% | 1.9% | 98.0% | 99.9% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.9% | 100.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 1.2% | 96.8% | 91.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.0% | 93.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 99.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.9% | 1.9% | 100.9% | 98.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% | 98.2% | 96.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 1.0% | 99.3% | 97.1% |

## SDC disabled and DLT Disabled

Anchor: HTM ver. 6.0

Compared conditions: SDC is disabled and DLT is disabled.

Table 7 BD rate results for 3-view case in common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.2% | -0.1% | 0.4% | 0.1% | 96.8% | 104.2% |
| Kendo | 0.0% | 0.0% | 0.1% | 0.0% | 1.3% | 0.3% | 96.3% | 99.0% |
| Newspaper_CC | 0.0% | -0.1% | 0.0% | 0.0% | 0.6% | -0.1% | 98.2% | 101.3% |
| GT_Fly | 0.0% | -1.7% | -1.8% | -0.4% | -0.9% | -0.6% | 94.0% | 97.1% |
| Poznan_Hall2 | 0.0% | -0.6% | -0.4% | -0.2% | -0.6% | -0.5% | 99.9% | 105.4% |
| Poznan_Street | 0.0% | -0.7% | -0.5% | -0.2% | -0.6% | -0.7% | 96.6% | 96.3% |
| Undo_Dancer | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | -0.2% | 98.6% | 104.2% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.8% | 0.1% | 97.1% | 101.5% |
| 1920x1088 | 0.0% | -0.7% | -0.7% | -0.2% | -0.6% | -0.5% | 97.3% | 100.7% |
| average | 0.0% | -0.5% | -0.4% | -0.1% | 0.0% | -0.2% | 97.2% | 101.1% |

Table 6 BD rate results for 3-view case in all intra conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 2.3% | 1.2% | 95.4% | 113.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 3.7% | 1.6% | 93.4% | 119.6% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 1.6% | 0.9% | 96.0% | 119.3% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.8% | 0.3% | 84.9% | 114.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.5% | 95.1% | 110.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.3% | 91.8% | 108.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.4% | 94.2% | 108.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 2.5% | 1.2% | 94.9% | 117.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.0% | 91.5% | 110.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 1.2% | 0.5% | 93.0% | 113.3% |

# Draft text in 3D-HEVC Test Model 3

The specifications in subclause H.7.3.2.1.2 should be replaced by the following.

H.7.3.2.1.2	Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V DepthLayerFlag[ layerId ] should be derived from dimension_id to be aligned with basic idea of MV-HEVC Draft 3. ] |  |
| ViewId[ layerId ] = layerId >> 1 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V ViewId[ layerId ] should be derived as specified in MV-HEVC Draft 3. ] |  |
| view_id[ layerId ] [ Ed. (GT): This syntax element is different from ViewId[ layerId ]. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

The following syntax in specifications in subclause H.7.4.2.1.2 should be deleted

dlt_flag[ layerId ] equal to 1 specifies that depth lookup table is used and that residual values for simplified depth coded coding units are to be interpreted as indices of the depth lookup table for depth view components with layer_id equal to layerId. dlt_flag[ layerId ] equal to 0 specifies that depth lookup table is not used and residual values for simplified depth coded coding units are not to be interpreted as indices for depth view components with layer_id equal to layerId. When dlt_flat[ layerId ] is not present, it shall be inferred to be equal to 0.

num_depth_values_in_dlt[ layerId ] specifies the number of different depth values and the number of elements in the depth lookup table for depth view components of the current layer with layer_id equal to layerId.

dlt_depth_value[ layerId ][ j ] specifies the j-th entry in the depth lookup table for depth view components with layer_id equal to layerId.

The specifications in subclause H.7.3.5.1.1	Slice header extension syntax  should be replaced by the following.

| slice_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_header_flag ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| if( DepthLayerFlag[ layerId ] && slice_type = = I ) { |  |
| dlt_flag | u1(v) |
| If(dlt_flag) { |  |
| num_depth_values_in_dlt | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ j ]; | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The specifications in subclause H.7.4.5.1 should be replaced by the following.

dlt_flag equal to 1 specifies that depth lookup table is used and that residual values for simplified depth coded coding units are to be interpreted as indices of the depth lookup table for depth view components. dlt_flag equal to 0 specifies that depth lookup table is not used and residual values for simplified depth coded coding units are not to be interpreted as indices for depth view components. When dlt_flag is not present, it shall be inferred to be equal to 0.

num_depth_values_in_dlt specifies the number of different depth values and the number of elements in the depth lookup table for depth view components of the current layer.

dlt_depth_value [ j ] specifies the j-th entry in the depth lookup table for depth view components.

The specifications in H.8.1.1(Decoding process for a coded picture with nuh_layer_id greater than 0) should be replaced with the following.

    - When dlt_flag[ nuh_layer_id ] is equal to 1, the decoding process for the depth lookup table in subclause Error: Reference source not found is invoked at the beginning of the decoding process of first slice.

    - When dlt_flag is equal to 1, the decoding process for the depth lookup table in subclause Error: Reference source not found.1 is invoked at the beginning of the decoding process of every intra slice.