| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0148 |
| --- | --- |

| Title: | 3D-CE4.h related: Limited inter-view residual prediction by CBF checking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current HTM6.0, the inter-view residual prediction (IVRP) is enabled when the temporal inter-view candidate with 2Nx2N partition is selected even if the reference residual block only contains few non-zero coefficients. In that case, the RD performance cannot justify the computational cost due to the addition operations. In this contribution, it is proposed to control the on/off of IVRP by checking the coded block flag (CBF) of the reference residual block. Simulation results reportedly show that the proposed method can reduce 9% decoding time with 0.1% BD-rate reduction for video 1 and video 2.

# Introduction

The inter-view residual prediction (IVRP) is based on a disparity vector (DV) derivation for the current block. A DV is firstly derived for a current prediction unit (PU), and then the residual block in the reference view that is indicated by the DV is used for predicting the residual of the current PU.

In current HTM6.0, the IVRP is enabled when the temporal inter-view merge candidate with 2Nx2N partition is selected [1]. A derived DoNBDV [2] is first used to locate the reference block in reference view for deriving the availability of the temporal inter-view merge candidate. If the temporal inter-view merging candidate is available, then the residual of the reference block will be used for IVRP.

However, in many cases, the reference residual block contains few non-zero transform coefficients. In those cases, the RD performance cannot justify the computational cost due to the addition operations.

# Proposed method

It is proposed to disable the IVRP if the transform unit (TU), which covers the center sample of the reference block in reference view, does not contain non-zero coded block flag (CBF). Thus, the inefficient reference residual blocks which mostly contain zero residual samples would be removed from the IVRP mode so that the average computational complexity is reduced.

The location of the center sample of the reference block in reference view is derived as follows.Assume the above-left position of the current coding unit (CU) is (x, y), the width and height of current CU are W and H respectively, and the derived DV has 1/4-sample accuracy, then the position of the center sample of the reference block is set to (x+((W-1)>>1)+((DV_x+2)>>2), y+((H-1)>>1)+((DV_y+2)>>2)) and is illustrated in Fig. 1.

Fig. 1.  Center sample position for cbf checking

# Experimental results

The coding efficiency of the proposed method is tested based on HTM6.0 with the common test conditions [3]. The experimental results are shown in Table 1. The proposed method can reduce 9% decoding time and can also achieve slight coding gain (0.1% BD-rate saving for video 1 and video 2).

Table 1.  Experimental results of the proposed method compared to HTM6.0

# Conclusion

This contribution proposed to control the on/off of IVRP by checking the CBF of the reference residual block. The simulation results show that the proposed method can reduce 9% decoding time with 0.1% BD-rate saving for video 1 and video 2.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei, “CE4.h results on removal of parsing dependency for inter-view residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0138, January, 2013.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “CE1.h: Depth-oriented Neighboring Block Disparity Vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.