| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0149 |
| --- | --- |

| Title: | 3D-CE2.h related: Inter-view scaling for DV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to extend the inter-view scaling (IVS) to the disparity vector (DV) derivation process. This contribution proposes to scale the disparity vector according to the difference between view indices. The experiment is conducted with HTM-6.0 based on the IBP view configuration. The experimental results reportedly show that the proposed method can achieve 4.2% BD-rate gain for view 2, and 0.6% gain for video only.

# Introduction

In current HTM6.0, The DV derivation process has some problems related to the reference view index. The DV derivation is processed as following steps:

- Derive NBDV [1] associated with a reference view index.

- Derive DoNBDV [2] by using the derived NBDV and depth map.

  - Problem1: The view index of NBDV and view index of depth map could not be the same, currently the depth map is always in base view.

After the DV (NBDV or DoNBDV) is derived, it will be used for temporal inter-view motion vector candidate (TIVM), disparity inter-view motion vector candidate (DIVM), and inter-view residual prediction (IVRP) as follow:

- DoNBDV used for TIVM in AMVP and merge.

  - Problem2: The reference view of TIVM, which is the smallest view that included in the reference list, could not be the reference view of the DoNBDV.

- DoNBDV used for DIVM in AMVP and merge.

  - Problem3: The reference view of DIVM, which is directly the reference picture, could not be the reference view of the DoNBDV.

- NBDV used for IVRP

  - Problem4: The reference view of IVRP, which is the base view in current test model, could not be the reference view of the NBDV.

The above mentioned problems are not presented in CTC, since other inter-view reference pictures apart from the base view are not allowed. However, to enable a more general approach, this should be discussed and possibly made more general in SW and text.

# Proposed methods

For problem 1, one solution is to use the depth map with the view index equal to that of NBDV, however that would have more depth map dependency. Another solution is to scale the derived NBDV to the view of depth map. Here we propose the latter one.

For problem 2, the DoNBDV will be scaled to the reference view of TIVM.

For problem 3, the DoNBDV will be scaled to the reference view of DIVM.

For problem 4, the NBDV will be scaled to the reference view of IVRP.

Therefore, the entire proposed method is listed as below:

The DV derivation is processed as following ordered steps:

- Derive NBDV associated with a reference view index.

- Derive DoNBDV by using the derived NBDV and depth map.

  - If the view index of derived NBDV is not equal to the view index of depth map, then the derived NBDV is scaled with the difference between these two view indices.

After the DV (NBDV or DoNBDV) is derived, it will be used for inter-view motion prediction (IVMP) and inter-view residual prediction (IVRP) as follow:

- DoNBDV used for temporal inter-view motion vector candidate (TIVM) in AMVP and merge.

  - If the reference view index of TIVM, which is the smallest view that included in the reference list, is not equal to the reference view index of the DoNBDV, then the DoNBDV is scaled with the difference between these two view indices.

- DoNBDV used for disparity inter-view motion vector candidate (DIVM) in AMVP and merge.

  - If the reference view index of DIVM, which is directly the view index of reference picture, is not equal to the reference view index of the DoNBDV, then the DoNBDV is scaled with the difference between these two view indices.

- NBDV used for IVRP

  - If the reference view index of IVRP, which is the 0 in current test model, is not equal to the reference view index of the NBDV, then the NBDV is scaled with the difference between these two view indices.

# Experimental results

The coding efficiency of this proposed method can not be evaluated in the prediction structure of common test conditions since this tool requires at least two inter-view reference pictures. The prediction structure is changed to the IBP inter-view prediction structure as shown in the right side in Fig. 1, which is the also used in JCT3V-C0115 [3] and JCT3V-C0116 [4]. The proposed method actually runs only in center view (V1) since this view can refer to both left view (V0) and right view (V2).

The experiment is conducted based on HTM6.0.

Fig. 1: Tested prediction structure

The experimental results are shown in Table 1, it can be seen that the proposed method can provide 4.2% BD-rate gain for view 2 and 0.6% gain for video only.

# Conclusion

This contribution proposes to extend the inter-view scaling (IVS) to the disparity vector (DV) derivation process by scaling the derived DV according to the difference between view indices. The experiment is conducted with the IBP view configuration based on HTM6.0. The experimental results show that the proposed method can achieve 4.2% BD-rate gain for view 2, and 0.6% gain for video only.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Li Zhang et al, “CE5.h: Disparity vector generation results”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-JCT3V-A0097, July 2012, Stockholm.

- Yu-Lin Chang et al, “CE1.h: Depth-oriented Neighboring Block Disparity Vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, Jan., 2013.

- Yoshitomo Takahashi , Ohji Nakagami , Sally Hattori , Teruhiko Suzuki , “3D-HEVC: Alignment of inter-view vector scaling,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0115, Jan., 2013.

- Yoshitomo Takahashi , Ohji Nakagami , Sally Hattori , Teruhiko Suzuki , “3D-HEVC: Inter-view vector scaling for AMVP,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0116, Jan., 2013.