| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20-26 Apr. 2013 | Document: JCT3V-D0150 |
| --- | --- |

| Title: | CE6.h: Results on Signalling and DPCM Coding of Depth Lookup Table |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ilsoon Lim, Hocheon Wey, Jin Young Lee, Byeongdoo Choi, Chanyul Kim Samsung Electronics Jin Heo, Sehoon Yea LG electronics | Tel: Email: | ilsoon.lim@samsung.com jin78.heo@lge.com |
| Source: | Samsung Electronics, LG electronics |  |  |

_____________________________

# Abstract

We propose intra slice-layer signalling in the depth lookup table(DLT) and an efficient coding method on DLT via DPCM in 3DV-HTM. According to CE 6 mandate (JCT3V-C1106), we investigate the combination in terms of signaling and DPCM coding of DLT with emphasis on slice-level signaling.

The test results in CTC show 0.0% (video PSNR / video bitrate), 0.0% (video PSNR / total bitrate), 0.0% (synth PSNR / total bitrate) BD-rate. The test results in all intra condition show 0.0% (video PSNR / video bitrate), 0.10% (video PSNR / total bitrate), 0.06% (synth PSNR / total bitrate) BD-rate when compared with 3DV-HTM ver. 6.0 [2].

Since the signalling and the DPCM coding have their benefits (robust to scene change, DLT size reduction) without loss of coding efficiency, we recommend this proposal to be adopted into 3DV-HTM.

# Introduction

## Signalling of DLT

For more detail, see the JCT3V-D0147.

## DPCM Coding of DLT

For more detail, see the JCT3V-D0142.

# Test results – (Intra-slice Signalling + DPCM)

Tests were based on the common test condition and all-intra conditions for the 3 view configuration [2]. The results are summarized in Table 1(CTC) and Table 2(All-intra).

## Common test conditions

Table 1 BD rate results for 3-view case in common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 100.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 96.8% | 100.0% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.8% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 100.0% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.4% | -0.2% | -0.3% | -0.2% | 99.6% | 100.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 100.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 100.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.0% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 99.9% | 100.0% |
| average | 0.0% | -0.1% | -0.1% | -0.03% | -0.03% | -0.02% | 99.8% | 100.0% |

## All intra conditions

Table 2 BD rate results for 3-view case in all intra conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 97.7% | 96.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 101.6% | 101.5% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 98.9% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.7% | 95.3% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 95.9% | 95.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.9% | 94.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 97.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 99.4% | 99.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.1% | 95.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.10% | 0.06% | 98.6% | 97.2% |

# Conclusion

We propose intra slice-layer signalling in the depth lookup table(DLT) and an efficient coding method on DLT via DPCM in 3DV-HTM. According to CE 6 mandate (JCT3V-C1106), we investigate the combination in terms of signaling and DPCM coding of DLT with emphasis on slice-level signaling.

The test results in CTC show 0.0% (video PSNR / video bitrate), 0.0% (video PSNR / total bitrate), 0.0% (synth PSNR / total bitrate) BD-rate. The test results in all intra condition show 0.0% (video PSNR / video bitrate), 0.10% (video PSNR / total bitrate), 0.06% (synth PSNR / total bitrate) BD-rate when compared with 3DV-HTM ver. 6.0 [2].

Since the signalling and the DPCM coding have their benefits (robust to scene change, DLT size reduction) without loss of coding efficiency, we recommend this proposal to be adopted into 3DV-HTM.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. and LG Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, JCT-3V 2nd Meeting, Shanghai, CN, 13-19 Oct. 2012

- “Common Test Conditions for 3DV experimentation,” JCT3V-C1100, JCT-3V 3rd Meeting, Geneva, CH, 17-23 Jan. 2013.

- “3D-CE6 Related: Improved Depth Lookup Table,” JCT3V-C0093, JCT-3V 3rd Meeting, Geneva, CH, 17-23 Jan. 2013.

# Draft text in 3D-HEVC Test Model 3

The specifications in subclause H.7.3.2.1.2 should be replaced by the following.

H.7.3.2.1.2	Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V DepthLayerFlag[ layerId ] should be derived from dimension_id to be aligned with basic idea of MV-HEVC Draft 3. ] |  |
| ViewId[ layerId ] = layerId >> 1 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V ViewId[ layerId ] should be derived as specified in MV-HEVC Draft 3. ] |  |
| view_id[ layerId ] [ Ed. (GT): This syntax element is different from ViewId[ layerId ]. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

The following syntax in specifications in subclause H.7.4.2.1.2 should be deleted

dlt_flag[ layerId ] equal to 1 specifies that depth lookup table is used and that residual values for simplified depth coded coding units are to be interpreted as indices of the depth lookup table for depth view components with layer_id equal to layerId. dlt_flag[ layerId ] equal to 0 specifies that depth lookup table is not used and residual values for simplified depth coded coding units are not to be interpreted as indices for depth view components with layer_id equal to layerId. When dlt_flat[ layerId ] is not present, it shall be inferred to be equal to 0.

num_depth_values_in_dlt[ layerId ] specifies the number of different depth values and the number of elements in the depth lookup table for depth view components of the current layer with layer_id equal to layerId.

dlt_depth_value[ layerId ][ j ] specifies the j-th entry in the depth lookup table for depth view components with layer_id equal to layerId.

The specifications in subclause H.7.3.5.1 should be replaced by the following.

H.7.3.5.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if ( layer_id ) |  |
| slice_ic_enable_flag [ Ed.(GT): Should be moved to extension.. ] | u(1) |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| slice_header_extension |  |
| slice_header_extension2_flag | u(1) |
| if ( slice_header_extension2_flag ) { |  |
| slice_header_extension2_length | ue(v) |
| for( i = 0; i < slice_header_extension2_length; i++) |  |
| slice_header_extension2_data_byte[ i ] | u(8) |
| } |  |
| } |  |
| if( slice_type = = I ) { |  |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

The specifications in subclause H.7.4.5.1 should be replaced by the following.

dlt_flag[ i ] equal to 1 specifies that depth lookup table is used and that residual values for simplified depth coded coding units are to be interpreted as indices of the depth lookup table for depth view components with layer_id equal to i. dlt_flag[ i ] equal to 0 specifies that depth lookup table is not used and residual values for simplified depth coded coding units are not to be interpreted as indices for depth view components with layer_id equal to i. When dlt_flat[ i ] is not present, it shall be inferred to be equal to 0.

num_depth_values_in_dlt[ i ] specifies the number of different depth values and the number of elements in the depth lookup table for depth view components of the current layer with layer_id equal to i.

dlt_depth_value[ i ][ j ] specifies the j-th entry in the depth lookup table for depth view components with layer_id equal to i.

The specifications in H.8.1.1(Decoding process for a coded picture with nuh_layer_id greater than 0) should be replaced with the following.

    - When dlt_flag[ nuh_layer_id ] is equal to 1, the decoding process for the depth lookup table in subclause Error: Reference source not found is invoked at the beginning of the decoding process of first slice.

    - When dlt_flag[ nuh_layer_id ] is equal to 1, the decoding process for the depth lookup table in subclause Error: Reference source not found is invoked at the beginning of the decoding process of every intra slice.