| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0153 |
| --- | --- |

| Title: | 3D-CE6.h related: A flexible DLT signaling method in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Yu-Wen Huang2, Jicheng An1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Rd., Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, yuwen.huang, jicheng.an, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a flexible depth lookup table (DLT) signaling method in 3D-HEVC. First, it is allowed to signal DLTs in SPS. Second, a use_sps_dlt_flag is signaled in the slice header to indicate whether to use a DLT coded in SPS. If the use_sps_dlt_flag is set to 1, a sps_dlt_idx is signaled to indicate which DLT in SPS is selected when there are more than one DLT coded in SPS; otherwise, a new DLT or a DLT from a reference picture can be selected as follows. For an I slice, a new DLT is coded in the slice header. For a B or P slice, a use_ref_pic_dlt_flag is signaled. If the use_ref_pic_dlt_flag is set to 1, a dlt_ref_idx is signaled to explicit indicate the reference picture DLT; otherwise, a new DLT is coded. The proposed method can accommodate encoders that can analyze sequences in advance. It can also allow quick adaptation to scene changes and avoid introducing much side information when there is no scene change. It is reported that the proposed method results in similar coding efficiency as the anchor when there is no scene change.

# Introduction

Simplified depth coding (SDC) and a depth lookup table (DLT) are adopted into HTM [1] [2]. For each depth coding unit (CU), if SDC is selected, one of three different prediction modes can be selected. After the prediction, the residual is not coded using transform. As the SDC prediction stage always results in one or two depth segments per coded block, a single residual DC depth value is coded for each of these segments. Moreover, the DLT is used to map coded depth values in SDC to valid depth values of the original depth map. The DLT is constructed based on an initial analysis of the input depth map and is then coded in a sequence parameter set (SPS).

DLT is an optional coding tool. In the current HTM, the encoder will not use DLT if more than half of the values from 0 to MAX_DEPTH_VALUE (e.g., 255 for 8-bit depth samples) appear in the original depth map at the analysis step. Otherwise, DLT will be coded in SPS. In order to code DLT, the number of valid depth values is coded with Exp-Golomb code first. Then each valid depth value is also coded with an exp-Golomb code.

To make the DLT coding more efficient, in JCTVC-C0142 [3] it is proposed to code DLT with a range constrained bit map (RCBM) approach. It has been shown that RCBM can reduce DLT bits significantly. To make the DLT robust to scene change, in JCTVC-C0093 [4], it is proposed to signal DLT in the intra slice header instead of the SPS. As noted in the Core Experiment (CE) 6 description [5], to signal DLT in all slice headers will also be studied. Despite of the capability to adapt to varying scenes, signaling DLT in intra slice headers or all slice headers results in significantly more DLT bits.

# Proposed method

A flexible DLT signaling method is proposed, which can keep the benefit of signaling in (intra) slice header and that of signaling in SPS. First, one or more DLTs may be constructed based on an initial analysis, and then these DLTs are coded in SPS. Alternatively, there may be no DLT coded in SPS. Second, a use_sps_dlt_flag is signaled in the slice header to indicate whether a DLT coded in SPS is used for this slice. If the use_sps_dlt_flag is true, a sps_dlt_idx is signaled to indicate which one is selected when there are more than one DLT coded in SPS. Otherwise (use_sps_dlt_flag is false), the process is as follows. For an I slice, a new DLT is coded in the slice header. For a B or P slice, a use_ref_pic_dlt_flag is signaled to indicate using a DLT from a reference picture or using a new DLT. If the use_ref_pic_dlt_flag is set to true, a dlt_ref_idx is signaled to explicit indicate the reference picture DLT; otherwise, a new DLT is coded in the slice header. To reduce the bits of DLT, RCBM [3] can be used to code DLTs.

# Experimental results

The proposed methods are integrated on HTM-6.0 [6]. In the implementation, one or none DLT is coded in SPS using the same encoder strategy as HTM-6.0. All slices are set to use the DLT coded in SPS. Table 1 and Table 2 show the results of the proposed method under common test conditions [7] and all-intra test conditions [5] respectively. It can be seen that the proposed method can achieve similar coding efficiency as the anchor.

Table 1.  Proposed method under common test conditions

Table 2.  Proposed method under all intra test conditions

[to be added]

# Conclusion

In this proposal, a flexible depth lookup table (DLT) signaling method is proposed. By signaling DLT in SPS or in the slice header optionally, DLT can be more robust to scene changes without introducing too much side information for non-scene changes.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1005, January, 2013.

- F. Jager, “3D-CE6.h: Simplified depth coding with an optional depth lookup table,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B0036, October, 2012.

- K. Zhang, J. An, S. Lei, “3D-CE6.h related: An efficient coding method for DLT in 3DVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0142, Janurary, 2013.

- I. Lim, H. Wey and D. Park, “3D-CE6.h related: Improved depth lookup table (DLT),” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0093, January, 2013.

- P. Merkle, “Description of Core Experiment 6 (CE6) on depth intra coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1106, January, 2013.

- HTM-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.